3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304111

Electronic Meeting, April 17 – 26, 2023

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Absolute physical layer throughput simulation results

Agenda item:	5.19.2

Document for: 	Discussion

Introduction

This paper presents Nokia’s simulation results for Rel. 18’s Application Throughput Layer requirement definition. As stated in [1], RAN4 has agreed to keep the same scope of the ATP SI captured in clause 5.10.3 of TR37.901-5 [2], where the following three tests are considered:

- Test 1: FR1 FDD, SCS/CBW=15kHz/10MHz, 2Tx, 2Rx/4Rx - Test 2: FR1 TDD, SCS/CBW=30kHz/40MHz, 2Tx, 2Rx/4Rx - Test 3: FR2-1 TDD, SCS/CBW=120kHz/100MHz, 2Tx, 2Rx

Table 1 shows the simulation assumptions used in the Rel.17 study item (SI) with one correction to the used MCS table for FR2 which was incorrect in the study (see also agreements from RAN4#105 + RAN#106).

Table 1 Simulation Assumptions used in Rel.17 SI

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
| Frequency range |  |  | FR1 | FR1 | FR2 |
| Bandwidth |  | MHz | 10 | 40 | 100 |
| Subcarrier spacing |  | kHz | 15 | 30 | 120 |
| Duplex Mode |  |  | FDD | TDD | TDD |
| TDD Slot Configuration |  |  | N/A | 7D1S2U S:6D+4G+4U | DDSU S:11D+3G+0U |
| SNR |  | dB | 0:2:20 | 0:2:20 | 0:2:16 |
| Propagation channel |  |  | TDLA30-5 | TDLA30-5 | TDLA30-35 |
| Antenna configuration |  |  | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2, ULA Low 2x4 | ULA Low 2x2 |
| Beamforming Model |  |  | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 | As defined in Annex B.4.1 in TS 38.101-4 |
| Receiver type |  |  | MMSE-IRC | MMSE-IRC | MMSE-IRC |
| PDSCH configuration | Mapping type |  | Type A | Type A | Type A |
|  | Starting symbol (S) |  | 2 | 2 | 2 |
|  | Length (L) |  | 12 | 12 | 12 |
|  | PRB bundling size |  | 2 | 2 | 2 |
|  | PRB bundling type |  | Static | Static | Static |
|  | VRB-to-PRB mapping interleaver bundle size |  | Non-interleaved | Non-interleaved | Non-interleaved |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 | Type 1 | Type 1 |
|  | Number of additional DMRS |  | 1 | 1 | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 | 1 | 1 |
| CSI measurement channels (Note 2) |  |  | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-1 Rank 2: TBS.2-2 | As specified in Table A.4-2 of TS 38.101-4: Rank 1: TBS.2-3 Rank 2: TBS.2-4 | As specified in Table A.4-1 of TS 38.101-4: Rank 1: TBS.1-1 Rank 2: TBS.1-2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 | 4 | 4 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5, (4) | Row 5, (4) | Row 5, (8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 9 | 9 | 13 |
|  | CSI-RS periodicity and offset | slot | 5/1 | 10/1 | 8/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Periodic | Periodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 2 | 2 | 2 |
|  | CDM Type |  | FD-CDM2 | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 3 (6) | Row 3 (6) | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | 13 | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| CSI-IM configuration | CSI-IM resource Type |  | Periodic | Periodic | Periodic |
|  | CSI-IM RE pattern |  | Pattern 0 | Pattern 0 | Pattern 1 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) | (4,9) | (8,13) |
|  | CSI-IM timeConfig periodicity and offset | slot | 5/1 | 10/1 | Not configured |
| ReportConfigType |  |  | Aperiodic | Aperiodic | Aperiodic |
| CQI-table |  |  | Table 2 | Table 2 | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI | cri-RI-PMI-CQI | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | not configured | not configured | not configured |
| timeRestrictionForInterferenceMeasurements |  |  | not configured | not configured | not configured |
| cqi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| pmi-FormatIndicator |  |  | Wideband | Wideband | Wideband |
| Sub-band Size |  | RB | 8 | 16 | 8 |
| csi-ReportingBand |  |  | 1111111 | 1111111 | 111111111 |
| CSI-Report periodicity and offset |  | slot | Not configured | Not configured | Not configured |
| Aperiodic Report Slot Offset |  |  | 5 | 9 | 7 |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 10) = 0, otherwise it is equal to 0 | 1 in slots i, where mod(i, 8) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 | 1 | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel | typeI-SinglePanel |
|  | Codebook Mode |  | 1 | 1 | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A | N/A | N/A |
|  | CodebookSubsetRestriction |  | Not configured | Not configured | Not configured |
|  | RI Restriction |  | N/A | N/A | N/A |
| Physical channel for CSI report |  |  | PUSCH | PUSCH | PUSCH |
| CQI/RI/PMI delay |  | ms | 6 | 5.5 | 1.375 |
| Maximum number of HARQ transmission |  |  | 1 | 1 | 1 |
| Test metric |  |  | T% of max throughput at target SNR. |  |  |
| Note 1: Other common test parameters are defined in Section 6.1.2 of 38.101-4 for Tests 1 and 2 and Section 8.1.2 of 38.101-4 for Test 3. Note 2: PDSCH is not scheduled on slots containing CSI-RS for CSI acquisition, CSI-RS for tracking and CSI-RS for beam refinement (for Test 3 only). |  |  |  |  |  |

Additional simulation assumptions after RAN4#106 are shown in Table 2:

Table 2 Additional simulation assumptions from RAN4#106

| HARQ ReTx | RV {0,2,3,1} with same MCS and rank as initial transmission For precoder following UE reported PMI |
| --- | --- |
| Tx EVM | 6% for both FR1 and FR2 |

Simulation Results

The following sections present Nokia’s simulation results under the agreed simulation assumptions detailed above.  Each simulation test is characterized by its absolute physical layer throughput, the overall block error rate (BLER), median rank indicator (RI) and median channel quality indicator (CQI). As per agreement in RAN4#106 HARQ re-transmission (re-Tx) is also included following the same simulation assumptions as the legacy SI configuration from Rel. 17 described above with the addition of the parameters summarized in Table 2 .

Table 3 Simulation configuration scenarios

| Configuration scenarios | Number of HARQ Re-transmissions |
| --- | --- |
| Legacy SI | 1 |
| HARQ re-TX enabled | 4 (RV {0,2,3,1}) |

Legacy physical layer throughput results

These results follow the legacy configuration from the SI (no HARQ reTx).

Table 4 ATP Link Adaptation results for Test 1 – FR1 FDD (EVM = 0%)

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 4.12 | 0.10 | 1 | 3 | 6.40 | 0.04 | 1 | 4 |
| 2 | 5.65 | 0.12 | 1 | 4 | 8.38 | 0.05 | 1 | 6 |
| 4 | 7.39 | 0.12 | 1 | 5 | 10.44 | 0.15 | 2 | 4 |
| 6 | 9.27 | 0.04 | 1 | 6 | 14.20 | 0.02 | 2 | 5 |
| 8 | 11.25 | 0.04 | 1 | 7 | 17.81 | 0.03 | 2 | 6 |
| 10 | 13.36 | 0.04 | 1 | 8 | 21.46 | 0.04 | 2 | 7 |
| 12 | 15.41 | 0.04 | 1 | 9 | 25.64 | 0.03 | 2 | 8 |
| 14 | 16.98 | 0.02 | 1 | 10 | 29.95 | 0.02 | 2 | 9 |
| 16 | 19.79 | 0.09 | 2 | 6 | 32.52 | 0.06 | 2 | 10 |
| 18 | 23.06 | 0.09 | 2 | 8 | 36.38 | 0.04 | 2 | 10 |
| 20 | 26.00 | 0.04 | 2 | 8 | 40.71 | 0.06 | 2 | 12 |

Table 5 ATP Link Adaptation results for Test 1 – FR1 FDD (EVM = 6%)

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 3.89 | 0.11 | 1 | 3 | 6.18 | 0.06 | 1 | 4 |
| 2 | 5.33 | 0.12 | 1 | 4 | 8.12 | 0.07 | 1 | 6 |
| 4 | 7.02 | 0.13 | 1 | 5 | 10.22 | 0.14 | 2 | 4 |
| 6 | 8.78 | 0.11 | 1 | 6 | 13.74 | 0.02 | 2 | 5 |
| 8 | 10.63 | 0.10 | 1 | 7 | 17.19 | 0.03 | 2 | 6 |
| 10 | 12.56 | 0.05 | 1 | 8 | 20.70 | 0.07 | 2 | 7 |
| 12 | 14.58 | 0.04 | 1 | 9 | 24.56 | 0.05 | 2 | 8 |
| 14 | 16.29 | 0.07 | 2 | 5 | 28.20 | 0.07 | 2 | 9 |
| 16 | 19.34 | 0.08 | 2 | 6 | 32.11 | 0.06 | 2 | 9 |
| 18 | 22.51 | 0.08 | 2 | 8 | 34.65 | 0.02 | 2 | 10 |
| 20 | 25.69 | 0.08 | 2 | 8 | 35.99 | 0.01 | 2 | 10 |

Table 6 ATP Link Adaptation results for Test 2 – FR1 TDD (EVM = 0%)

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 13.08 | 0.09 | 1 | 3 | 20.77 | 0.03 | 1 | 4 |
| 2 | 17.97 | 0.11 | 1 | 4 | 27.26 | 0.04 | 1 | 6 |
| 4 | 23.34 | 0.12 | 1 | 5 | 34.93 | 0.12 | 2 | 4 |
| 6 | 29.40 | 0.10 | 1 | 6 | 45.77 | 0.02 | 2 | 5 |
| 8 | 35.97 | 0.08 | 1 | 7 | 57.27 | 0.03 | 2 | 6 |
| 10 | 42.49 | 0.08 | 1 | 8 | 69.78 | 0.03 | 2 | 7 |
| 12 | 48.81 | 0.04 | 1 | 9 | 83.05 | 0.06 | 2 | 8 |
| 14 | 54.27 | 0.06 | 2 | 5 | 91.92 | 0.10 | 2 | 9 |
| 16 | 64.53 | 0.07 | 2 | 6 | 103.36 | 0.01 | 2 | 9 |
| 18 | 76.12 | 0.07 | 2 | 8 | 116.28 | 0.03 | 2 | 10 |
| 20 | 85.54 | 0.08 | 2 | 8 | 127.97 | 0.07 | 2 | 12 |

Table 7 ATP Link Adaptation results for Test 2 – FR1 TDD (EVM = 6%)

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 11.95 | 0.12 | 1 | 3 | 19.96 | 0.05 | 1 | 4 |
| 2 | 16.42 | 0.14 | 1 | 4 | 26.09 | 0.06 | 1 | 6 |
| 4 | 21.74 | 0.15 | 1 | 5 | 34.32 | 0.11 | 2 | 4 |
| 6 | 27.24 | 0.06 | 1 | 6 | 43.62 | 0.14 | 2 | 5 |
| 8 | 33.35 | 0.11 | 1 | 7 | 54.62 | 0.11 | 2 | 6 |
| 10 | 39.49 | 0.10 | 1 | 8 | 67.96 | 0.06 | 2 | 7 |
| 12 | 45.46 | 0.06 | 1 | 9 | 80.73 | 0.04 | 2 | 8 |
| 14 | 52.87 | 0.06 | 2 | 5 | 92.58 | 0.04 | 2 | 9 |
| 16 | 62.59 | 0.07 | 2 | 6 | 104.04 | 0.05 | 2 | 9 |
| 18 | 73.42 | 0.06 | 2 | 8 | 113.19 | 0.07 | 2 | 10 |
| 20 | 84.85 | 0.05 | 2 | 8 | 115.61 | 0.01 | 2 | 10 |

Table 8 ATP Link Adaptation results for Test 3 – FR2-1 TDD (EVM = 0%)

| SNR (dB) | 2x2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 20.16 | 0.07 | 1 | 5 |
| 2 | 26.73 | 0.15 | 1 | 7 |
| 4 | 35.17 | 0.09 | 1 | 8 |
| 6 | 43.10 | 0.11 | 1 | 9 |
| 8 | 52.12 | 0.04 | 1 | 10 |
| 10 | 63.27 | 0.10 | 1 | 11 |
| 12 | 75.43 | 0.07 | 1 | 12 |
| 14 | 87.00 | 0.06 | 1 | 13 |
| 16 | 101.67 | 0.04 | 2 | 9 |

Table 9 ATP Link Adaptation results for Test 3 – FR2-1 TDD (EVM = 6%)

| SNR (dB) | 2x2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 18.62 | 0.10 | 1 | 5 |
| 2 | 24.57 | 0.09 | 1 | 6 |
| 4 | 31.61 | 0.20 | 1 | 8 |
| 6 | 39.80 | 0.19 | 1 | 9 |
| 8 | 48.09 | 0.18 | 1 | 10 |
| 10 | 58.54 | 0.07 | 1 | 10 |
| 12 | 68.50 | 0.05 | 1 | 11 |
| 14 | 82.52 | 0.15 | 2 | 9 |
| 16 | 97.29 | 0.15 | 2 | 10 |

The throughput results for the legacy SI configuration are graphically represented in Figure 1 for both EVM assumptions. The EVM 0% results for the legacy SI configuration show good alignment with the provided company results from RAN4#106 [4].  The EVM 6% results show a slight performance degradation, particularly at higher SNRs with high CQI reported values (2x4 scenarios). This is to be expected as the 6%EVM limits the effective SNR to around 20DB at 64QAM, which is exactly what is seen from the simulations.
As such an EVM of 6% will not impact the demodulation performance for SNR levels up to 20 dB.

Figure  1 –Throughput (Mbps) for legacy SI configuration (EVM 0% vs. EVM 6%).

HARQ re-Tx-enabled physical layer throughput results

This set of results takes the maximum number of HARQ transmission as defined in Table 2 with the redundancy-version-sequence="0 2 3 1".

Table 10 ATP Link Adaptation results for Test 1 – FR1 FDD (EVM = 0%) – updated results from RAN#106

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 3.81 | 0.03 | 1 | 3 | 6.52 | 0.03 | 1 | 4 |
| 2 | 5.67 | 0.07 | 1 | 4 | 8.56 | 0.04 | 1 | 6 |
| 4 | 7.68 | 0.08 | 1 | 5 | 10.58 | 0.06 | 2 | 4 |
| 6 | 9.42 | 0.03 | 1 | 6 | 14.36 | 0.02 | 2 | 5 |
| 8 | 11.43 | 0.03 | 1 | 7 | 18.07 | 0.03 | 2 | 6 |
| 10 | 13.54 | 0.03 | 1 | 8 | 21.79 | 0.03 | 2 | 7 |
| 12 | 15.66 | 0.03 | 1 | 9 | 25.94 | 0.02 | 2 | 8 |
| 14 | 17.14 | 0.02 | 1 | 10 | 30.16 | 0.02 | 2 | 9 |
| 16 | 20.58 | 0.06 | 2 | 6 | 33.56 | 0.04 | 2 | 10 |
| 18 | 24.01 | 0.07 | 2 | 8 | 36.94 | 0.03 | 2 | 10 |
| 20 | 26.49 | 0.03 | 2 | 8 | 41.68 | 0.04 | 2 | 12 |

Table 11 ATP Link Adaptation results for Test 1 – FR1 FDD (EVM = 6%)

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 4.13 | 0.08 | 1 | 3 | 6.36 | 0.04 | 1 | 4 |
| 2 | 5.69 | 0.10 | 1 | 4 | 8.35 | 0.05 | 1 | 6 |
| 4 | 7.44 | 0.10 | 1 | 5 | 11.06 | 0.10 | 2 | 4 |
| 6 | 9.19 | 0.09 | 1 | 6 | 13.90 | 0.02 | 2 | 5 |
| 8 | 11.04 | 0.08 | 1 | 7 | 17.41 | 0.02 | 2 | 6 |
| 10 | 12.82 | 0.04 | 1 | 8 | 21.29 | 0.05 | 2 | 7 |
| 12 | 14.85 | 0.03 | 1 | 9 | 25.08 | 0.04 | 2 | 8 |
| 14 | 16.88 | 0.05 | 2 | 5 | 28.96 | 0.05 | 2 | 9 |
| 16 | 20.04 | 0.06 | 2 | 6 | 32.90 | 0.04 | 2 | 9 |
| 18 | 23.34 | 0.06 | 2 | 8 | 34.92 | 0.01 | 2 | 10 |
| 20 | 26.63 | 0.06 | 2 | 8 | 36.13 | 0.01 | 2 | 10 |

Table 12 ATP Link Adaptation results for Test 2 – FR1 TDD (EVM = 0%) – updated results from RAN#106

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 11.70 | 0.02 | 1 | 3 | 21.08 | 0.03 | 1 | 4 |
| 2 | 17.80 | 0.06 | 1 | 4 | 27.68 | 0.03 | 1 | 6 |
| 4 | 24.18 | 0.08 | 1 | 5 | 34.55 | 0.04 | 2 | 4 |
| 6 | 30.50 | 0.08 | 1 | 6 | 46.24 | 0.01 | 2 | 5 |
| 8 | 37.03 | 0.07 | 1 | 7 | 58.10 | 0.02 | 2 | 6 |
| 10 | 43.77 | 0.07 | 1 | 8 | 70.76 | 0.02 | 2 | 7 |
| 12 | 49.54 | 0.03 | 1 | 9 | 85.46 | 0.03 | 2 | 8 |
| 14 | 55.69 | 0.04 | 2 | 5 | 97.30 | 0.06 | 2 | 9 |
| 16 | 66.42 | 0.05 | 2 | 6 | 109.39 | 0.02 | 2 | 9 |
| 18 | 78.08 | 0.05 | 2 | 8 | 113.80 | 0.12 | 2 | 10 |
| 20 | 88.50 | 0.06 | 2 | 8 | 124.53 | 0.16 | 2 | 12 |

Table 13 ATP Link Adaptation results for Test 2 – FR1 TDD (EVM = 6%)

| SNR (dB) | 2x2 |  |  |  | 2x4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 12.79 | 0.10 | 1 | 3 | 20.47 | 0.04 | 1 | 4 |
| 2 | 17.73 | 0.12 | 1 | 4 | 26.80 | 0.05 | 1 | 6 |
| 4 | 23.32 | 0.12 | 1 | 5 | 36.72 | 0.07 | 2 | 4 |
| 6 | 28.13 | 0.06 | 1 | 6 | 47.19 | 0.08 | 2 | 5 |
| 8 | 34.20 | 0.06 | 1 | 7 | 56.43 | 0.01 | 2 | 6 |
| 10 | 40.34 | 0.06 | 1 | 8 | 69.90 | 0.03 | 2 | 7 |
| 12 | 44.38 | 0.04 | 2 | 4 | 82.26 | 0.02 | 2 | 8 |
| 14 | 54.26 | 0.05 | 2 | 5 | 94.74 | 0.02 | 2 | 9 |
| 16 | 64.24 | 0.06 | 2 | 6 | 106.68 | 0.03 | 2 | 9 |
| 18 | 75.31 | 0.05 | 2 | 8 | 112.72 | 0.01 | 2 | 10 |
| 20 | 86.27 | 0.04 | 2 | 8 | 115.99 | 0.004 | 2 | 10 |

Table 14 ATP Link Adaptation results for Test 3 – FR2-1 TDD (EVM = 0%) – updated results from RAN#106

| SNR (dB) | 2x2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 20.85 | 0.06 | 1 | 5 |
| 2 | 28.52 | 0.11 | 1 | 7 |
| 4 | 36.60 | 0.07 | 1 | 8 |
| 6 | 45.38 | 0.09 | 1 | 9 |
| 8 | 53.15 | 0.03 | 1 | 10 |
| 10 | 65.92 | 0.08 | 1 | 11 |
| 12 | 77.56 | 0.05 | 1 | 12 |
| 14 | 89.14 | 0.05 | 1 | 13 |
| 16 | 103.31 | 0.03 | 2 | 9 |

Table 15 ATP Link Adaptation results for Test 3 – FR2-1 TDD (EVM = 6%)

| SNR (dB) | 2x2 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Throughput (Mbps) | BLER | Median Rank | Median CQI |
| 0 | 19.54 | 0.08 | 1 | 5 |
| 2 | 25.63 | 0.07 | 1 | 6 |
| 4 | 35.01 | 0.16 | 1 | 8 |
| 6 | 43.54 | 0.15 | 1 | 9 |
| 8 | 52.23 | 0.14 | 1 | 10 |
| 10 | 60.38 | 0.05 | 1 | 10 |
| 12 | 70.12 | 0.04 | 1 | 11 |
| 14 | 88.99 | 0.12 | 2 | 9 |
| 16 | 104.33 | 0.12 | 2 | 10 |

The throughput results for the legacy SI configuration are graphically represented in Figure 1 for both EVM assumptions. The EVM 0% results for the HARQ re-Tx configuration show good alignment with the few companies who provided results from RAN4#106 [4].  The EVM 6% show the same trend as in the Legacy result set, with a slight at higher SNRs with high CQI reported values (2x4 scenarios).

Figure 2 - Throughput (Mbps) for HARQ configuration with 4 HARQ-reTx (EVM 0% vs. EVM 6%).

Conclusion

This paper presents Nokia’s simulation results based on the agreed configuration from the discussion of feasibility to define requirements for Application Layer Throughput with link adaptation.

Both the legacy and HARQ re-Tx results show good alignment with company results provided in RAN4#106 [4].

References

- R4-2220276 - “WF on physical layer throughput requirements”, Intel, RAN4 #105, November 2022.

- 3GPP TR 37.901-5 “Study on 5G NR User Equipment (UE) application layer data throughput performance (Release 16)”, V16.8.0.

- R4-2300703 - “On Absolute physical layer throughput requirements with link adaptation”, Nokia

- R4-2300484 - Summary UE Physical Layer Data Throughput