3GPP TSG-RAN WG4 Meeting # 106bis-e		R4-2304112

Online, April 17 – April 26, 2023

Agenda Item:	4.16.2

Source: 	Nokia, T-Mobile USA

Title: 	TP to TR 37.829: System level simulation results for coexistence study on 31dBm UE Power Class for LTE Band 41 and NR Band n41

Document for:	Approval

# 1.	Introduction

The revised WID on High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands was approved at TSG RAN #98e [1]. One of the objectives of this work item is to define power class 1 (31 dBm) requirements for NR Band n41. Hence system level simulations need to be performed for the coexistence study similar to those performed on 29 dBm UE Power Class for LTE Band 41 and NR Band n41 and recorded in Annex C of TR 38.817-01 [2].

The TP on the system level simulation methodology and assumptions for coexistence study on 31 dBm UE Power Class for LTE Band 41 and NR Band n41 was approved [2] and simulation results were presented [3] at TSG RAN4 #106bis. This contribution provides a text proposal to record the simulation results into annex A in TR 37.829 [4].

# 2.	Text proposal

<Start of change>

Annex A:
Coexistence studies for 31 dBm UE Power Class for LTE Band 41 and NR Band n41

# A.1	Simulation assumptions

#### A.1.1	Macro cell Propagation model

##### A.1.1.1	Macro cell Propagation model – Urban and Suburban Areas

The propagation model is a derived from TR 36.942 [3].

Considering a carrier frequency of 2.6 GHz and a base station antenna height of 15 m above average rooftop level, the propagation model is given by the following equation:

L=130.5+37.6log10(R)

where:

R is the base station-UE separation in kilometres

##### A.1.1.2 	Macro cell Propagation model – Rural Area

The propagation model is a derived from TR 36.942.

For rural area, the Hata model is not applicable for a carrier frequency of 2.6 GHz, while the modified Hata model can be used:

Case 1:		d  0.6 km

L = 100.7 + 10 log(d2 + 0.00189225)

Case 2:		d  0.6 km

L = 103.9 + 34.1 log(d)

where: d is the base station-UE separation in kilometres

#### A.1.2	Power Control Simulation Parameters

Table A.1.2-1 CLx-ile parameters for +23 dBm UE

(a) CLx-ile parameters for +23 dBm UE using 0.75 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 109 | 112 |
| Set 1’ | 1 | 117 | 120 |
| Set 2 | 0,8 | 133 | 137 |

(b) CLx-ile parameters for +23 dBm UE using 2.8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 133 | 136 |
| Set 2 | 0,8 | 149 | 153 |

(c) CLx-ile parameters for +23 dBm UE using 6 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 117 | 120 |
| Set 2 | 0,8 | 132 | 136 |

(d) CLx-ile parameters for +23 dBm UE using 8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 122 | 124 |
| Set 2 | 0,8 | 136 | 140 |

Table A.1.2-2 CLx-ile power control algorithm parameters for +31 dBm UE

(a) CLx-ile power control algorithm parameters for +31 dBm UE using 0.75 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 117 | 120 |
| Set 1’ | 1 | 125 | 128 |
| Set 2 | 0,8 | 143 | 147 |

(b) CLx-ile power control algorithm parameters for +31 dBm UE using 2.8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 141 | 144 |
| Set 2 | 0,8 | 159 | 163 |

(c) CLx-ile power control algorithm parameters for +31 dBm UE using 6 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 125 | 128 |
| Set 2 | 0,8 | 142 | 146 |

(d) CLx-ile power control algorithm parameters for +31 dBm UE using 8 km inter-site distance and 2.6 GHz carrier frequency

| Parameter set | Gamma | Modified CLx-ile |  |
| --- | --- | --- | --- |
|  |  | 20 MHz bandwidth | 10 MHz bandwidth |
| Set 1 | 1 | 130 | 132 |
| Set 2 | 0,8 | 146 | 150 |

#### A.1.3	Cell Layout

Base stations with 3 sectors per site are placed on a hexagonal grid with distance of 3*R, where R is the cell radius (see Figure A.1.3-1), with wrap around. The number of sites shall be equal to or higher than 19. Uncoordinated macro cellular deployment is assumed, where interfering UE may be at cell edge of the serving base station but close to the victim base station (hence transmitting with highest power and causing highest interference).

Figure A.1.3-1: Uncoordinated macro cellular deployment

The inter-site distances considered in the present document are provided in Table A.1.3-1 below.

Table A.1.3-1: Inter-site distances and Propagation model

| Environment | ISD (km) | ISD (miles) |
| --- | --- | --- |
| Urban | .75 | .47 |
| Suburban | 2.8 | 1.74 |
| Rural | 6 | 3.73 |
| Rural | 8 | 5 |

#### A.1.4	Other Simulation Assumptions

Other simulation assumptions are summarized in Table A.1.4-1 below:

Table A.1.4-1: Simulation parameters for Band 41 system

(a) With 23 dBm UE

|  | Base Station | UE |
| --- | --- | --- |
| Carrier frequency | 2600 MHz |  |
| Channel bandwidth | 20 MHz, 10 MHz |  |
| Inter-site distance | Use Table A.1.3-1 |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |
| Frequency reuse | 1x3x1 |  |
| Lognormal fading | 10 dB |  |
| Shadowing correlation | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) | 70 dB (urban and suburban areas) 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | 17 dBi, = 65 degrees, Am = 20 dB | Omni-directional antenna with -3.5 dBi. |
| Noise figure | 5 dB | 9 dB |
| Transmit power | 46 dBm | 23 dBm |
| Antenna height | 45 m | 1.5 m |
| ACLR | 45 dB | Use Table 5.2 in TR 36.942 ACLR1: 30+X, ACLR2: 43+X Where X is 1 dB |
| ACS | 45 dB | 27 dB (20 MHz), 33 dB (10 MHz) |

(b) With 31 dBm UE

|  | Base Station | HPUE |
| --- | --- | --- |
| Carrier frequency | 2600 MHz |  |
| Channel bandwidth | 20 MHz, 10 MHz |  |
| Inter-site distance | Use Table A.1.3-1 |  |
| Cell layout | Wrap-around 19 tri-sector cells, uncoordinated |  |
| Frequency reuse | 1x3x1 |  |
| Lognormal fading | 10 dB |  |
| Shadowing correlation | Between cells: 0.5, between sites: 1.0 |  |
| MCL (including antenna gain) | 70 dB (urban and suburban areas) 80 dB (rural area) |  |
| Antenna gain and horizontal antenna pattern | 17 dBi, = 65 degrees, Am = 20 dB | Omni-directional antenna with -3.5 dBi. |
| Noise figure | 5 dB | 9 dB |
| Transmit power | 46 dBm | 31 dBm |
| Antenna height | 45 m | 1.5 m |
| ACLR | 45 dB | Use Table 5.2 in TR 36.942 ACLR1: 30+X, ACLR2: 43+X Where X is 1 dB |
| ACS | 45 dB | 27 dB (20 MHz), 33 dB (10 MHz) |

Simulations should assume the worst case of 100 % HPUEs in the scenarios with HPUEs.

#### A.1.5	Simulation Procedure

For the co-existence study, the following procedure shall be performed:

1)	Run the Band 41 UL to UL coexistence study, assuming parameters of both systems are according Table A.1.4-1 (a). Power control parameters in Table A.1.2-1 are used. This corresponds to the coexistence of two commercial networks operating in adjacent channel and with similar deployment parameters. This is used as the reference. Band 41 victim system performance degradation results in this scenario are used as the baseline. Provide a CDF plot of UE transmit power.

2)	Run the Band 41 UL to UL coexistence study, assuming +31 dBm power class UE is deployed in Band 41 interfering system only, and obtain the victim system performance degradation results. The simulation parameters in Tables A.1.4-1 (a) and A.1.4-1 (b) are used for the victim and interfering system, respectively. And the power control parameters in Tables A.1.2-1 and A.1.2-2 are used for the victim and interfering system, respectively. Provide a CDF plot of UE transmit power.

3)	Compare the Band 41 victim system performance degradation obtaining in steps 1) and 2), choose the 31 dBm UE ACLR value so that the victim system performance degradation due to 31 dBm UE in 2) is the same as 1).

A.2	Simulation results

#### A.2.1	0.75 km inter-site distance

The CDFs of the UE transmit power as well as the victim system UL throughput loss Vs ACLR offset (with different power control parameter sets) for 0.75 km inter-site distance with 20 MHz channel bandwidth are shown in Figure A.2.1-1 below.

| ACLR offset X [dB] | 0 | 7 |
| --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 2.99% |  |
| 5%-tile throughput loss (23 dBm interfering UE) | 3.02% |  |
| Average throughput loss (31 dBm interfering UE) | 3.35% | 1.68% |
| 5%-tile throughput loss (31 dBm interfering UE) | 4.30% | 1.48% |

(a) With Power Control Parameter Set 1

| ACLR offset X [dB] | 0 | 7 |
| --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 2.66% |  |
| 5%-tile throughput loss (23 dBm interfering UE) | 3.21% |  |
| Average throughput loss (31 dBm interfering UE) | 2.70% | 1.31% |
| 5%-tile throughput loss (31 dBm interfering UE) | 3.25% | 1.27% |

(b) With Power Control Parameter Set 1’

| ACLR offset X [dB] | 0 |
| --- | --- |
| Average throughput loss (23 dBm interfering UE) | 1.07% |
| 5%-tile throughput loss (23 dBm interfering UE) | 1.49% |
| Average throughput loss (31 dBm interfering UE) | 1.07% |
| 5%-tile throughput loss (31 dBm interfering UE) | 1.49% |

(c) With Power Control Parameter Set 2

Figure A.2.1-1: For 0.75 km inter-site distance with 20 MHz channel bandwidth

It can be seen from the CDFs of the UE transmit power in Figure A.2.1-1 that the CDFs of the 23 dBm UE and the 31 dBm UE are identical until the UE reach their maximum output power. This is expected as the CLx-ile is adjusted according to the UE maximum output power. Comparing the CDFs of the UE transmit power with Set 1 and Set 1’, it can be seen that more (~10% of 23 dBm UE and ~1.5% of 31 dBm UE) of the UE population transmitted with their maximum output power with the more aggressive Set 1.

Moreover, it can be seen from the victim system UL throughput loss Vs ACLR offset results in Figure A.2.1-1 that with the more aggressive Set 1, the 37 dB ACLR of the 31 dBm UE (currently specified for power class 1 UE) will ensure that the victim system performance degradation due to 31 dBm interfering UE is not larger than that due to 23 dBm interfering UE.

#### A.2.2	2.8 km inter-site distance

The CDFs of the UE transmit power as well as the victim system UL throughput loss Vs ACLR offset (with different power control parameter sets) for 2.8 km inter-site distance with 20 MHz channel bandwidth are shown in Figure A.2.2-1 below.

| ACLR offset X [dB] | 0 | 7 |
| --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 1.35% |  |
| 5%-tile throughput loss (23 dBm interfering UE) | 12.21% |  |
| Average throughput loss (31 dBm interfering UE) | 1.43% | 0.83% |
| 5%-tile throughput loss (31 dBm interfering UE) | 13.03% | 6.81% |

(a) With Power Control Parameter Set 1

| ACLR offset X [dB] | 0 |
| --- | --- |
| Average throughput loss (23 dBm interfering UE) | 0.31% |
| 5%-tile throughput loss (23 dBm interfering UE) | 0.53% |
| Average throughput loss (31 dBm interfering UE) | 0.31% |
| 5%-tile throughput loss (31 dBm interfering UE) | 0.53% |

(b) With Power Control Parameter Set 2

Figure A.2.2-1: For 2.8 km inter-site distance with 20 MHz channel bandwidth

Similar observations can be made from the results in Figure A.2.2-1, namely that the CDFs of the 23 dBm UE and the 31 dBm UE are identical until the UE reach their maximum output power, more (~6% of 23 dBm UE and ~1% of 31 dBm UE) of the UE population transmitted with their maximum output power with the more aggressive Set 1, and the 37 dB ACLR of the 31 dBm UE (currently specified for power class 1 UE) will ensure that the victim system performance degradation due to 31 dBm interfering UE is not larger than that due to 23 dBm interfering UE.

#### A.2.3	6 km inter-site distance

The CDFs of the UE transmit power as well as the victim system UL throughput loss Vs ACLR offset (with different power control parameter sets) for 6 km inter-site distance with 20 MHz channel bandwidth are shown in Figure A.2.3-1 below.

| ACLR offset X [dB] | 0 | 7 |
| --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 1.12% |  |
| 5%-tile throughput loss (23 dBm interfering UE) | 0.80% |  |
| Average throughput loss (31 dBm interfering UE) | 1.24% | 0.52% |
| 5%-tile throughput loss (31 dBm interfering UE) | 1.16% | 0.30% |

(a) With Power Control Parameter Set 1

| ACLR offset X [dB] | 0 |
| --- | --- |
| Average throughput loss (23 dBm interfering UE) | 0.42% |
| 5%-tile throughput loss (23 dBm interfering UE) | 0.62% |
| Average throughput loss (31 dBm interfering UE) | 0.42% |
| 5%-tile throughput loss (31 dBm interfering UE) | 0.62% |

(b) With Power Control Parameter Set 2

Figure A.2.3-1: For 6 km inter-site distance with 20 MHz channel bandwidth

Similar observations can be made from the results in Figure A.2.3-1, namely that the CDFs of the 23 dBm UE and the 31 dBm UE are identical until the UE reach their maximum output power, more (~6% of 23 dBm UE and ~1% of 31 dBm UE) of the UE population transmitted with their maximum output power with the more aggressive Set 1, and the 37 dB ACLR of the 31 dBm UE (currently specified for power class 1 UE) will ensure that the victim system performance degradation due to 31 dBm interfering UE is not larger than that due to 23 dBm interfering UE.

#### A.2.4	8 km inter-site distance

The CDFs of the UE transmit power as well as the victim system UL throughput loss Vs ACLR offset (with different power control parameter sets) for 8 km inter-site distance with 20 MHz channel bandwidth are shown in Figure A.2.4-1 below.

| ACLR offset X [dB] | 0 | 7 |
| --- | --- | --- |
| Average throughput loss (23 dBm interfering UE) | 1.21% |  |
| 5%-tile throughput loss (23 dBm interfering UE) | 1.29% |  |
| Average throughput loss (31 dBm interfering UE) | 1.32% | 0.58% |
| 5%-tile throughput loss (31 dBm interfering UE) | 1.55% | 0.56% |

(a) With Power Control Parameter Set 1

| ACLR offset X [dB] | 0 |
| --- | --- |
| Average throughput loss (23 dBm interfering UE) | 0.37% |
| 5%-tile throughput loss (23 dBm interfering UE) | 0.49% |
| Average throughput loss (31 dBm interfering UE) | 0.37% |
| 5%-tile throughput loss (31 dBm interfering UE) | 0.49% |

(b) With Power Control Parameter Set 2

Figure A.2.4: For 8 km inter-site distance with 20 MHz channel bandwidth

Similar observations can be made from the results in Figure A.2.4-1, namely that the CDFs of the 23 dBm UE and the 31 dBm UE are identical until the UE reach their maximum output power, more (~5% of 23 dBm UE and ~1% of 31 dBm UE) of the UE population transmitted with their maximum output power with the more aggressive Set 1, and the 37 dB ACLR of the 31 dBm UE (currently specified for power class 1 UE) will ensure that the victim system performance degradation due to 31 dBm interfering UE is not larger than that due to 23 dBm interfering UE.

#### A.2.5	BS received signal power

The 99.99%-tile of the victim BS received signal power for the simulated 31 dBm UE cases with 20 MHz channel bandwidth are summarized in Table A.2.5-1 below. It can be seen that the 99.99%-tile received signal power in all simulated cases, except with the more aggressive Set 1 for 0.75 km inter-site distance, are lower than the current -43 dBm in-band blocking requirements specified in RAN4 specifications for wide-area BS. In the exception case, site engineering solutions (e.g., larger distance between victim BS and interfering FWA UE, better RF filtering in the victim BS receiver chain) will be required to ensure satisfactory coexistence between the victim BS and interfering 31 dBm UE.

Table A.2.5-1: 99.99%-tile victim BS received signal power with 20 MHz channel bandwidth

| Power control parameters | 0.75 km inter-site distance | 2.8 km inter-site distance | 6 km inter-site distance | 8 km inter-site distance |
| --- | --- | --- | --- | --- |
| Set 1 | -38.9983 | -47.0811 | -49.2813 | -50.0217 |
| Set 1’ | -43.7443 |  |  |  |
| Set 2 | -57.1937 | -60.0083 | -62.8238 | -62.6169 |

<End of change>

# References

[1]	RP-223424, “Revised WID: High-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands”, Nokia.

[2]	R4-2303445, “TP to TR 37.829: System level simulation methodology and assumptions for coexistence study on 31dBm UE Power Class for LTE Band 41 and NR Band n41”, Nokia, Nokia Shanghai Bell.

[3]	R4-2300190, “System level simulation results for coexistence study on 31dBm UE Power Class for LTE Band 41 and NR Band n41”, Nokia, Nokia Shanghai Bell.

[4]	R4-2300419, “TR 37.829 v0.4.0”, Nokia.