3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304139

# Online, April 17 – April 26, 2023

Agenda item:	5.19.1.2

Source:	Apple

Title:	On test parameters and simulation results for MU-MIMO

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#106 PDSCH demodulation requirements for MU-MIMO with advance receiver were discussed and way forward [1] was agreed.  In this contribution we present our simulation results and views on test parameters for advanced receiver in MU-MIMO.

# 2. Simulation Results

In RAN4#106 simulation parameters were agreed for initial simulation alignment and phase I for advanced receiver for MU-MIMO study.

The simulation assumptions for the results we present are summarized below:

- Ant config and rank for 2 paired UEs:

  - 2x2: 1+1

  - 4x4: 2+2

- DMRS ports:

  - 2x2: 1+1 – [0,1]

  - 4x4: 2+2 – [(0,1),(2,3)]

- Precoder selection for interference UE

  - Orthogonal

  - Random and unique (for 2x2 only)

- Codebook

  - SP Type I codebook

  - Random PMI

- Channel and antenna correlation

  - Rank 1+1: ULA medium

  - Use TDLC300-100 when the rank of the target UE is 1

  - Use TDLA30-10 and TDLC300-100 when the rank of the target UE is 2

- MCS for target UE

  - Rank 1: 16QAM (MCS13)

  - Rank 2: 16QAM (MCS13)

- MCS of co-scheduled UE

  - For rank 1+1: QPSK (high priority for the next meeting)

  - For rank 2+2: 64QAM (high priority for the next meeting)

  - For rank 2+2: QPSK (high priority for the next meeting)

2x2

The results below are for 2x2 antenna configuration with 1+1 layer combination:

Figure 1: Simulation results for 2x2

For 2x2 with medium correlation the gain with R-ML is high, as expected. The performance of MMSE-IRC and E-MMSE-IRC is comparable. Also, the degradation with random precoder is ~3dB with IRC receivers in medium channel correlation, where as ~1dB with R_ML receiver

Observation #1: The gain with R-ML receiver is high with medium correlation with 2x2.

Observation #2: Performance with E-IRC and baseline MMSE-IRC receiver is comparable with 2x2.

Observation #3: With random precoder in medium correlation performance degradation of 3dB with IRC receivers and ~1dB with R-ML receiver in 2x2.

4x4

For 4x4 antenna configuration we present results for 2+2 layer combination with orthogonal precoder for co-scheduled UE.

TDL-A Channel

Figure 2: Simulation results for 4x4 with TDLA channel

With 2+2 TDL-A channel, low correlation, the gain with R-ML receiver is seen with co-scheduled UE using QPSK. Also, the gain with E-IRC is not much with MCS13 for target UE.

Observation #4: With orthogonal precoders, with TDL-A channel and 2+2 gain with R_ML receiver is observed when co-scheduled UE is using QPSK.

Observation #5: Gain with E-IRC receiver is not significant with MCS 13 with TDL-A channel and 2+2.

TDL-C Channel

Figure 3:  Simulation results for 4x4 with TDLC channel

With TDL-C channel, low correlation, 2+2 larger gain is observed with co-scheduled UE with QPSK. The gain with E-IRC is not significant with MCS13 on target UE.

Observation #4: With orthogonal precoders, with TDL-C channel and 2+2 larger gain with R-ML receiver is observed when co-scheduled UE is using QPSK.

Observation #5: Gain with E-IRC receiver is not significant with MCS 13 with TDL-C channel and 2+2.

We summarize the simulation results in the table below:

Table 1: Summary of simulation results

| Rank Combo | Modulation of co-sch UE | MIMO | Channel Model | Precoder of co-sch UE | SNR @ 70% Max TP |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | MMSE-IRC | E-IRC | R-ML |
| 1+1 | QPSK | 2x2 ULA Med | TDLC300-100 | Orthogonal | 21.4 | 21.3 | 15.4 |
|  |  |  |  | Random | 24.3 | 24.2 | 16.4 |
| 2+2 | QPSK | 4x4 ULA Low | TDLA30-10 | Orthogonal | 14.5 | 14.1 | 11.8 |
|  | 64QAM |  |  | Orthogonal | 14.5 | 14.1 | 14.1 |
|  | QPSK |  | TDLC300-100 | Orthogonal | 17.1 | 16.7 | 13.3 |
|  | 64QAM |  |  | Orthogonal | 17.1 | 16.7 | 15.5 |

# 3. Test Parameters

In RAN4#106, there was an agreement to simulate case of 1+3 rank combination with 2x2. For DMRS length 1 the rank 1+3 combo is such that the co-scheduled UE uses 2 CDM groups. Such DMRS port combination across 2 CDM groups is not typically used for MU-MIMO scheduling. We propose to not consider the case of 1+3 and instead consider 1+2 where each UE can be scheduled in 1 CDM group each and on different CDM groups.

Observation #6: Rank 1+3 combination used DMRS ports across 2 CDM groups for co-scheduled UE. Such DMRS port combination is not used for MU-MIMO scheduling.

Proposal #1: Use rank combination 1+2 with 4x4 channel for further evaluation.

So far, the MCS for target UE has been MCS13. We would like to propose to also consider MCS17/19 in 64QAM range to further evaluate performance with advanced receivers.

Proposal #2: Further evaluate performance with MCS 17 or MCS19 for target UE.

# 4. Conclusion

In this paper, we provide present our simulation results and views on test parameters for advanced receiver in MU-MIMO. Our observations and proposals are captured below:

Simulation Results

2x2 TDLC channel:

Observation #1: The gain with R-ML receiver is high with medium correlation with 2x2.

Observation #2: Performance with E-IRC and baseline MMSE-IRC receiver is comparable with 2x2.

Observation #3: With random precoder in medium correlation performance degradation of 3dB with IRC receivers and ~1dB with R-ML receiver in 2x2.

4x4:

Observation #4: With orthogonal precoders, with TDL-A channel and 2+2 gain with R_ML receiver is observed when co-scheduled UE is using QPSK.

Observation #5: Gain with E-IRC receiver is not significant with MCS 13 with TDL-A channel and 2+2.

Observation #4: With orthogonal precoders, with TDL-C channel and 2+2 larger gain with R-ML receiver is observed when co-scheduled UE is using QPSK.

Observation #5: Gain with E-IRC receiver is not significant with MCS 13 with TDL-C channel and 2+2.

Test Parameters:

Observation #6: Rank 1+3 combination used DMRS ports across 2 CDM groups for co-scheduled UE. Such DMRS port combination is not used for MU-MIMO scheduling.

Proposal #1: Use rank combination 1+2 with 4x4 channel for further evaluation.

Proposal #2: Further evaluate performance with MCS 17 or MCS19 for target UE.

# Reference

- R4-2302929, “WF on advanced receiver for MU-MIMO scenario”, China Telecom.