3GPP RAN WG4 Meeting #106-bis-e		R4-2304163

Electronic meeting, April 17 - 26, 2023

Agenda item:	5.8.2.1

Source:	Murata Manufacturing Co., Ltd.

Title:	Discussion on simulation assumption for FR2-1 multi-Rx

Document for:	Discussion

- Introduction

In RAN4#106, we discussed about simulation assumptions for evaluation and UE assumption ([1]).

In this contribution, we share our view on UE assumptions.

- Discussion

2.1	AoA separation for UE RF requirement

In RAN4#106 we agreed bellow ([1] 1.2.6).

Agreement: n AoA offset value should be an integer multiple of the step size of the constant step size measurement test grid. n In the simulation, all AoA separation values in the list {30°, 60°, 90°, 120°, 150°, 180°} shall be simulated.

From [2]-[5] if AoA offset become larger, CDF seems become better. Figure 1 shows its example.

Figure 1. The CDF of multi-Rx with changing AoA offset (places of panels are top and right)

Observation 1:	If AoA offset become larger, CDF seems become better.

In Figure 1, there are small difference of normalized gain at 50%-tile CDF from 90 to 180 degrees AoA offset. However, in 30 or 60 degrees AoA offset, the normalized gain at 50%-tile CDF is degraded compared with the result of over 90 degrees AoA offset. We concern that specification will be degraded when it is estimated under the condition of AoA offset = 30 degrees. In RAN4#106, there was a proposal using MMSE-IRC as receiver assumptions ([1] 1.2.11). This technique will be able to improve specification. So, we tried evaluating the effect of MMSE-IRC under the worst condition of AoA offset = 30 degrees.

In adopting MMSE-IRC to beamforming, we define elevation and azimuth as Figure 2 in this T-doc. Antenna panel can control the beam only on the elevation direction, so we need calculating the angle for MMSE-IRC on the antenna panel. We assume elevation angle as written in Figure 2 in this T-doc.

Figure 2. The angle between TRP1 (blue) and TRP2 (red) and elevation angle image in this T-doc

In most cases the elevation angle is smaller than AoA offset, and TRP1 and TRP2 come from the same side to antenna panel like Figure 3.

Figure 3. The angle between TRP1 (blue) and TRP2 (red, purple) for MMSE-IRC

In this case it will be possible receiving one TRP at main lobe and receiving another TRP at null point by MMSE-IRC. However, if the elevation angle become too small to main lobe, it will be severe acquiring the merit of MMSE-IRC.

There would be a few cases where the elevation angle is larger than AoA offset. Figure 4 shows one example.

Figure 4. Example where the angle between TRP1 (blue) and TRP2 (red) for MMSE-IRC is larger than AoA offset.

In the Figure 4 case, it is possible receiving one TRP from front-side of the antenna panel, but it is severe receiving another TRP from back-side of the antenna panel. In this case, it may be severe making SINR better by MMSE-IRC.

Observation 2:	From the viewpoint of antenna panel, the angle between TRP1 and TRP2 can be both larger than and smaller than AoA offset.

As written above, the elevation angles for MMSE-IRC are different in each measurement grid. In this contribution, we focus on two examples. Figure 5 shows the example of heatmap without using MMSE-IRC.

Figure 5. Heat map without using MMSE-IRC

We assumed UE receives wanted signal (TRP1) at Phi = 90 degrees and Theta = 45 degrees. UE also receives interference signal (TRP2) at Phi = 90 degrees and Theta = 75 degrees (worse SINR case) or 15 degrees (better SINR case).

Figure 6 shows our simulation result of MMSE-IRC. In the worse SINR case, we could make SINR 5.1 dB better. However, in the better SINR case, we could make SINR only 0.8 dB better. This result suggests that the effect of MMSE-IRC is different among measurement grids, but MMSE-IRC may be able to improve CDF.

Figure 6. The simulation result of MMSE-IRC (Phi = 90 degrees)

Observation 3:	The effect of MMSE-IRC is different among measurement grids.

Observation 4:	For example, SINR become 0.9 or 5.1 dB better in this T-doc by MMSE-IRC.

Proposal 1:	We support using MMSE-IRC for the case the simulation result of AoA offset = 30 degrees much worse than results of other AoA offset cases.

In RAN4#106, there were also the discussion of MIMO correlation matrix ([6] Topic #1). In the small AoA offset case, 4x4 MIMO may have 4 dimensions and may be able to divide signal and interference signal. However, we believe that we do not need considering MIMO correlation matrix in discussing specification. Because we agreed “The UE RF requirement is derived assuming the worst-case polarization match between the 2 TRPs.” ([1] 1.2.5), and the MIMO correlation matrix will make polarization match to better case.

2.2	UE simulation assumptions

In RAN4#106, there were discussion on UE simulation assumptions ([1] 1.2.12, 1.2.15).

We believe we should use parameters used in legacy discussion (ex. [7]). Because if we use other parameters for assumption, legacy specification will be better. This is not the result of using multi-Rx. So, when we compare the specification of multi-Rx with that of legacy, we should use the same parameters for simulation assumptions. And if we use other parameters in multi-Rx discussion, we should compare with one panel simulation using the same assumption.

For example, we assumed the resolution of phase shifter is 45 degrees in [7]. If we change the resolution of phase shifter smaller than 45 degrees, beam pattern will cover wider area than legacy UE and make CDF better. By this limitation, the result of MMSE-IRC will not be ideal.

In the multi-Rx discussion, the gain of LNA for MMSE-IRC would be added to parameter. If there is any limitation to LNA setup, companies should clarify.

Observation 5:	If we change parameters from legacy, CDF can be better without using multi-Rx.
(ex. better resolution of phase shifter will improve CDF of legacy UE.)

Proposal 2:	When we compare the specification of multi-Rx with that of legacy, we should use the same parameters with legacy. And if we use other parameters, we should compare the specification of multi-Rx with that of one panel simulation using the same assumption.

In multi-Rx discussion, there are too many parameters, so it is feasible limiting simulation assumption like [1] 1.2.12 and 1.2.15. However, we want to add plastic to side cover assumption, because some legacy smartphones have antenna on side.

Table 1. UE packaging simulation assumption

| Item | Simulation assumption | Note |
| --- | --- | --- |
| Front cover (Plastic, Glass, Ceramic, Metal) | Glass | This information is meaningful only if it’s the same with the material which covers antennas. |
| Back cover (Plastic, Glass, Ceramic, Metal) | Glass |  |
| Side cover / Frame (Plastic, Glass, Ceramic, Metal) | Metal, Plastic |  |
| Display panel – Full (Y) or Partial (N) | Y |  |
| Bezel Margin | 1.5mm | Module can’t be placed outer edge of UE to secure mechanical reliability |

Observation 6:	Some legacy smartphones have antenna on side.

Proposal 3:	Add plastic to side cover assumption.

- Conclusions

Observation 1:	If AoA offset become larger, CDF seems become better.

Observation 2:	From the viewpoint of antenna panel, the angle between TRP1 and TRP2 can be both larger than and smaller than AoA offset.

Observation 3:	The effect of MMSE-IRC is different among measurement grids.

Observation 4:	For example, SINR become 0.9 or 5.1 dB better in this T-doc by MMSE-IRC.

Proposal 1:	We support using MMSE-IRC for the case the simulation result of AoA offset = 30 degrees much worse than results of other AoA offset cases.

Observation 5:	If we change parameters from legacy, CDF can be better without using multi-Rx.
(ex. better resolution of phase shifter will improve CDF of legacy UE.)

Proposal 2:	When we compare the specification of multi-Rx with that of legacy, we should use the same parameters with legacy. And if we use other parameters, we should compare the specification of multi-Rx with that of one panel simulation using the same assumption.

Observation 6:	Some legacy smartphones have antenna on side.

Proposal 3:	Add plastic to side cover assumption.

| Item | Simulation assumption | Note |
| --- | --- | --- |
| Front cover (Plastic, Glass, Ceramic, Metal) | Glass | This information is meaningful only if it’s the same with the material which covers antennas. |
| Back cover (Plastic, Glass, Ceramic, Metal) | Glass |  |
| Side cover / Frame (Plastic, Glass, Ceramic, Metal) | Metal, Plastic |  |
| Display panel – Full (Y) or Partial (N) | Y |  |
| Bezel Margin | 1.5mm | Module can’t be placed outer edge of UE to secure mechanical reliability |

- References

R4-2303708, “WF on NR FR2 multi-Rx chain DL reception,” Samsung, Apple

R4-2300146, “Discussion for FR2 multi-Rx FOM,” Murata

R4-2300268, “RF requirement for NR FR2 multi-Rx chain DL reception,” Apple

R4-2300949, “Discussion on UE RF requirements for simultaneous DL reception,” LG Electronics

R4-2301572, “Evaluation on UE requirement of multiRx DL reception”, vivo

R4-2302941, “WF for UE demodulation and CSI requirements of Rel-18 NR FR2 multi-Rx chain DL reception,” Qualcomm

R4-1801202, “WF on EIRP CDF for spherical coverage,” Samsung, MediaTek