3GPP TSG-RAN WG4 Meeting # 106bis-e                             R4-2304170

Electronic Meeting, 17th - 26th April 2023

Agenda item:	5.24.3.1

Source: 	ZTE Corporation

Title: 	DL interruption for Tx switching across 3/4 bands

Document for:	Approval

Introduction

Multi-carrier enhancements for NR was discussed and the WF [1] was approved during RRM sessions on RAN4#106. In this contribution, we are discussing the various remaining issues in WF.

Discussion

During RAN4#106, there were agreements on DL interruption length for 2TAGs case when RTD not lager than 9us and don’t define RRM requirement for RTD>9us. Meanwhile, there are some issues need further study. The outstanding issues of the meeting we want to discuss are duplicated below.

<Way Forward>: Issue 1-3: Consideration on the scenario that one downlink carrier is indicated to be interrupted by two band pairs for dynamic switching simultaneously - Proposals - Option 1(Huawei): If one downlink carrier is indicated to be interrupted by two band pairs for dynamic switching simultaneously, the DL interruption length and location on the victim carrier is determined by the maximum of uplink switching periods of the two band pairs. - Option 2 (QC): If one downlink carrier is indicated to be interrupted by two band pairs for dynamic switching simultaneously, the DL interruption length and location on the victim carrier is determined by the union of uplink switching periods of the two band pairs <Way Forward>: Issue 1-4: Additional signalling for Tx switching with 2 TAGs - Proposals - Option 1(Nokia): UE shall inform network the RTD value or at least if the RTD side condition is fulfilled. <Way Forward>: Issue 1-5: Minimum Separation Time between two consecutive UL Tx Switching - Proposals - Option 1(Nokia): - The UE shall not perform UL Tx switching if it has performed an UL Tx switching within MinSeparationTime. - The UE shall not perform UL Tx switching if it has performed an UL Tx switching within MinSeparationTimeforMultiTAGs in multi-TAGs scenario. Encourage companies to check and identify whether there are impacts on RAN4 RRM.

Regarding the DL interruption length, the scenario that one downlink carrier is indicated to be interrupted by two band pairs for dynamic switching simultaneously needs further consideration. Before discussing DL interruption length, we need to analyze the uplink switching periods under this scenario.

For simultaneous dynamic switching, we need to discuss this issue in different cases. For example, two band pairs partially overlap or completely overlap. According to our understanding, it is reasonable to choose the maximum uplink switching periods of the two band pairs for the case of complete overlap. Meanwhile, for the case of partial overlap, the definition of maximum or union of uplink switching periods of the two band pairs may depend on UE capability.

- Proposal 1: For the scenario that one downlink carrier is indicated to be interrupted by two band pairs for dynamic switching simultaneously, the DL interruption length and location on the victim carrier is determined by the maximum of uplink switching periods of the two band pairs for the case of complete overlap.

- RTD exists to complete the calculation of the downlink interrupt length (without changing the value of the original protocol MRTD). Meanwhile, for a few scenarios with RTD>9us, it can be avoided through appropriate network scheduling. Therefore, the network should konw what the RTD value is and choose the network scheduling base on it.

Proposal 2: UE shall inform network the RTD value or at least if the RTD side condition is fulfilled.

RAN1#111: - (working assumption) If two uplink switching are triggered and result in UL transmissions on more than 2 bands within any two consecutive reference slots, then the time duration between the end of all transmission(s) prior to the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a sum of X us and the switching gap required for the second uplink switching. - X us is subject to UE capability with a value set of {0us, 500us}

Regarding the minimum separation time, working assumption was concluded at RAN1#111 meeting. And depiction of the minimum separation time between two consecutive UL Tx switching is shown in Figure 1 [2].

Figure1. Depiction of minimum separation time between two consecutive UL Tx switching

In our understanding, two consecutive UL Tx switching need to meet the minimum separation time requirement. In other words, if the minimum separation time is not met, the UE should not perform UL Tx switching.

Proposal 3: The UE shall not perform UL Tx switching if the minimum separation time is not satisfied.

Conclusions

In this contribution, we provide our views on the RRM impacts for enhanced Redcap UE and the proposals are:

- Proposal 1: For the scenario that one downlink carrier is indicated to be interrupted by two band pairs for dynamic switching simultaneously, the DL interruption length and location on the victim carrier is determined by the maximum of uplink switching periods of the two band pairs for the case of complete overlap.

- Proposal 2: UE shall inform network the RTD value or at least if the RTD side condition is fulfilled.

- Proposal 3: The UE shall not perform UL Tx switching if the minimum separation time is not satisfied.

References

- R4-2303188,WF on RRM requirements for MC enhancements for NR, Huawei, HiSilicon

- R4-2301359, Others for Tx switching across 3/4 bands, Nokia, Nokia Shanghai Bell