3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2304178

Electronic meeting, April 17-26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	L1 part enhancement for FR2 SCell activation

Agenda item:	5.9.2.2

Document for:	Discussion

Introduction

At RAN4#106 meeting, the down scoping of candidate solutions was discussed and the following table was agreed in the WF for FR2 unknown SCell activation enhancement [1]. In this contribution, we will continue the discussion on the issues remaining in the scope.

| L1 part | Issue 2-1-1 | Beam sweeping factor enhancement in L1-RSRP measurement (not related with WI of FR2 multi-Rx chain DL reception) |  | Yes, continue discussion |
| --- | --- | --- | --- | --- |
|  | Issue 2-1-2 | Skip L1-RSRP measurement of FR2 unknown SCell activation |  | Yes, continue discussion |
|  | Issue 2-1-3 | Prioritization enhancement for L1-RSRP measurement | L1-RSRP measurement is performed in non-DRX mode even DRX is configured. | Yes, continue discussion |
|  |  |  | Whether prioritize L1 over L3 is under discussing | Deprioritized |
|  | Issue 2-2-1 | Skip fine timing tracking for SSB corresponding to the TCI state |  | Deprioritized |
|  | Issue 2-2-2 | TCI activation enhancement |  | Deprioritized |
|  | Issue 2-3-1 | Aperiodic RS for TFineTiming |  | Yes, continue discussion |
|  | Issue 2-3-2/ Issue 2-4-2 | Aperiodic RS for L1-RSRP measurement |  | Deprioritized |

Discussion

In RAN4#105, it was agreed to discuss below sub-cases to reduce FR2 SCell activation delay for unknown case. The L1 part enhancement need also be discussed considering the two scenarios separately.

RAN4#105 Agreement

- FFS whether to define new sub-states for known/unknown state for FR2 SCell

- Discuss the following sub-cases to reduce FR2 SCell activation delay for unknown case

  - UE reports [a valid L3 measurement result] after SCell activation command

  - UE does not report [a valid L3 measurement result] after SCell activation command

  - Note: the categorization above does not limit potential enhancements

If the UE is able to report a valid L3 measurement result after SCell activation command, this provides network the latest beam information as the UE does before SCell activation command for a known SCell. Hence the SCell can be activated as if it is known. As illustrated in Fig.1, network can send the TCI activation command in response to the valid L3 measurement reporting therefore L1-RSRP measurement and reporting are not needed [2].

Fig.1 Skipping L1-RSRP if UE reports a valid L3 measurement after SCell activation command

Proposal 1: If UE reports a valid L3 measurement result after SCell activation command, L1-RSRP measurement and reporting are not needed.

Otherwise, if the UE cannot send a valid L3 measurement report, the UE may fall back to legacy unknown SCell activation and beam sweeping factor reduction can be considered. In last meeting, the following is agreed to allow the UE to indicate the capability in support of a smaller Rx beam sweeping factor X1 for cell detection (X1*Trs) and X2 for SSB based L1-RSRP measurement. It is FFS if the two factors need to be indicated separately or one value in combination of the two would be sufficient.

Beam sweeping factor reduction

Issue 1-2-1: Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

Issue 2-1-1: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

Agreement:

  - For unknown FR2 SCell activation enhancement, introduce the UE capability to support Rx beam sweeping factor can be less than 8 for cell detection part of L3 and SSB based L1-RSRP measurement.

    - if UE has full set (N=8) of beam sweeping during AGC settling part in L3.

      - Beam sweeping factor capability of X1 for cell detection part (X1*Trs) of L3 and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement

      - FFS on capability indication for X1 and X2

From network point of view, the L3 part and L1-RSRP measurement are to acquire the beam information of the to-be-activated SCell. What the network expects is the valid L1-RSRP report which is used to determine the TCI state and it is not aware how long the UE spends in each of the activation steps. If the UE indicates X1 and X2 separately or (X1+X2) makes no difference to the network, as in the end the expected time to receive the L1-RSRP reporting are the same (as shown in Fig.2). However, what matters is the performance of the valid L1-RSRP report. When a smaller beam sweeping factor is assumed to reduce the SCell activation delay, the performance of L1-RSRP report shall not be degraded. The L1-RSRP report based on a smaller beam sweeping factor shall fulfil the performance requirements as specified in TS38.133 clause 10.1.20.1.

Fig.2 Smaller beam sweeping factor for unknown SCell activation

Observation #1: What the network expects is the valid L1-RSRP report which is used to determine the TCI state.

Observation #2: If the UE indicates X1 and X2 separately or (X1+X2) makes no difference to the network, as in the end the expected time to receive the valid L1-RSRP reporting are the same.

Proposal 2: The L1-RSRP reporting based on a smaller beam sweeping factor shall fulfil the performance requirements as specified in TS38.133 clause 10.1.20.1.

Skipping L1-RSRP measurement

Besides a smaller beam sweeping factor, there are also proposals to skip L1-RSRP measurements and/or reporting. In our views, as the UE has completed AGC, cell search and T/F sync before L1-RSRP measurement, it is possible to derive the L1-RSRP report based on the measurements during L3 part steps. In some cases e.g. if the reference signals configured for L1-RSRP reporting has been measured during L3 part, or is QCL-typeD with the SSB being measured during L3 part, the L1-RSRP measurement can be skipped.

Proposal 3: The L1-RSRP measurement can be skipped if the reference signals configured for L1-RSRP reporting has been measured during L3 part, or is QCL-typeD with the SSB being measured during L3 part.

When L1-RSRP measurement is skipped, we believe L1-RSRP reporting (or some other message alike) is needed to inform network the beam information of the SCell. As the value to be reported is not measured via Rx beam sweeping, the UE shall indicate the L1-RSRP reporting is based on L3 measurements to differentiate from the legacy L1-RSRP reporting.

Proposal 4: L1-RSRP reporting cannot be skipped. It is needed to inform network the beam information of the SCell.

Proposal 5: The UE shall indicate the L1-RSRP reporting is derived by skipping L1-RSRP measurement.

Aperiodic RS for TFineTiming

One more issue remaining in the scope of L1 part enhancement is aperiodic RS for TFineTiming during unknown FR2 SCell activation. There was proposal in last meeting to clarify what A-TRS means.

Issue 2-3-1: Aperiodic RS for TFineTiming during FR2 unknown SCell activation

- Proposals (vivo)

  - Clarify that ‘A-TRS’ agreed in last meeting as the A-TRS for Fast SCell activation.

  - Clarify that ‘A-TRS’ agreed in last meeting can not be configured as QCL source for CSI-RS for CQI and PDCCH/PDSCH.

In TS38.133, TFineTiming is defined as cited below. This time period starts after TCI activation command has been received and processed, hence only one complete SSB is needed for the fine time tracking. In our understanding, this fine time tracking is the same as the AGC and cell search steps when fast SCell activation is applied. Therefore, the same aperiodic CSI-RS resources i.e. A-TRS as specified for fast SCell activation can be used also for fine time tracking.

TFineTiming is the time period between UE finish processing the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and the timing of first complete available SSB corresponding to the TCI state.

Proposal 6: The A-TRS as specified for fast SCell activation can be used also for fine time tracking.

Conclusion

This contribution discusses the L1 part enhancement for FR2 SCell activation delay reduction. The observations and proposals are summarized as below:

Proposal 1: If UE reports a valid L3 measurement result after SCell activation command, L1-RSRP measurement and reporting are not needed.

Observation #1: What the network expects is the valid L1-RSRP report which is used to determine the TCI state.

Observation #2: If the UE indicates X1 and X2 separately or (X1+X2) makes no difference to the network, as in the end the expected time to receive the valid L1-RSRP reporting are the same.

Proposal 2: The L1-RSRP reporting based on a smaller beam sweeping factor shall fulfil the performance requirements as specified in TS38.133 clause 10.1.20.1.

Proposal 3: The L1-RSRP measurement can be skipped if the reference signals configured for L1-RSRP reporting has been measured during L3 part, or is QCL-typeD with the SSB being measured during L3 part.

Proposal 4: L1-RSRP reporting cannot be skipped. It is needed to inform network the beam information of the SCell.

Proposal 5: The UE shall indicate the L1-RSRP reporting is derived by skipping L1-RSRP measurement.

Proposal 6: The A-TRS as specified for fast SCell activation can be used also for fine time tracking.

References

- R4-2303228, WF on R18 eFeRRM, Apple

- R4-2304177, L3 part enhancement for FR2 SCell activation delay reduction, Nokia