3GPP TSG RAN WG4 #106 BIS-e meeting 	 R4-2304182

Electronic Meeting, 17th – 26th April, 2023

Agenda Item:	5.5.2.3.1

Source:	Meta Ireland

Title:	Updated the lower MSD capability signalling

Document for:	Approval

- Introduction

In RAN4 #106 meeting, RAN4 concluded that lower MSD is feasible [1] and reported to RAN as follow

Conclusion proposal for the study phase of lower MSD in RAN#99

- The feasibility for MSD improvement for all kinds of MSD has been confirmed based on the evaluation from companies on the selected example band combinations.

- Several promising options for allowing a UE to signal improved lower MSD performance have been discussed, with which companies think it is feasible to introduce the lower MSD capability.

- Details of the lower MSD capability will be further discussed in the UE RF FR1 WI.

Also, RAN approved the updated objectives in the WI [2] for FR1 RF further enhancements. Specially RAN2 WG was involved to specify the lower MSD capability signalling. The remaining open issues will be continually discussed and addressed in 2nd phase discussion in the WI based on the approved the WF [1] as follow:

Issue 2-1-2: Conditions to indicate the lower MSD capability Option 1: UE could indicate Lower MSD capability for a band combination as long as one kind of MSD from one victim band is improved. Additionally, it is unnecessary to report the Lower MSD values in case the specified MSD itself is small or the MSD improvement is not significant. However, if UE is willing to report the values under these cases, it should not be prohibited. Option 2: measures must be discussed to avoid a situation that UE can report lower MSD capability without any actual improvement. Option 3: UE report improved MSD as long as the real MSD is below the largest value range, for example 20dB. The band combinations with less than [5dB] MSD requirements in the spec doesn’t need to report the improved MSD. Option 4: Others - WF - UE could indicate Lower MSD capability for a band combination as long as one kind of MSD from one victim band is improved. - The amount of MSD improvement necessary for indication needs further study Issue 2-1-4: Other information suggested for the lower MSD capabilty Option 1: In order to facilitate the network to estimate the actual self-interference level at the UE (based on Path Loss, CSI, etc), allow and enable the UE to report the ratio of MSD reduction to Tx power reduction. (HW) Option 2: A way to indicate MSD = 0 dB region(s) on top of lower MSD capability following the conventional MSD test configuration (Nokia) Option 3: Define a single-bit low-MSD indicator for a UE to signal to the network that all MSDs related to a given band combination is ≤ [5]dB (HW) Option 3a: A joint solution of one bit low MSD indication per BC with the per victim band per MSD type per band combination signaling, one bit low MSD indication can be used if all MSD types for this BC have been improved to above a threshold. (CHTTL) Option 4: Others - WF – FFS in next meeting. Issue 2-1-6: Orders for IMD MSD to be reported Option 1: Orders aligned with MSD requirements and test points (Samsung). - For one band combination with 2CC as UL, when multiple IMD occurs for one victim band within the band combination, maximum two IMD orders are allowed in terms of Lower MSD information reporting, among which the lowest order is mandatory and one other higher order IMD could be optionally included. - For one band combination with 3CC as UL, only the lowest order IMD (triple beat) is considered for the victim band in terms of Lower MSD information reporting. - The selected IMDs should be with the same UL/DL configurations and test points as for the minimum requirements. Option 1a: Slightly different alternative compared with option 1 (ZTE) - For one band combination with 2CC as UL: - When the 2CC configured with intra-band UL CA configured in one of the two band, the lowest order IMD is recommended - When the 2CC configured with 1UL CC in each of UL band, if multiple IMD orders occur per victim band, the lowest order IMD is recommended as worst case to represent the whole spectrum of the inter-band CA combinations. Optionally, a second MSD test point corresponds to the lowest even and the lowest odd order IMD. - For one band combination with 3CC as UL: - 1st order triple-beat product. Option 2: Only define and verify the lowest order of IMD in case of multiple orders exist for a band combination for Low MSD capability (vivo, Xiaomi) Option 3: No limitation on the reported orders (Nokia, CMCC, MTK) - if there is no explicitly MSD relation between lower order IMD and higher order IMD, all orders are suggested to be report since lower order MSD can’t cover all the victim RBs caused by higher order IMD (CMCC) Option 4: For a given band combination the UE can declare the low MSD capability separately for each impairment (i.e. IMD2, IMD4, HD2, HD3, Rx LO H2 etc.) where the UE performs better than in the current standard. (Qualcomm) Option 5: Others - WF - Consider together with issue 2-1-5 Issue 2-1-7: Candidate MSD thresholds Option 1: Two bits threshold range. 0/5/10/15dB as PC3 thresholds applicable for all kinds of MSD, while 3dB could be considered as the offset vs power class. (Samsung) Bit; Maximum allowed actual MSD (i.e. Thresholds); Lower MSD Capability classes; Note. 00; 0dB; Ⅰ; Actual MSD = 0. 01; 5 dB; Ⅱ; 0 ＜ Actual MSD ≤ 5. 10; 10 dB; Ⅲ; 5 ＜ Actual MSD ≤ 10. 11; 15 dB; IV; 10 ＜ Actual MSD ≤ 15. Option 2: Three bits threshold range. (Meta) Bit map; Maximum allowed actual MSD (i.e. Thresholds); Lower MSD Capability classes; Note. 000; -; Not supported the lower MSD optional capability; Not supported the lower MSD capability. Only apply the existing MSD requirements in TS38.101-1 and TS38.101-3.. 001; 3 dB; Ⅰ; 0 ≤ Actual MSD ≤ 3. 010; 6 dB; Ⅱ; 3 < Actual MSD ≤ 6. 011; 9 dB; Ⅲ; 6 < Actual MSD ≤ 9. 100; 12 dB; IV; 9 < Actual MSD ≤ 12. 101; 15 dB; V; 12 < Actual MSD ≤ 15. 110; 18 dB; VI; 15 < Actual MSD ≤ 18. 111; > 18 dB; VII; Actual MSD > 18. Option 3: OPPO - 0≤UE Real MSD＜5dB - 5≤UE Real MSD＜10dB - 10≤UE Real MSD＜15dB - 15≤UE Real MSD＜20dB Option 4: Xiaomi Threshold; Actual MSD range. 1; 3; 0 ≤ Actual MSD ≤ 3. 2; 6; 3 ＜ Actual MSD ≤ 6. 3; 12; 6 ＜ Actual MSD ≤ 12. 4; 18; 12 ＜ Actual MSD ≤ 18. Option 5: Small granularity and more higher upper limit thresholds should be considered for multiple thresholds, such as [0, 2.5, 5, 7.5, 10, 12.5, 15, 17.5, 20] dB. (ZTE) - WF - FFS in next meeting. Issue 2-1-8: Differentiation of the lower MSD capability for power classes Option 1: Consider 0/5/10/15dB as PC3 thresholds applicable for all kinds of MSD, while 3dB could be considered as the offset vs power class. Lower MSD capability is applicable for PC1.5, PC2 and PC3. Allow UE to report Lower MSD capability for different power classes. (Samsung) Option 2: identical thresholds to all aggressor power classes (HW, ZTE, vivo) Option 3: different thresholds per MSD PC are needed or any other measures must be discussed to avoid a situation that UE can report lower MSD capability without any actual improvement (Nokia) Option 4: Allow the UE to report the MSD value for a band combination at a given UL power level. The UL power level is requested by the network, and can be one of the filtering parameters during the capability query. The process may be executed once before a SCell is configured or activated, and can be viewed as an extension of reporting MSD per power class (i.e. MSD for 23/26/29dBm) but works more efficiently. (HW) - WF - FFS in next meeting.

In this paper, we propose how to solve the open issues and propose the detail reported lower MSD value range, MSD threshold and granularity for the lower MSD capability.

- Signalling for lower MSD capability

  - Other information for signalling of the lower MSD capability

In the last RAN4 meeting, some vendors requested to report the other parameters such as the ratio of MSD reduction, indication of the region of MSD =0, a bit indication of small MSD capability signalling (MSD <= 5dB) and individual power class.

The ratio of MSD reduction or individual indication of UE power class will be further discussed when the RAN4 completes the evaluation of the difference of MSD levels among different power classes UE.  The ratio of MSD reduction can be useful to distinguish the individual power class from 1.5 to power class 3 device to reduce signalling overhead based on the reported ratio of MSD reduction when RAN4 concludes to apply the different MSD threshold mechanism per each UE power class.

Also, a bit indication of the small MSD capability is useful to apply the real improved MSD level by high order IMD/harmonic problems.

The region of MSD =0dB can be considered in NW to schedule the dynamic RB allocation. But the regions of MSD =0dB can be different among UEs and expect high complexity in NW perspective compared to the real benefits in sensitivity.

Therefore, we provided two observations as follow,

Observation #1: The ratio of MSD reduction or individual indication of UE power class can be useful to apply the different MSD threshold according to the individual power class from 1.5 to power class 3 device to reduce signalling overhead based on the reported ratio of MSD reduction.

Observation #2: A bit indication of small MSD capability (MSD <= [3]dB) can be useful to apply the improved MSD level by the high order IMD/harmonic problems (e.g. 5th IMD, 5th Harmonic) in the inter-band CA/DC band combinations.

Based on these observations, we propose the following.

Proposal #1: RAN4 can consider the ratio of MSD reduction or the indication of individual UE power classes when RAN4 concludes to apply the different MSD threshold mechanism per each UE power class.

Proposal #2: Single bit indication of small MSD capability (MSD <= [3]dB) can be reported to apply the improved MSD level by the high order IMD/harmonic problems.

  - Reporting order of IMD/Harmonics for lower MSD

From the LTE CA/DC band combinations, RAN4 had some rules to define the MSD requirements for the high order CA/DC band combinations since the high-order CA/DC had the same problem as observed in the own DL bands by the dual uplink transmission. Hence, we prefer to follow the basic principle as following option 1 in WF [1] at last RAN4 #106 meeting.

Option 1: Orders aligned with MSD requirements and test points.

- For one band combination with 2CC as UL, when multiple IMD occurs for one victim band within the band combination, maximum two IMD orders are allowed in terms of Lower MSD information reporting, among which the lowest order is mandatory and one other higher order IMD could be optionally included.

- For one band combination with 3CC as UL, only the lowest order IMD (triple beat) is considered for the victim band in terms of Lower MSD information reporting.

- The selected IMDs should be with the same UL/DL configurations and test points as for the minimum requirements.

Option 4: For a given band combination the UE can declare the low MSD capability separately for each impairment (i.e. IMD2, IMD4, HD2, HD3, Rx LO H2 etc.) where the UE performs better than in the current standard.

Based on the consistency from LTE CA and NR DC/CA band combinations, we propose as follow,

Proposal #3: Above proposals in option 1 or option 4 will be applied to reporting orders in CA/DC band combinations to define the lower MSD capability signalling.

  - Candidate MSD thresholds for all MSD types

For the example CA band combinations in study phase, the achievable MSD levels are derived based on the results from the interested companies for different MSD sources and different orders within the same MSD source.

In MSD evaluation results in TR38.881 [4], the expected MSD improvements in case of 2nd harmonic product, cross band isolation and dual uplink transmissions are less than 10dB compared to the existing MSD requirements in TS38.101-1 and TS38.101-3. Also, the evaluation from interested companies results [4] showed that up to 20 dB MSD improvements were feasible. Based on these evaluation results, we propose to report the lower MSD capability as a compromise.

Table 1: MSD reporting bitmap for lower MSD capability

| Bit map | MSD range (i.e. Thresholds) | Note |
| --- | --- | --- |
| 000 | - | Not supported the lower MSD capability. Only apply the existing MSD requirements in TS38.101-1 and TS38.101-3. |
| 001 | 3 dB | 0 ≤ Actual MSD ≤ 3 |
| 010 | 6 dB | 3 < Actual MSD ≤ 6 |
| 011 | 9 dB | 6 < Actual MSD ≤ 9 |
| 100 | 12 dB | 9 < Actual MSD ≤ 12 |
| 101 | 15 dB | 12 < Actual MSD ≤ 15 |
| 110 | 18 dB | 15 < Actual MSD ≤ 18 |
| 111 | > 18 dB | Actual MSD > 18 |

Table 2: MSD reporting bitmap for lower MSD capability when a bit indication of small MSD capability is enabled (MSD <= [3]dB)

| Bit map | MSD range (i.e. Thresholds) | Note |
| --- | --- | --- |
| 00 | - | Not supported the lower MSD capability. Only apply the existing MSD requirements in TS38.101-1 and TS38.101-3. |
| 01 | [1] dB | 0 ≤ Actual MSD ≤ [1] |
| 10 | [2] dB | 1 < Actual MSD ≤ [2] |
| 11 | [3] dB | 2 < Actual MSD ≤ [3] |

Proposal #4: RAN4 introduces multiple MSD thresholds using 3-bit MSD reporting bitmap in Table 1 to apply lower capability for all CA/DC band combinations according to the different MSD types and different order within the same MSD source.

Proposal #5: When a single bit indication of small MSD capability is enabled(MSD <=[3]dB), RAN4 can use the 2-bit MSD reporting bitmap in Table 2.

  - The applicability of lower MSD capability signalling for different power class UE

To support lower MSD capability for both power class 3 and power class 2, RAN4 needs detailed evaluations on how to achieve lower MSD levels for different power classes. Currently, RAN4 agreed on a RF architecture restriction for the higher power devices such as PC2 NR inter-band CA UE (dual PA w/ 23+26 dBm or w/ 26+26 dBm) and PC1.5 NR inter-band CA (dual PA w/ 26+26 dBm). These PA/Antenna restrictions will make different MSD evaluation outcomes from that of power class 3 NR CA UE. Thus, RAN4 would need further discussion as case by case for an individual high power NR inter-band CA UE whether the same MSD threshold could apply or not.

Based on this, we propose to consider the same threshold mechanism for power class 3 to apply for a high-power NR CA band combinations UE as a starting point to introduce an MSD threshold for the capability of lower MSD of high-power NR CA UE. RAN4 could recommend a single difference MSD value which could be reported with a 3-bit MSD reporting bitmap in Table 1 as a starting point for high power NR CA UE. Based on this, we propose the following.

Proposal #6: RAN4 can consider the above 3-bit MSD reporting bitmap in Table 1 as a starting point for a high-power NR CA band combinations UE. If RAN4 concludes that different MSD mechanisms or MSD thresholds are needed, the ratio of MSD reduction or individual indication of UE power class can be considered.

- Conclusions

In this contribution, we provided our view on how to report the MSD levels, how to apply the MSD thresholds for the different MSD source, different order within the same MSD source and different power classes. Also, we prefer to consider the ratio of MSD reduction or indication of individual power class when RAN4 concludes that the different MSD mechanism or MSD threshold are needed.  Furthermore, if RAN4 considers the indication of small MSD capability (MSD <=[3]dB), the reported MSD thresholds in Table 2 are considered to apply actual improved MSD levels for the CA/DC band combination. Based on our discussion points for the remaining open issues, we proposed as follow

Proposal #1: RAN4 can consider the ratio of MSD reduction or the indication of individual UE power classes when RAN4 concludes to apply the different MSD threshold mechanism per each UE power class.

Proposal #2: Single bit indication of small MSD capability (MSD <= [3]dB) can be reported to apply the improved MSD level by the high order IMD/harmonic problems.

Proposal #3: Above proposals in option 1 or option 4 will be applied to reporting orders in CA/DC band combinations to define the lower MSD capability signalling.

Proposal #4: RAN4 introduces multiple MSD thresholds using 3-bit MSD reporting bitmap in Table 1 to apply lower capability for all CA/DC band combinations according to the different MSD types and different order within the same MSD source.

Proposal #5: When a single bit indication of small MSD capability is enabled (MSD <=[3]dB), RAN4 can use the 2-bit MSD reporting bitmap in Table 2.

Proposal #6: RAN4 can consider the above 3-bit MSD reporting bitmap in Table 1 as a starting point for a high-power NR CA band combinations UE. If RAN4 concludes that different MSD mechanisms or MSD thresholds are needed, the ratio of MSD reduction or individual indication of UE power class can be considered.

Reference

[1] R4-2303695, “WF on study for lower MSD,” Huawei

[2] RP-230753, “Revised WID on Further RF requirements enhancement for NR and EN-DC in FR1,” Huawei

[3] R4-2300040, “lower MSD signaling,” Nokia

[4] TR38.881: Study on lower MSD for inter-band CA/EN-DC/DC combinations

[5] R4-2300206, “Lower MSD capability signalling,” Meta

[6] R4-2302744, “Signalling for low MSD,” Qualcomm

[7] TS38.101-1

[8] TS38.101-3