3GPP TSG-RAN WG4 Meeting # 106bis-e										R4-2304189

Online, April 17 – April 26, 2023

Agenda item:	5.28.1

Source: 	Fujitsu

Title: 	Discussion on the potential solutions for the enhancement for SAR issue

Document for:	Approval

# 1. Introduction

In RAN4#106 meeting, the discussion for the enhancement for SAR issue mitigation was conducted and some potential solutions were captured in the WF [1]. In this contribution, we share our thought on those solutions and how to proceed the RAN4 discussion considering the current situation.

# 2. Discussion

Regarding the issue on whether and how PHR reporting enhancement should be considered for FR1 carriers, the followings are the exert of approved WF [1].

Issue 5: Whether and how PHR reporting enhancement should be considered for FR1 carriers

<Recommended WF>

- RAN4 discussion will focus on the following solutions that have been proposed in this meeting:

    - Power class fallback ΔPPowerClass with aperiodic PHR.

      - Report power-class fallback ΔPPowerClass in the PHR per serving cell, any power-class change, fallback or return to declared power class, should trigger an aperiodic PHR. This also includes FDD PC2.

      - Report power-class fallback ΔPPowerClass,CA in the multi-entry PHR for the BC; any BC power-class change, fallback or return to advertised BC power class, should also trigger an aperiodic PHR.

      - For EN-DC report power-class fallback ΔPPowerClass,EN-DC in the multi-entry PHR for the BC.

    - Power class being used by the UE. Because reporting ΔPPowerClass must be a huge burden for both UE and network.

      - For single band HPUE operation, PC being used by a UE must be able to be reported per serving cell.

      - For UL inter band CA HPUE operation, PC being used by a UE must be able to be reported per serving cell per band within a band combination as well as CA PC being used CA for the band combination itself.

    - The sustainable duty cycle over a certain duration that would prevent triggering a power class fallback at the UE, as well as period of applicability of the ∆PPowerClass report.

    - Introduce a scheme for a UE to report uplink symbol evaluation period and starting timing.

- Enhance the current power headroom reporting framework to enable P-MPR reporting (via MPE field) for FR1 carriers.

From the above description, there are 5 potential solutions as follows.

- Report power-class fallback ΔPPowerClass with aperiodic PHR.

- Report power class being used by the UE.

- Report the sustainable duty cycle over a certain duration.

- Report the uplink symbol evaluation period and starting timing.

- Report P-MPR for FR1 carriers.

Our understanding is that they do not interfere the UE behavior for avoiding the SAR violation, that is, the UE’s transmit power reduction scheme itself is the same even if which solution is implemented. The intention of all solutions is just to add the functions to report the transmission power related information to gNB more accurately and timely. And those information results in the enhancement of the gNB scheduler performance. From this point, there is no issue from the viewpoint of SAR violation no matter which solution we select.

Observation 1: From the viewpoint of SAR violation, there is no issue whichever solution we select.

Considering the above observation, RAN4 should discuss the potential solutions other than the SAR violation point of view. As already described above, the intention of all solutions is to enhance the gNB scheduler performance, so it is straightforward to compare the system performance gain by introducing them. One concern is that RAN4 is not the suitable group for evaluating the system performance of them because the behavior of gNB scheduler should be considered.

Observation 2: RAN4 is not the suitable group for evaluating the system performance gain by introducing the potential solutions because the behavior of gNB scheduler needs to be considered.

Agreement

Further discussions in RAN1 concerning means to facilitate higher power transmissions in CA and DC, if applicable, can target increasing gNB awareness of UE’s Tx power, e.g., PHR reporting enhancement such as current power class, power class change, or application of P-MPR by UE (subject to RAN4’s input).

  - FFS: details.

It is clear that RAN1 is the suitable group for its evaluation. And according to the agreement in February meeting, RAN1 thinks that further input from RAN4 is needed. The followings are the exert of RAN1 report [2].

Considering the above situation in RAN1, our proposal is to send an LS to RAN1 to ask for the performance comparison among potential solutions.

Proposal 1: RAN4 sends an LS to RAN1 to compare the performance of potential solutions.

In addition to it, RAN4 needs to clarify how to select the potential solutions considering the other aspects. If there is nothing else to compare them other than the performance aspect, RAN4 should request RAN1 to handle the discussion on it.

Proposal 2: RAN4 requests RAN1 to handle the discussion on which solution is specified if there is nothing else to compare the potential solutions other than the performance aspect.

Before sending the LS to RAN1, it is important to know that RAN1 has to complete the discussion at RAN1#114 meeting, held at Aug. 2023 according to the allocated TU information. Considering it, RAN4 work to narrow down the potential solutions before sending LS will help RAN1 work.

Observation 3: RAN4 work to narrow down the potential solutions will help RAN1 work because of the limitation of RAN1 TU.

We already have common understanding that currently gNB cannot know the information of power class fallback and P-MPR, then knowing them at gNB can enhance the performance of its scheduler. From the perspective of which information is reported to gNB, our understanding is that the potential solutions can be categorized as follows.

- Power class fallback: solution 1, 2, 3, 4

- P-MPR: solution 3, 5

For solution 3, it reports the duty cycle to continue the operation of high-power UE according to [3]. Our understanding of this solution is that P-MPR related information is implicitly reported because the power reduction due to P-MPR can be avoided if the gNB scheduler assign the radio resource not to exceed the sustainable duty cycle. Considering the above, our proposal is the following.

Proposal 3: At least solution 3 (sustainable duty cycle) should be included in RAN1 LS.

# 3. Conclusions

In this contribution, we share our observations and proposals for the enhancement for SAR issue as follows.

Observation 1: From the viewpoint of SAR violation, there is no issue whichever solution we select.

Observation 2: RAN4 is not the suitable group for evaluating the system performance gain by introducing the potential solutions because the behavior of gNB scheduler needs to be considered.

Proposal 1: RAN4 sends an LS to RAN1 to compare the performance of potential solutions.

Proposal 2: RAN4 requests RAN1 to handle the discussion on which solution is specified if there is nothing else to compare the potential solutions other than the performance aspect.

Observation 3: RAN4 work to narrow down the potential solutions will help RAN1 work because of the limitation of RAN1 TU.

Proposal 3: At least solution 3 (sustainable duty cycle) should be included in RAN1 LS.

# 4. Reference

- R4-2303560	WF on coverage enhancement part 1, Huawei, HiSilicon

- Draft Report of 3GPP TSG RAN WG1 #112

- R4-2300711, On UE signaling to enhance ULCA and DC, Qualcomm Incorporated