3GPP TSG-RAN WG4 Meeting #106bis-e                                                    	R4-2304218

Electronic Meeting, 17– 26 April, 2023

Source:	Huawei, Hisilicon

Title:	TP to TR 38.718-02-01 Addition of CA_n3A-n75A

Agenda item:			4.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.718-02-01 to include CA_n3A-n75A with UL configuration CA_n3A-n75A BCS 4 & 5.

During the previous meeting，some companies expressed concern regarding the introduction of a new maximum CBW for n3 in BCS4 at 50MHz, so cross-band isolation for CA_n3A-n75A should be double-checked whether it should be align with CA_n3-n74. In this paper, we’d like to discuss and address this issue.

2 Discussion

We use main/diversity antenna topology as RF front-end architecture in our analysis, as shown in figure 1.

Figure 1 main/diversity antenna topology

The attenuation of n75 filter can be shown in figure 2 （a）and （b）. The minimum attenuation value for band n3 TX frequency is 34 dB.

Figure 1  （a）Electric Characteristic of n75 filter

Figure 1  （b）Electric Characteristic of n75 filter

The receiver filter of n74 is referred to Band11/ Band21filter as following:

In comparison to the n74 RX filter (refer to Band 11/21 RX filter), the n75 RX filter offers better than 35dB suppression at the n3 transmission frequency, while n74 RX filter (reference to Band 11/21 RX filter) only offers 20dB suppression at the n3 transmission frequency.

Thus, table 1 summarize the MSD analysis results for CA_n3-n75.

Table 1 MSD calculation for victim Rx of band n75

| Item | Unit | parameter | Main | Diversity |
| --- | --- | --- | --- | --- |
| n3 TX power at antenna port (Primary path) | dBm | 23 |  |  |
| n3 PA output noise power at n75 freq | dBm/Hz | （50MHz UL） | -130 | -130 |
| Antenna isolation Primary <-> Diversity |  | 10 |  |  |
| n75 Rx bandwidth | MHz | 50 |  |  |
| n3 PA output noise power |  |  | -53.0103 | -53.0103 |
| n75 RX port on n3 TX freq | dB | 34 |  |  |
| Diplexer loss, dB |  | 1.5 |  |  |
| diplexer out of band isolation | dB | 10 |  |  |
| n3 PA noise power at DPX- COM port |  |  | -97.0103 | -108.5103 |
| front end loss |  | 4 |  |  |
| n3 PA noise power at ant port |  |  | -93.0103 | -104.5103 |
| noise figure dB | dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) |  | -165 |  |  |
| Thermal noise, dBm |  |  | -88.0103 | -88.0103 |
| TX IM2 noise dBm |  |  | -94 | -116 |
| totle noise level at ANT port, dBm |  |  | -86.06 | -87.91 |
| SNR requirement for QPSK |  | -1 |  |  |
| REFSENSE (referred to antenna)(10MHz BW) |  |  | -87.06 | -88.91 |
| combined REFSENS(10MHz BW), dBm |  |  | -91.09 |  |
| Margin | dB | 2.5 |  |  |
| combined REFSENS(10MHz BW)after considering Margin, dBm |  |  | -88.59 |  |
| MSD | dB |  | 1.08 |  |

Based on the analysis above, the MSD is 1.1dB which is caused by cross band isolation for CA_n3-n75.

Text Proposal

<TP for TR 38.718-02-01>

<Start of Text Proposal>

5.x	CA_n3A-n75A

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band n3+n7

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n75 | NA |  |  | 1432 MHz | – | 1517 MHz | SDL |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n3+n7

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n75A | CA_n3A-n75A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

5.x.1.3	UE co-existence studies

The harmonic / harmonic mixing issue have been analyzed in previous release.

5.x.1.4	∆TIB,c and ∆RIB,c values

The TIB,c and RIB have been analyzed in previous BCS 0 and 1.

5.x.1.5	REFSENS requirements

For CA_n3A-n75A, there are no harmonics or harmonic mixing issues. Therefore, MSD due to harmonic or harmonic mixing is not needed. Here we discuss whether cross band isolation issue needs to be considered for this band combination.

Based on the cross band isolation analysis in R4-2304218, the MSD exception for CA_n3-n75 are specified below.

Table 5.X.5-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n75 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 1.1 | >ACLR2 |

5.x.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirements

<End of Text Proposal>