3GPP TSG-RAN WG4 Meeting # 106bis-e                                     R4-2304232

Online, April 17 – April 26, 2023

Agenda Item:	5.10.2.3

Source:	Intel Corporation

Title:	Discussion on RRM requirements for combinations of NCSG and multiple concurrent MGs

Document for:		Discussion

# Introduction

In the last RAN4 meeting, Rel18 NR measurement gap enhance WI was initially discussed. In this paper, the further considerations on requirements for combinations of NCSG and multiple concurrent MGs under objective 1 in [1]was provided.

The following objectives are considered in this WI: - Enhancements of pre-configured MGs, multiple concurrent MGs and NCSG - Define RRM requirements for UEs configured with a combination of pre-configured MGs, and/or concurrent MGs and/or NCSG [RAN4] - Prioritize at least joint requirements for UE configured with - Case 1: Pre-configured MG(s) and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a Pre-configured MG) - Case 2: NCSG and concurrent MG(s) (i.e., the network has provided UE with multiple measurement gap patterns where at least one gap pattern is a NCSG) - Note 1: Gaps that are configured for NTN are precluded in Case 1 and Case 2 - Note 2: The requirement discussions on the scenarios that NCSG is considered in Case 1 and that Pre-configured MG is considered in Case 2 will be started after RAN#99. - Note 3: Prioritization among other possible combinations of pre-configured MG, concurrent MG, NTN gaps and NCSG can be discussed after RAN#99 - Note 4: This WID does not include any inter-working with MUSIM gaps

# Discussion

# Parallel measurement for NCSG+NCSG

As agreed in the last meeting, the scenario of “NCSG + NCSG” should be supported in Rel18 WI. The further clarification on this scenario is captured in WF [2] also.

And in the last RAN4 meeting, the following issues was discussed.

### Issue 4-1-2: [Case 2] Whether the same RF chain is assumed for the two NCSG patterns < Wayforward >: - FFS the options: - Option 1: - Yes, the same RF chain is assumed for the two NCSG patterns. - Option 2: - No need to discuss whether the same RF chain is assumed for the two NCSG patterns (not necessary). - Option 2a: - No need to discuss whether the same RF chain is assumed but to consider the assumption on parallel measurement. - Option 3: - At least two spare RF chains are assumed. - Option 4: - UE signals its capability on number of receiver chains per band to the network.

In our view, UE to support NCSG capability can share the same RF chain which is different with the one for UE serving data reception.

Proposal 1: The same RF chain is assumed for the two NCSG patterns.

### Issue 4-2-3: [Case 2] Whether to consider parallel measurements upon gap collision < Wayforward>: - FFS the options: - Option 1: - Yes - Option 2: - No - Option 2a: - No, when the RF chains for the two NCSG patterns are different. - Option 3: - Up to UE capability, - For UE supporting this capability, both NCSGs can work when colliding. - For the UE not supporting this capability, R17 priority rules when colliding can be reused. - Option 4: - RAN4 to study a general solution to allow both NW and UE to know the parallel measurements combination when UE supports NCSG parallel measurement capability. - Option 5: - RAN4 to agree on investigating relevant scenarios with gap collision for the Case 2 scenario.

As illustrated in the figure below, there are several cases below at least when both NCSG and other legacy measurement gaps are configured within the concurrent measurement gaps.

  - Case a). the start/end point of two gaps are interlaced.

  - Case b). the legacy MGs (Type2-MG) can be fully contained by NCSG

  - Case c). NCSG can be fully contained by the legacy MGs (Type2-MG)

Figure 1. Overlapping among NCSG and legacy MGs

For case a), UE can perform the measurement on the carrier (f1) from t1. And as UE can support NCSG for “f0+f1” band combination, UE can also receive the data on PCC (f0) from t1 to t3. Thus when UE needs to perform the measurement on the other carriers (f2) from t2, UE needs to drop one of measurement based on Rel17 rules below. However, if UE can also support the simultaneous multiple RF chains operation (like NCSG capability) in f1 and f2, UE can retune its available RF chain (which was used for data reception during [t1,t2]) to f2. Then measurements on both f1 and f2 can be performed. For case b) and c) we have similar observations.

Observation 1: In case of concurrent MGs in which one of NCSG was configured, the parallel measurements on the multiple collided concurrent gaps are feasible.

For the possible scenarios with the parallel measurement for two NCSG, RAN4 agreed that:

### Issue 4-2-5: [Case 2] Whether to support parallel measurements in the following scenarios for two NCSG < Wayforward >: - FFS the following options Scenario 1: NW only configures deactivated SCells’ measurement - Option 1: - No. - Option 2: - Yes. Scenario 2: NW only configures the MOs in intra-bands in which UE reports to support ‘NCSG’ - Option 1: - No. - Option 2: - Yes. Scenario 3: NW configures MOs in intra-band associated with NCSG1 and MOs in inter-band associated with NCSG2 if UE reports ‘NCSG’ for these bands - Option 1: - No. - Option 2: - Yes.

Observation 2: For these scenarios if the measurement objects can be contained within the NCSG supported bands, the parallel measurements which are occurred within the different gap duration are feasible.

Also, when SCell was deactivated, the corresponding MO needs to be measured within a measurement gap. In this case if UE can support the multiple NCSGs, it is reasonable to enable UE to measure this SCell within other NCSG.

Observation 3: The parallel measurement on the deactivated SCell is feasible without any additional UE capability beside that requested by “NCSG + Multiple concurrent MG”.

Proposal 2: Parallel measurements in case 2 (NCSG + concurrent gap) shall be supported.

# Gap collision handling

## Issue 4-2-4: [Case 2] Potential changes to UE behaviour upon gap collision < Wayforward >: - RAN4 to postpone the gap collision rule changes’ discussion until RAN4 has a consensus on parallel measurement scenarios.

As illustrated in the section above, when both NCSG and other legacy measurement gaps are configured within the concurrent measurement gaps, the parallel measurement are feasible and helpful to reduce the measurement latency effectively at least for the following scenarios .

  - the legacy MGs (Type2-MG) are overlapped with NCSG

  - deactivated SCell’s measurement after being activated

Then measurements on both f1 and f2 can be performed. That is no need to drop either one of concurrent gaps (ncsg + type1-MG) under such circumstance.

Observation 4: In case of concurrent MGs in which one of NCSG was configured, UE needs NOT to drop any of gap instances when they are collided if UE can support the NCSG capability for both(f0+f1) band combination and (f1+f2) band combination.

Hence the current rules for gap collision in Rel17 below can be optimized given limited impacts on the standardization works.

Rel17 gap collision rules [TS38.133 v17.6.0]: “In case of collision between two measurement gap occasions, the UE shall perform measurements in the occasion of the measurement gap with higher priority, and the occasion of the measurement gap with lower priority is considered to be dropped. The UE shall be able to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots that are not interrupted according to requirements in clause 9.1.2B.4. “

Therefore, we suggest that:
Proposal 3: RAN4 shall consider potential enhanced requirements on UE behavior for collision handling in Case 2 [1].

# Gap interruption

## Issue 4-4-2: [Case 2] Gap interruption < Wayforward >: Discuss the options (FFS) - Option 1: The interruption requirements for the multiple measurement gaps when NCSG being included in the concurrent measurement gaps can be defined as: Tint_ncsg+Tint_other-Toverlap, wherein, Tint_ncsg,Tint_other represented the allowed interruption due to NCSG and legacy measurements defined in clause 9.1.2 and 9.1.9.1 of TS38.133[4] respectively. And Toverlap is the overlapped time duration in slot among NCSG RTT time and legacy measurement gap length. - Option 2: Update the existing gap interruption requirements for Case 2 as follows. - A slot is considered as interrupted if it is interrupted by an occasion of any of the configured concurrent measurement gaps following the measurement gap interruption requirements in clause 9.1.2, or by VIL occasion of any of the configured NCSG following the NCSG interruption requirements in clause 9.1.9.1, except for a dropped measurement gap or NCSG occasion.

Accordingly given the Rel17 gap collision handling is optimized (e.g. we need not drop any gap instance when they are collided), we can also investigate on the gap interruption requirements enhancement.

Proposal 4: The interruption requirements for NCSG within concurrent MGs can be FFS especially given the further potential dropping rules optimization when gaps colliding in Case 2.

For instances, as illustrated in the figure below, there are two typical cases when we consider the interruption when both NCSG and other legacy measurement gaps are configured within the concurrent measurement gaps. One is there is no overlapping among NCSG and other legacy gaps used for the measurements on the different carriers as that of measurements by NCSG.

Figure 2. Two cases to define the interruption in Case 2

Observation 5-1: If UE support the NSCG within the concurrent measurement gaps, the interruption due to the individual gap instances which are not overlapped can be completely independent.

That is the interruptions due to individual legacy gap and NCSG which are not overlapped shall follow the requirements in clause 9.1.2 and 9.1.9.1 respectively.

On the other hand, for the cases when the multiple measurement gaps(NCSG or other measurement gaps) was overlapped, the interruption on UE due to both of the different measurement gaps can be overlapped also.

Observation 5-2: The total interruption to UE due to the multiple measurement gaps which are overlapped within the concurrent gaps can be coupled together.

Observation 5-3: For the window between NCSG VIL1 and VIL2(e.g. [t1,t2] in the figure above), during the interval in which there is no any overlapped interruptions UE can received the data as normal.

Therefore, the total interruption length due to both NCSG and other measurement gaps can be equal to

Tint_ncsg+Tint_other-Toverlap               (1)

Wherein, Tint_ncsg,Tint_other represented the allowed interruption due to NCSG and legacy measurements defined in clause 9.1.2 and 9.1.9.1 of TS38.133[4] respectively. And Tint_other is the overlapped time duration in slot among NCSG RTT time and legacy measurement gap length.

Proposal 5:  The interruption requirements for the multiple measurement gaps when NCSG being included in the concurrent measurement gaps can be defined as:

Tint_ncsg+Tint_other-Toverlap

Wherein, Tint_ncsg,Tint_other represented the allowed interruption due to NCSG and legacy measurements defined in clause 9.1.2 and 9.1.9.1 of TS38.133[4] respectively. And Toverlap is the overlapped time duration in slot among NCSG RTT time and legacy measurement gap length.

# Gap association

### Issue 4-3-1: [Case 2] Potential changes for NCSG upon SCell activation < Wayforward >: - FFS the options - Background: When NW configures a NCSG and a Type-2 MG, the scenario for this deactivated SCell issue is as follow. - The deactivated SCell is measured within NCSG. - After MAC-based SCell activation, the deactivated SCell’s MO needs to be measured within MG if the related SSB is outside the active BWP - Proposal: - Option 1: - Do not pursue optimizations for deactivated SCell measurements with NCSG in Case 2 - Option 2: - A new indication shall be introduced enable support of NCSG for deactivated SCell only. - Option 2a: - Indicated explicitly by “ncsgInd-r17”. - Option 3: - Request UE to be responsible for the capability report considering all possible MAC CE triggered NW configuration update. - Option 4: - When the SCell is activated, the MG association is based on NW configuration - When the MO is associated to a type-2 MG and the SCell is deactivated, the MO is implicitly associated to NCSG with which the SMTC is partially or fully overlapped. - Option 5: - If the NCSG is converted into another Type-2 MG upon SCell activation, collision handling between this Type-2 MG for deactivated SCells and the other Type-2 MG needs to be defined. Hence the priority level assigned to NCSG may be maintained or subject to change

As Option 2 pointed out, the motivation to change gap association is mainly for NCSG which was configured for the measurement on the deactivated SCell. When the deactivated SCell to be measured was activated, UE can perform the measurements outside of the gap indeed and vice versa. However, for such case we believe there are at least two feasible ways according to current Rel17 association or configuration.

- Alt1. NCSG can associated with the deactivated SCell explicitly by ‘ncsgInd-r17’. And after the SCell activation, if necessary this indication can be updated explicitly.

- Alt2. The NCSG can be configured as Pre-MG indeed. When the SCell activation conditions was trigged, this gap can be deactivated also. And such aspects can also be beneficial with the SCells which was deactivated.

In our view, Alt2 is more efficient. However, as we agreed in RANP, this mechanism will be deferred to the further release. Thus we suggest:

Proposal 6: RAN4 can further investigate the following options to resolve the issue when NCSG was configured for the measurements on SCells which can be changed between the activation/deactivation.

- Whether NCSG can be associated with SCells to be activated/deactivated is indicated explicitly by “ncsgInd-r17”.

# Conclusion

In this contribution, serval issues related to the measurement gap enhancement WI in Rel18 are discussed. The proposals can be summarized as:

Proposal 1: The same RF chain is assumed for the two NCSG patterns.

Observation 1: In case of concurrent MGs in which one of NCSG was configured, the parallel measurements on the multiple collided concurrent gaps are feasible.

Observation 2: For these scenarios if the measurement objects can be contained within the NCSG supported bands, the parallel measurements which are occurred within the different gap duration are feasible.

Observation 3: The parallel measurement on the deactivated SCell is feasible without any additional UE capability beside that requested by “NCSG + Multiple concurrent MG”.

Proposal 2: Parallel measurements in case 2 (NCSG + concurrent gap) shall be supported.

Observation 4: In case of concurrent MGs in which one of NCSG was configured, UE needs NOT to drop any of gap instances when they are collided if UE can support the NCSG capability for both(f0+f1) band combination and (f1+f2) band combination.

Proposal 3: RAN4 shall consider potential enhanced requirements on UE behavior for collision handling in Case 2 [1].

Proposal 4: The interruption requirements for NCSG within concurrent MGs can be FFS especially given the further potential dropping rules optimization when gaps colliding in Case 2.

Observation 5-1: If UE support the NSCG within the concurrent measurement gaps, the interruption due to the individual gap instances which are not overlapped can be completely independent.

Observation 5-2: The total interruption to UE due to the multiple measurement gaps which are overlapped within the concurrent gaps can be coupled together.

Observation 5-3: For the window between NCSG VIL1 and VIL2(e.g. [t1,t2] in the figure above), during the interval in which there is no any overlapped interruptions UE can received the data as normal.

Proposal 5:  The interruption requirements for the multiple measurement gaps when NCSG being included in the concurrent measurement gaps can be defined as:

Tint_ncsg+Tint_other-Toverlap

Wherein, Tint_ncsg,Tint_other represented the allowed interruption due to NCSG and legacy measurements defined in clause 9.1.2 and 9.1.9.1 of TS38.133[4] respectively. And Toverlap is the overlapped time duration in slot among NCSG RTT time and legacy measurement gap length.

# References

- RP-222337 New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps

- R4-2303197, WF on NR_MG_enh2 Part 1

- TS38.133 v17.8.0

- TS38.331 v17.2.0