3GPP TSG-RAN WG4 Meeting #106 bis-e                   	                                  R4-2304254

# Electronic meeting, April 17 – April 26, 2023

Agenda item:	5.19.1.2

Source:	Intel Corporation

Title:	Initial simulation results of advanced receivers for MU-MIMO

Document for:	Discussion

- Introduction

In RAN4 #106 requirements for advanced receiver for MU-MIMO were discussed and way forward [1] was approved. It was encouraged to bring simulation results for alignment. In this paper simulation results for alignment for the cases with high priority.

- Simulation Results

  - Conditions and Test Cases

In RAN4#106, it was agreed that companies bring simulation results for alignment in RAN4 #106bis-e to decide the gain of E-MMSE-IRC / R-ML receiver comparing with MMSE-IRC receiver. It was agreed that the required parameters on interfering signals are known (genie receiver) and test cases with higher priority was derived. For the simulation parameters in the table 1 we provide we simulation results in the following sub-sections.

In the WF [1], RAN4 agreed as follows.

- MCS for target UE o Cover MCS 13 for rank 1 and rank 2 for initial simulation o Further discuss whether to cover MCS 4 for rank 1 and MCS 19 for rank 2 in the next meeting

To investigate the overall performance in various SNR region, it is desirable to consider additional cases.

Proposal 1. RAN4 to consider additional simulation case of MCS4 (QPSK, rank 1) and MCS19 (64QAM, rank 2) for target UE under Max. 64QAM MCS table.

Table 1: Simulation Parameters for Advance Receiver

- Precoding model for target UE - Random precoding with Single panel Type 1 codebook in PRB bundling size (i.e. 2 RB) per slot - MU-Interference Model The PDSCH and DMRS of the paired UEs are precoded prior to transmission. The precoder for each user is denoted by W1' and W2' respectively. Wi is the precoder matrix of ith UE, of size NTX x NLi from Type I single panel codebook. The combined precoder W = [W1 W2], of size NTX x NL where, NL is the total number layers across all users. The precoder of the target UE W1 is randomly selected. For the co-scheduled UE, the precoder W2 is selected in one of the two ways below. - Orthogonal precoder: W2 is randomly selected from the codebook with a constraint that the combined precoder W has orthogonal columns, i.e., the off-diagonal entries of WHW are zero - Random precoder: W2 is randomly selected from the codebook ensuring any column of W2 is not identical to any column of W1 To maintain the average per UE signal power, an additional scaling is applied to each precoding considering total number of layers across all users NL and the number of layers for each user, NLi. Wi'= NLiNL*Wi - Test metric: SNR gain @ 70% of maximum Throughput for FRC (Test MCS) - CSI-RS configuration: 2 Tx/ 4 Tx NZP CSI-RS with ZP CSI-RS for CSI acquisition, CSI-RS for tracking - Other detailed simulation assumptions MCS under Max. 64 QAM . Target UE: MCS 13 for Rank 1 and Rank 2 . Interfering UE: MCS 4 (QSPK), MCS 19 (64QAM) PDSCH configuration: Mapping Type A with S = 2 and L = 12, Static PRB bundling with size 2 PDSCH DMRS config.: DMRS Type 1, 1 additional DMRS, Max. # of symbols DL front loaded DMRS = 1

| Case Number (Note 1) |  | Co-scheduled UE number | Rank for the target UE | Rank for the C-scheduled UE | Modulation order for the co-scheduled UE | MIMO configuration | Channel model |  | Precoder selection for the Co-scheduled UE (Note 3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 1 | 1 | 1 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 |  | orthogonal |  |
| 2 |  |  |  |  |  |  |  |  | random |  |
| 3 |  |  |  | 3 | 16QAM | 4Tx 4Rx ULA Low |  |  | orthogonal |  |
| 4 |  |  |  |  |  |  |  |  | random |  |
| 5 |  |  | 2 | 2 | 64QAM |  |  |  | orthogonal |  |
| 6 |  |  |  |  |  |  |  |  | random |  |
| 7 |  |  |  |  | QPSK |  |  |  | orthogonal |  |
| 8 |  |  |  |  |  |  |  |  | random |  |
| 9 |  |  |  |  | 64QAM |  | TDLA30-10 |  | orthogonal |  |
| 10 |  |  |  |  |  |  |  |  | random |  |
| 11 |  |  |  |  | QPSK |  |  |  | orthogonal |  |
| 12 |  |  |  |  |  |  |  |  | random |  |
| 13 |  | 2 | 1 | 1 for each Co-scheduled UE | 64QAM and QPSK | 4Tx 4Rx ULA medium A | TDLC300-100 |  | orthogonal |  |
| 14 |  |  |  |  |  |  |  |  | random |  |
| 15 |  |  |  |  | 64QAM and 16QAM |  |  |  | orthogonal |  |
| 16 |  |  |  |  |  |  |  |  | random |  |
| 17 |  |  |  |  | 64QAM and QPSK | 4Tx 4Rx XPL medium |  |  | orthogonal |  |
| 18 |  |  |  |  |  |  |  |  | random |  |
| 19 |  |  |  |  | 64QAM and 16QAM |  |  |  | orthogonal |  |
| 20 |  |  |  |  |  |  |  |  | random |  |
| Note 1: The cases in Red are of high priority for the April meeting, the cases in Green are of high priority for the May meeting, the cases in Orange are of low priority and the cases in Blue are of the lowest priority. Note 2: Target UE - MCS 13 for Rank 1 and Rank 2 under Max. 64 QAM Note 3: Simulation was done in FDD 10 MHz, 15 KHz SCS and Opt. 1 / Opt. 2 below denotes orthogonal PMI / random PMI, respectively |  |  |  |  |  |  |  |  |  |  |

  - Simulation Results

# To find the focused SNR region for 70% throughput, simulations with medium size samples were carried out to get rough figures below. The SNR point for 70% throughput in summary was done with larger samples in the focused SNR region.

    - Case 1-2) Rank 1+1 under 2x2 medium, TDLC300-100 channel

    - Case 5 ~ 8) Rank 2+2 under 4x4 low, TDLC300-100 channel

    - Case 9 ~ 12) Rank 2+2 under 4x4 low, TDLA30-10 channel

  - Summary

| Case | Number of co-scheduled UEs | Rank (Target UE) | Co-scheduled UE |  | MIMO Config. | Channel model | Precoder selection (Co-scheduled UE) | SNR dB @ 70 % Throughput |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Rank | Modulation |  |  |  | R-ML | E-IRC | IRC (Baseline) | Gain (R-ML) | Gain (E-IRC) |
| 1 | 1 | 1 | 1 | QPSK | 2Tx 2Rx ULA medium | TDLC300-100 | orthogonal | 13.4 | 19.1 | 19.3 | 5.9 | 0.2 |
| 2 |  |  |  |  |  |  | random | 14.4 | 21.3 | 22.3 | 7.9 | 1.0 |
| 3 |  |  | 3 | 16QAM | 4Tx 4Rx ULA Low |  | orthogonal | - |  |  |  |  |
| 4 |  |  |  |  |  |  | random | - |  |  |  |  |
| 5 |  | 2 | 2 | 64QAM |  |  | orthogonal | 13.5 | 14.2 | 14.8 | 1.3 | 0.6 |
| 6 |  |  |  |  |  |  | random | 18.5 | 20.4 | 30.0 | 11.5 | 9.6 |
| 7 |  |  |  | QPSK |  |  | orthogonal | 11.0 | 14.2 | 14.8 | 3.2 | 0.6 |
| 8 |  |  |  |  |  |  | random | 13.3 | 20.4 | 29.9 | 16.6 | 9.5 |
| 9 |  |  |  | 64QAM |  | TDLA30-10 | orthogonal | 12.2 | 12.5 | 12.5 | 0.3 | 0.0 |
| 10 |  |  |  |  |  |  | random | 16.8 | 17.6 | 17.5 | 0.7 | -0.1 |
| 11 |  |  |  | QPSK |  |  | orthogonal | 9.9 | 12.5 | 12.4 | 2.5 | -0.1 |
| 12 |  |  |  |  |  |  | random | 11.8 | 17.7 | 17.5 | 5.7 | -0.2 |

- Conclusion

In this paper, we provide initial simulation results for alignment for defining advanced receiver for MU-MIMO.

Proposal 1. RAN4 to consider additional simulation case of MCS4 (QPSK, rank 1) and MCS19 (64QAM, rank 2) for target UE under Max. 64QAM MCS table.

- References

[1] R4-2302929, WF on advanced receiver for MU-MIMO scenario, China Telecom