3GPP TSG-RAN WG4 Meeting #106-bis-e		R4- 2304262

Electronic Meeting, April 17 – April 26, 2023

Agenda item:	5.6.2

Source:	Intel Corporation

Title:	Views on the UE channel raster for FR1

Document for:	Discussion

# 1	Introduction

In the previous RAN 99 meeting, a new work item was created to address the issues related to 100kHz channel raster [1].  This WI addresses issues that arose during the study of efficient spectral utilization or Irregular channel BW.

During that study, different views emerged as to when the cell’s carrier RB grid must be on raster, or if the assigned UE channel bandwidth must be on raster or both must be on raster.  For channel bandwidths below 3GHz, most bands are required to use a 100kHz raster for RF channel placement which does not allow convenient alignment between RBs of width 180kHz, 360kHz or higher.  If a cell, intended for use by multiple UEs, is defined with a certain carrier resource block grid, it is only possible to be aligned to both the carrier RB grid and the 100kHz channel raster for least common multiples of the RB width and the channel raster (e.g., lcm(180,100) = 900kHz).  This reduces the feasible placement of UE channels, resulting in wasted spectrum from un-used RBs.  The 100kHz raster requirement also causes wasted RBs to form a gap in CA applications.  A further problem exists for 15kHz SCS, where cells defined with carrier bandwidth (CBW) using an odd number of RBs have no raster aligned locations for UE CHBW using an even number of RBs.  The complementary situation also is a problem for a cell defined with an even number of CBW RBs as it does not allow any raster aligned locations for UE CHBW using odd number of RBs.  Eliminating the need for UEs to be required to be on 100kHz raster would resolve these issues.

The objectives of the work item are the following:

- Specify necessary changes to the UE channel raster such that configuring a narrower UE channel BW inside a wider gNB channel BW is always possible [RAN4].

- Changes to BS channel raster can be considered if required [RAN4].

- Specify the corresponding UE capability, if needed, to enable changes to the channel raster [RAN2, RAN4]:

  - RAN4 is to identify the release of the specifications 38.101-1 and 38.104 and the possibility of early implementation. If corresponding capability signalling is provided for early implementation and such early implementation is possible, the change is to be release independent from the identified release.

NOTE: Changes to channel raster need to be compliant with the definition of global channel raster in RAN4 specification.

The following WI objectives were defined in RAN #99 [1]:

# Discussion

2.1 Discussion of changes to fix the 100kHz raster issue

In this section we discuss potential changes to address the problems of requiring UEs to be on 100kHz raster, as listed in section 1.  The primary change is to allow network to make the decision whether the UE’s attaching to a cell need to be on 100kHz raster or not.  In Rel-15, the original motivation for requiring NR channel BWs to be on 100kHz raster was concern that the RB placement needed to be SC aligned to the LTE RB placement.

Observation 1: In Rel-15, the original motivation for requiring NR channel BWs to be on 100kHz raster was concern that the RB placement needed to be SC aligned to the LTE RB placement.

This allowed the sub-carriers to be aligned so that they could all fit within gNB FFT.  Since the network is aware of which cells are being deployed in dual use LTE / NR, the network is capable of making the decision whether it needs to utilize a deployment with UEs on 100kHz raster or 15kHz raster.

Observation 2: Since the network is aware of which cells are being deployed in dual use LTE / NR, the network is capable of making the decision whether it needs to utilize a deployment with UEs on 100kHz raster or 15kHz raster.

The other argument for UEs being on 100kHz raster was for testing requirements to be standardized so that UEs in the field work the in same situations they have been tested in.  However, current testing requirements for UEs have been shown to only test UEs at three locations: left, right and centre of the operating band.  The per RB UE performance is guaranteed by design.  Thus, it is possible for UEs that are capable of working on a 15kHz raster basis, to indicate this to the network via a UE capability.  In this way, after connection establishment, the network can determine if a UE is capable of being placed on 15kHz raster or limited to 100kHz raster placement.

Observation 3: With the introduction of a new UE capability, after connection establishment, the network can determine if a UE is capable of being placed on 15kHz raster or limited to 100kHz raster placement.

Proposal 1: A new UE capability should be defined for UEs that are capable of operating on an 15kHz based raster for any FR-1 band.

In [2,3,4,5] we introduce spec changes for Rel-18. In addition, RAN2 would have to be requested to prepare CRs to introduce the new UE capability. These CRs will enable new UEs that are compliant to Rel-18 and later to support the 15kHz based raster and indicate that support to the network. However, there may be future new UEs that are compliant to Rel-15, 16 or 17 that are able to support the 15kHz based raster and it would be beneficial if these UEs could also use the new capability signalling to indicate the support to the network. Finally, there may be many UEs in existence today that are already capable of supporting an 15kHz based raster.  For these Rel-15,16 and 17 UEs, it should be possible for vendors to make a software update that would enable them with the capability signalling as long as they are compliant. Consequently, we think that the changes to support 15kHz based raster should be introduced from Rel-18 and be introduced in a release independent manner that enables implementation by UEs starting from Rel-15.

Proposal 2: CRs for 15kHz based raster are introduced from Rel-18 and are introduced in a release independent manner that enables implementation by UEs starting from Rel-15.

Another question is whether the 15kHz based raster should be mandatory (with capability signalling) for UEs to support from a certain release. We consider that it would be too late to mandate this for UEs of Rel-17 but that it would be useful for this to be mandatory with capability signalling for UEs from Rel-18, to help ensure that operators will eventually have a high population of UEs that support this functionality that helps them better utilise their spectrum.

Proposal 3: 15kHz based channel raster is mandatory with capability signalling from Rel-18

Another issue raised in the WID, is whether the BS channel raster should be enhanced to not require a 100kHz channel raster.  In our view, this is not necessary.  Of the problems described in section 1, most of these would not be completely fixed by allowing the BS CBW to be on a non-100kHz raster.

Proposal 4: Although we don’t see a need for any functional change to the BS channel raster, we do think it would be beneficial to make some minor changes to the TS 38.104 specification to ensure text consistency between TS 38.101-1 and TS 38.104.

When RAN4 has concluded these discussions then communication should be made to RAN2 requesting implementation of this UE capability.

2.2 Discussion of Initial Access Procedure with the proposed capability signaling

To better illustrate the how a UE with capability signalling would work, we consider a simple example and describe the relevant steps that the network and UE take during initial access:

- BS advertises an SSB

  - Within the MIB, the pdcch-ConfigSIB1 indicates the example CORESET0 of 5MHz with 15kHz SCS and a number of RB offset.

  - Also, within the MIB, the ssb-SubcarrierOffset indicates the offset Kssb between the SSB grid and the carrier RB grid.

  - The BS has no foreknowledge of the UEs CHBW or raster capabilities and the MIB is sent with common signaling.

- The UE receives the SSB and

  - The SSB is on sync raster, and it doesn’t matter if the UE is capable of RB raster or 100kHz raster at this point.

  - The UE uses this information to set itself up to receive CORESET0.  The RBs of CORESET0 will be aligned to the carrier RB grid.  The left most RB location is known from the RB Offset given in the CORESET table using the index from pdcch-ConfigSIB1 and the SCS offset from Kssb.

  - Point A, and the size of the Carrier Resource Grid are unknown at this step, since SIB1 hasn’t been received, but this information is unnecessary to receive CORESET0.

- The UE receives SIB1

  - Within SIB1 within servingCellConfigCommon, is the offsetToPointA, carrierBandwidth and offsetToCarrier, which define PointA and the starting location and bandwidth of the carrier resource grid.

  - Also, within SIB1 can be indication of the initial BWP that the UE is to use if different from that of the CORESET0.  However, the default is to just re-use the CORESET0 RB grid.

  - At this point, the UE can keep its CHBW filter sized for the initial BWP, or it could select the CHBW of the entire carrier resource grid.  Since the carrier resource grid is on 100kHz raster, it doesn’t matter if the UE is RB-raster capable or 100kHz raster capable.

- The UE enters RRC connected mode

  - During the connection establishment procedure, the gNB must assume that the UE is only capable of 100kHz channel raster and configure the UE accordingly

  - After the UE enters connected mode, the gNB receives the UE capabilities (either from the core network or directly from the UE) and can then configure the UE according to those capabilities.

  - UEs without capability signaling, can be placed through dedicated signaling on any supported channel BW that is centered on an RB on 100kHz.

  - UEs with 15kHz based raster capability signaling, can be placed through dedicated signaling on any supported channel BW that is centered on whichever RB the network chooses.  This allows optimal placement flexibility.

  - Alternatively, the UE could simply remain on the initial BWP if the network chooses not to setup a specific channel BW through dedicated signaling.

This procedure shows how both legacy UEs, and updated UEs with capability signaling would connect during the initial access procedure.  Both types of UEs should be able to connect without issue.  The RB-raster based UEs allow the network to more flexibly place UE channel BWs to fully utilize operator’s available spectrum.

# 3	Conclusions

In summary, this paper discussed the initial access procedure, indicating when the UE is signalled to have BWP, and CHBW locations, and when these can be known to be on raster.

Observation 1: In Rel-15, the original motivation for requiring NR channel BWs to be on 100kHz raster was concern that the RB placement needed to be SC aligned to the LTE RB placement.

Observation 2: Since the network is aware of which cells are being deployed in dual use LTE / NR, the network is capable of making the decision whether it needs to utilize a deployment with UEs on 100kHz raster or 15kHz raster.

Observation 3: With the introduction of a new UE capability, after connection establishment, the network can determine if a UE is capable of being placed on 15kHz raster or limited to 100kHz raster placement.

Proposal 1: A new UE capability should be defined for UEs that are capable of operating on an 15kHz based raster for any FR-1 band.

Proposal 2: CRs for 15kHz based raster are introduced from Rel-18 and are introduced in a release independent manner that enables implementation by UEs starting from Rel-15.

Proposal 3: 15kHz based channel raster is mandatory with capability signalling from Rel-18

Proposal 4: Although we don’t see a need for any functional change to the BS channel raster, we do think it would be beneficial to make some minor changes to the TS 38.104 specification to ensure text consistency between TS 38.101-1 and TS 38.104

# 4	References

[1] RP-230813, “New WID on Channel raster enhancement”, Ericsson, RAN #99, March 2023

[2] R4-2304263, “CR 38.104 - Clarification of UE capability enhancement of channel raster in bands below 3GHz”, Intel, RAN4 #106-bis-e, April 2023

[3] R4-2304264, “CR 38.101-1 - Clarification of UE capability enhancement of channel raster in bands below 3GHz”, Intel, RAN4 #106-bis-e, April 2023

[4] R4-2304265, “CR 38.101-2 - Clarification of UE capability enhancement of channel raster in bands below 3GHz”, Intel, RAN4 #106-bis-e, April 2023

[5] R4-2304266, “CR 38.3-7 - Clarification of UE capability enhancement of channel raster in bands below 3GHz”, Intel, RAN4 #106-bis-e, April 2023