# 3GPP TSG-RAN WG4 Meeting #106-bis-e			R4- 2304266

# Electronic Meeting, April 17 – April 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR | draft | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement of channel raster in bands below 3GHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_channel_raster_enh |  |  |  |  |  | Date: |  |  | 2023-4-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | UEs operating under 3GHz will no longer be required in all cases to be on 100kHz channel raster, instead the option to put them on channel raster will be up to the network based on UE raster capability signalling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text changes are added, to give the network the freedom to determine if the UE also needs to be placed on a channel bandwidth which is on a 100kHz raster or an RB based raster. Clause 5.4: The first change adds a row in Table 5.4-1 Addiional requirements of releaes independent features for UE capability for 15kHz channel raster Clause B.4. Adds a row in Table B.4.1-1: Common UE RF requirements for a release independent band for UE capability for 15kHz channel raster |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The problems related to 100kHz raster will continue. This results in wasted spectrum from un-used RBs between spectrum blocks assigned to UEs by the network. It also leads to a forced spectrum gap in CA applications. Further problems exist for 15kHz SCS, where cells defined with carrier bandwidth using an odd (or even) number of RBs have no raster aligned locations for UE CHBW using an even (or odd) number of RBs. Eliminating the need for UEs to be required on 100kHz raster would resolve these issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4, B.4.X (new subclause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.101-1 CR R4-2304264, TS 38.104 CR R4-2304263, TS 38.101-2 CR R4-2304265 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

## 5.4	Other release independent features for NR frequency range 1

This clause covers requirements for a Rel-15 UE coming from all other release independent features that are not covered under clause 5.1, 5.2 and 5.3, e.g. generic baseband requirements or requirements that are not band/CA/SUL configuration specific.

Table 5.4-1: Additional requirements of other release independent features

| Feature | Release independent from | Requirements to be fulfilled (see 38.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| RRM requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.1-1 | Rel-16 WI NR_HST introduced band independent RRM requirements: see Table C.1-1 |
| UE demodulation requirements for high speed train scenario | Rel-15 (NOTE 1) | Table C.2-1 | Rel-16 WI NR_HST introduced band independent UE demodulation requirements: see Table C.2-1 |
| RF requirements for 4Rx UEs | Rel-15 | Table B.4.10-1, Table B.4.10-2 |  |
| [Transparent Tx diversity] | [Rel-15] | [Table B.4.11-1] | [Rel-17 WI NR_RF_TxD introduced transparent Tx diversity requirements: see Table B.4.11-1] |
| UE demodulation and CSI requirements for MMSE-IRC receiver for scenarios with inter cell and intra cell inter user interference | Rel-15 | Table B.3.3-1 | Rel-17 WI NR_demod_enh2-Perf: see Table B.3.3-1. These requirements are optional for Rel-15 and Rel-16 UEs and can be executed based on UE declaration. |
| RRM enhancement for FR1 high speed train scenario enhancement | Rel-16 (NOTE 2) | Table C.3-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent RRM enhancement: see Table C.3-1 |
| HST-SFN CA demodulation enhancement for FR1 high speed train scenario enhancement | Rel-16 (NOTE 3) | Table C.4-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent HST-SFN CA demodulation enhancement: see Table C.4-1 |
| HST-DPS CA demodulation enhancement for FR1 high speed train scenario enhancement | Rel-15 (NOTE 4) | Table C.4-1 | Rel-17 WI NR_HST_FR1_enh introduced band independent HST-DPS CA demodulation enhancement: see Table C.4-1 |
| [15kHz UE channel raster for FR1 low bands] | Rel-15 | Table B.4-1 | Rel-18 WI NR_channel_raster_enh band independent UE capability for 15kHz channel raster |
| NOTE 1: Rel-15 UEs supporting the high speed train are assumed to read the Rel-16 high speed train scenario information, which is broadcast to all UEs. NOTE 2: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 3: Rel-16 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. NOTE 4: Rel-15 UEs supporting the high speed train are assumed to read the Rel-17 high speed train scenario information, which is broadcast to all UEs. |  |  |  |

< text omitted >

## B.4.X	Common UE RF requirements for 15kHz UE channel raster for FR-1 low bands

The requirements and test cases listed in Table B.4.X-1 are specified in REL-18 version of TS 38.101-1 [2].

Table B.4.X-1: Common UE RF requirements for a release independent band

| Clause / Clause | Description |
| --- | --- |
| 5.3 | UE Channel bandwidth |
| 5.4 | Channel arrangement |

< end of changes >