3GPP TSG-RAN WG4 Meeting #106-bis-e									         R4-2304287

Electronic, April 17 – 26, 2023

Agenda item:	5.10.3.1

Source:	Apple

Title:	On measurement without gaps for UEs reporting NeedForGapsInfoNR

Document for:	Discussion

# Introduction

NeedForGaps RRM requirement was widely discussed during the previous RAN4 meetings. The last agreements can be found in [1]. In this contribution, we continue discussing the open issue.

# Discussion

The first open issue is about framework of interruption requirements:

Issue 1-1-2: Framework of the interruption requirements

< Agreement >:

  - The following aspects will be defined in the requirements of interruption:

    - Interruption length

< Way forward >:

    - Interruption ratio can be defined depending on the measurement cycle length and interruption length as:

      - Option 1:

        - with up to [1.25%] probability of interruption per a UE measurement sample cycle when it is NOT less than [160ms] ms

        - FFS on whether and how to define the interruption ratio requirements when the UE measurement sample cycle is less than [160ms]

      - Other options not precluded

    - FFS on possible measurement delay requirements extension

    - FFS whether there is a need to define the interruption location

We still believe it is better to define clear interruption location so that the negative impact on system throughput can be minimized. From network scheduling point of view, NW can schedule other UE during the interruption location so that there is system throughput degradation. Nevertheless, for the sake of progress, we can compromise to interruption ratio-based framework as long as the interruption ratio is properly defined. Some company proposed 0.5% interruption ratio in the previous RAN4 meeting. Take FR1 for example, for every measurement occasion UE may cause 2ms interruption (VIL1+VIL2), which has been widely discussed in NCSG design. If 0.5% interruption is agreed, then UE can only switch on/off the RF chain every 2ms/0.5%=400ms. In other word, if actual measurement sample interval is shorter than 400ms, UE may have to keep the RF chain on most of the time during this 400ms window even if it is not doing measurement. That is unacceptable from UE power consumption perspective.

Option 1 could be a starting point. Some companies showed interest in defining requirement for measurement sample cycle less than 160ms as well. If that is the case, we probably need to introduce a formula to separately calculate interruption ratio for different sample cycle.

One thing we would like to highlight is that the interruption here is only caused by RF tuning/retuning and BB preparation. Some company commented under scheduling restriction issue that no need to define scheduling restriction but consider it as interruption. We do not support that. But if that is the case, then interruption ratio would be increased.

Proposal 1: for interruption due to RF tuning/retuning and BB preparation but not caused by scheduling unavailability during SMTC, the ratio can be defined depending on the measurement cycle length and interruption length as:

- Option 1: with up to [1.25%] probability of interruption per a UE measurement sample cycle when it is NOT less than [160ms] ms

- Option 2: with up to [X%] probability of interruption per a UE measurement sample cycle when it is NOT less than [160ms] ms

  - X=2*interruption length/SMTC periodicity, where interruption length equals to 1ms in FR1 and 0.75ms in FR2.

The next issue is about interruption length:

### Issue 1-1-3: Requirements on the interruption length , if allowed

< Way forward >:

- FFS on:

  - Option 1:  Apple, Intel, CMCC, xiaomi, vivo, OPPO, Huawei, MTK

    - As a starting point, the interruption length can be same as these defined for NCSG,e.g.

      - When UE reporting “[no-gap,TBD]” in [NeedForGapInfoNR, TBD]  the interruption length can be VIL=1ms in FR1 and VIL=0.75ms in FR2.

      - When UE reporting “[others,TBD]” in [NeedForGapInfoNR, TBD] no interruption allowed

  - Option 2: CATT, Nokia,ZTE

    - As a starting point, when UE reporting “no-gap [TBD]” in [NeedForGapInfoNR, TBD]  , the interruption length can be specified based on the same RTT assumption as for NCSG (0.5ms in FR1 and 0.25ms in FR2) interruption occasion.

  - Option 3: Ericsson

    - The interruption length equalling 0.5ms for deactivated SCell measurement can be reused for NeedForGaps measurement.

  - Option 4: Nokia

    - Smaller interruption than these for NCSG is expected.

The cause of interruption in NeedForGap is extremely similar with that of NCSG. Typically, UE would use additional RF and BB resource to conduct measurement so that it can keep data Rx/Tx with serving cell(s) concurrently, which has been extensively discussed in R17 NCSG design. Therefore, it is straightforward to reuse the same interruption length as defined in NCSG.

Proposal 2: interruption length in NeedForGap is same as that defined in NCSG, i.e. 1ms in FR1 and 0.75ms in FR2.

Next issue is about requirement for intra/inter-freq measurement without gap when interruption allowed (case 2):

### Issue 1-2-1 Requirement for intra/inter-freq measurement without gap when interruption allowed (case 2)

< Way forward/Agreement >:

  - Option 1: Apple, MTK

    - Can be FFS after RAN4 agree how to define the interruption (length, location or ratio)

  - Option 2:vivo, Huawei, Ericsson,

    - The deactivated SCell measurement except the measCycleSCell can be a start point

  - Option 2a: 	Huawei

    - Measurement cycle larger than 160ms can be considered

  - Option 3: CATT, Xiaomi, Qualcomm, Nokia,

    - For inter-f case 2,take requirements in 38.133, clause 9.3.9 (inter-freq w/o gap) as a starting point

    - For intra-f case 2, Take requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) as a starting point for the definition of requirements

  - Option 3a: Nokia,

    - considering both deriveSSB-IndexFromCellInter-r17 enabled and disabled

  - Option 4: OPPO, ZTE, MTK

    - Take requirements NCSG requirements in TS38.133 clause 9.2.7 and 9.3.10 as a starting point for intra-f and inter-f case2 respectively.

This issue highly depends on how the interruption will be defined for NeedForGap in previous issues, e.g. whether to define interruption location or interruption ratio. If we go with interruption ratio framework, we believe option 3 could be a good starting point.

Proposal 3: Requirement for intra/inter-freq measurement without gap when interruption allowed (case 2):

- For inter-f case 2,take requirements in 38.133, clause 9.3.9 (inter-freq w/o gap) as a starting point

- For intra-f case 2, Take requirements in Section 9.2.5 of TS38.133 (intra-freq w/o gap) as a starting point for the definition of requirements.

Regarding inter-frequency measurement w/o gap case 1:

### Issue 1-2-2: Requirement for inter-freq measurement without gap (Inter-f case 1)

< Way forward >:

- FFS on:

  - Proposal 1: CATT, CMCC, Huawei

    - to update the definition of inter-frequency SSB based measurements without measurement gaps to include the case when UE indicates ‘no-gap’ via interFreq-needForGap

  - Proposal 2: Intel, CATT,Huawei

    - Updates/Clarification on CSSFoutside_gap.

  - Proposal 3: Nokia, ZTE,  Huawei

    - Define measurement reporting delay requirements for UEs indicating no-gap with interruption considering both deriveSSB-IndexFromCellInter-r17 enabled and disabled

When interruption is not needed, the simplest way is to update the definition of inter-frequency SSB based measurements without measurement gaps to include this case.

Proposal 4: Requirement for inter-freq measurement without gap (Inter-f case 1): update the definition of inter-frequency SSB based measurements without measurement gaps to include the case when [UE indicates [no-gap-no-interruption] in the new indication introduced in R18]. [] is subject to RAN2 design.

Next issue is about mapping between NeedForGap and NCSG capabilities when UE supports both of them:

### Issue 1-3-1: Mapping between NeedForGap and NCSG capabilities when UE supports both of them

< Way forward >:

- FFS on:

  - [NeedForGapsInfoNR] and NeedForGapNCSG-InfoNR are not expected to be enabled for the same UE at the same time

  - No need to establish the mapping between UE’s indication for NeedForGaps and NCSG

We proposed to define some restriction in UE reporting when both features are supported and configured. The motivation is to make sure UE and NW have exactly same understanding on whether interruption is needed on which band(s). Since this issue has been discussed for a while and some companies still don’t want to introduce any restriction, one possible solution is as captured in the possible wayforward. Besides, if RAN4 can reach consensus, it is better to inform RAN2 to avoid potential dependency of two features.

Proposal 5: NeedForGapsInfoNR and NeedForGapNCSG-InfoNR are not expected to be enabled for the same UE at the same time. RAN4 shall inform RAN2 about this.

Next issue is:

### Issue 1-4-1: General principles to define scheduling restriction requirements

< Way forward/ >:

- FFS on:

  - Option 1: Nokia,vivo, OPPO

    - whether the UE supports simultaneousRxTxInterBandCA in FR1.

    - whether deriveSSB-IndexFromCellInter-r17 is enabled and supported by the UE in FR1 and FR2.

  - Option 1a: Nokia,OPPO

    - whether the UE supports simultaneousRxTxInterBandCA in FR1.

    - whether deriveSSB-IndexFromCellInter-r17 is enabled and supported by the UE in FR1 and FR2.

    - whether IBM is supported in FR2.

  - Option 2: Qualcomm

    - No need to introduce scheduling restriction due to interruption for performing inter-frequency measurements.

This issue also highly related to interruption framework. If interruption location is clearly defined, scheduling restriction can also be clearly defined since UE measurement time can be implicitly known by checking the interruption location. If we go with interruption ratio based framework, it is unclear to NW when UE would perform measurement on target MO which could results in interruption.

Figure 1

In above example, UE uses 1st searcher to measure PCC and uses 2nd searcher to measure SCC and neighbor CC. Assuming measurement on SCC does not need any gap, interruption or scheduling restriction while measurement on neighbor CC is done via NeedForGaps with no gap but interruption, and with scheduling restriction. UE would measure neighbor CC on every other SMTC. However, this is unknown to NW since which CC to measure during each SMTC is up to UE implementation. Conservatively, scheduling restriction needs to be applied for all SMTC.

Observation 1: if interruption location is clearly defined, NW knows when UE would perform measurement on MO via NeedForGaps. Therefore, scheduling restriction can be clearly defined.

Observation 2: if interruption ratio based framework is agreed without clear interruption location, NW doesn’t know when UE would perform measurement on CC which could cause scheduling restriction.

Proposal 6: if interruption ratio framework is agreed without clear interruption location, scheduling restriction shall apply on all the SMTC in the MO indicated with [interruption] in NeedForGaps.

# Conclusion

In this contribution, we provide further discussion on NeedForGaps requirements. After discussion, the following conclusions are provided:

Proposal 1: for interruption due to RF tuning/retuning and BB preparation but not caused by scheduling unavailability during SMTC, the ratio can be defined depending on the measurement cycle length and interruption length as:

Proposal 2: interruption length in NeedForGap is same as that defined in NCSG, i.e. 1ms in FR1 and 0.75ms in FR2.

Proposal 3: Requirement for intra/inter-freq measurement without gap when interruption allowed (case 2):

Proposal 4: Requirement for inter-freq measurement without gap (Inter-f case 1): update the definition of inter-frequency SSB based measurements without measurement gaps to include the case when [UE indicates [no-gap-no-interruption] in the new indication introduced in R18]. [] is subject to RAN2 design.

Proposal 5: NeedForGapsInfoNR and NeedForGapNCSG-InfoNR are not expected to be enabled for the same UE at the same time. RAN4 shall inform RAN2 about this.

Observation 1: if interruption location is clearly defined, NW knows when UE would perform measurement on MO via NeedForGaps. Therefore, scheduling restriction can be clearly defined.

Observation 2: if interruption ratio based framework is agreed without clear interruption location, NW doesn’t know when UE would perform measurement on CC which could cause scheduling restriction.

Proposal 6: if interruption ratio framework is agreed without clear interruption location, scheduling restriction shall apply on all the SMTC in the MO indicated with [interruption] in NeedForGaps.

# References

- R4-2303305, WF on NR_MG_enh2 Part 2, Intel Corporation.

- RP-230755, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation