3GPP TSG-RAN WG4 Meeting #106-bis-e									         R4-2304299

Electronic, April 17 – 26, 2023

Agenda item:	5.26.2.3

Source:	Apple

Title:	On R18 MUSIM - network A requirements

Document for:	Discussion

# Introduction

Network A requirements were widely discussed during the previous RAN4 meetings. The last agreements can be found in [1], in which there are still some open issues.

In this contribution, we continue discussing the open issues.

# Discussion

The key issue is about principle on layer 1 and layer 3 measurement requirements after gap collision handling:

Issue 3-1-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling

- Proposals

  - P1: Frameworks of LBT failure in NR-U design can be used as starting point when discussing NW A L3/L1 requirement impact due to MUSIM gaps (Apple)

  - P2: Reuse the principle used in Rel-17 concurrent gaps WI as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Ntotal /Navailable for network A requirements when MUSIM gaps are configured. (CMCC xiaomi vivo ZTE oppo Huawei Ericsson Qualcomm MTK Nokia)

  - P3: Since principle used in Rel-17 concurrent gaps WI on layer 1 and layer 3 measurement requirements is derived after gap collision handling, it can be reused for even the gap handling solution within MUSIM gap is not fully determined, no need to postpone the discussion. (vivo)

  - P4: RAN4 to postpone the detail NW-A’s requirement discussion until RAN4 achieves the consensus on MUSIM gaps’ priority. (Ericsson)

Issue 3-1-2: On parameters for L1/L3 measurement requirements

- Proposals

  - P1: (CMCC):

    - For L3 measurement, Navailable need to be updated to cover MUSIM gaps

    - For L1 measurement, Noutside_MG and Navailable need to be updated to cover MUSIM gaps

  - P2: The following parameters need to be updated to account for collisions with MUSIM gaps (Qualcomm MTK):

    - Kp for intra-frequency and inter-frequency measurements without gaps (Xiaomi)

    - Kgap for intra-frequency and inter-frequency measurements with gaps (Xiaomi)

    - Kgap_EUTRA for inter-RAT measurements

    - Kp_CSI-RS for CSI-RS L3 measurements

    - Kp,PRS,I for NR positioning measurements

    - CSSFintra for intra-frequency measurements

    - CSSFinter for intra-frequency measurements

    - CSSFinterRAT for intra-RAT measurements

    - P scaling factor for L1-RSRP and L1-SINR measurements (xiaomi)

  - P3: Suggest the following update on parameters for L1/L3 measurement requirements (vivo ZTE)

    - For SSB based or CSI-RS based RLM, BFD and CBD, scaling factor P can be reused without any update, the definition of Ntotal, Noutside_MG and Navailable need updated.

    - For intra-frequency and inter-frequency measurements without gaps, scaling factor Kp can be reused and the definition of Ntotal and Navailable will be updated.

    - For intra-frequency and inter-frequency measurements with gaps, scaling factor Kgap can be reused and the definition of Ntotal and Navailable will be updated.

    - For inter-RAT E-UTRAN TDD/FDD measurement, scaling factor Kgap_EUTRA can be reused and the definition of Ntotal and Navailable will be updated.

    - For L1-RSRP/L1-SINR measurement, scaling factor P can be reused and the definition of Ntotal, Noutside_MG and Navailable will be updated.

    - For NR measurement for positioning, scaling factor Kp,PRSIi can be reused and the definition of Ntotal and Navailable will be updated.

    - For CSI-RS based L3 measurements, scaling factor Kp_CSI-RS can be reused and the definition of Ntotal and Navailable will be updated.

  - P4: (Huawei)

    - For L3 measurement outside MG, Kp in the requirements is updated

      - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion or non-dropped MUSIM gap occasion within the window W.

    - For L3 and positioning measurement with MG, existing requirements can be re-used.

    - For L1 measurement outside MG, Navailable, Noutside_MG in the requirements are updated

      - Noutside_MG is the number of SSB resource occasions that are not overlapped with any MG nor MUSIM gap within the window W

      - Navailable is the number of SSB resource occasions that are not overlapped with any MG, MUSIM gap nor any SMTC occasion within the window W

  - P5: Clarification for L3 measurement without gap, SMTC should not be fully overlapping with MUSIM gap (oppo)

Recommended WF

Depends on outcome of issue 3-1-1

Issue 3-1-3: On the time window W for aperiodic gap

- Proposals

  - P1: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer. (vivo ZTE oppo Huawei)

  - P2: max(SMTC period, MGRP_max)+[M], where MGRP_max is the largest periodicity among all the periodic gaps and [M] is a time margin for the one-shot aperiodic MUSIM gap. (MTK)

    - P2-1: If P3 is used, the new window length proposed by P3 will only apply during the window where aperiodic gap locates. (vivo)

Technically both P1 and P2 under issue 3-1-1 can address the collision between MUSIM gap and L1/L3 measurement occasion. Option 1 is based on NR-U design while option 2 is based on concurrent gaps design. We prefer option 1 since option 2 may result in unnecessary additional measurement delay. Option 2 works well in concurrent gaps because MGRP of concurrent gap is relatively short compared to measurement period. However, MGRP of MUSIM gap can be up to 5120ms, which is much longer than L1/L3 measurement period typically. Reusing concurrent gaps approach here may become inefficient. For example, if the paging cycle is 2.56s and there is MUSIM with 2.56s MGRP, which is overlapped with SMTC for L3 measurement. Assuming SMTC sampling interval is 40ms, then Kx= Navailable / Ntotal = 63/64.

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.8.3.

Measurement period would be ceil( 5 x Kp) x SMTC period = ceil(5.08) x 40 = 6 x 40. As can be observed, during a window W of 2560ms, the measurement period is 6 x 40. However, this is unnecessary since most of the time there is no collision between MUSIM gap and SMTC. Thus, the measurement period could be 5 x 40 most of the time during W and only one measurement period needs to be extended by one SMTC.

Therefore, NR-U framework is preferred, i.e. counting unavailable L3/L1 occasion due to MUSIM gaps. Besides, it can also address the collision due to aperiodic MUSIM gap.

Observation 1: when long MGRP of MUSIM gap is configured, reusing concurrent gaps design (Kx = Navailable / Ntotal) would result in unnecessary long L3/L1 measurement period.

|  | Intra-frequency measurement delay |
| --- | --- |
| NR-U framework | 5 x 40ms for most of the time during 2560ms, only one measurement cycle is extended to 6 x 40ms. |
| Concurrent gaps framework | 6 x 40ms during whole 2560ms window |
| Assumption: 2.56s paging cycle and MUSIM MGRP is 2.56s, SMTC sampling interval is 40ms. |  |

Observation 2: LBT failure model in NR-U requirement design can also address collision between MUSIM gap and L3/L1 measurement occasion without above problem. Besides, it can better address aperiodic MUSIM gap.

Proposal 1: Frameworks of LBT failure in NR-U design can be used as a starting point when discussing NW A L3/L1 requirement impact due to MUSIM gaps.

# Conclusion

In this contribution, we provide further discussion on improvement on network A requirements. After discussion, the following conclusions are provided:

Observation 1: when long MGRP of MUSIM gap is configured, reusing concurrent gaps design (Kx = Navailable / Ntotal) would result in unnecessary long L3/L1 measurement period.

|  | Intra-frequency measurement delay |
| --- | --- |
| NR-U framework | 5 x 40ms for most of the time during 2560ms, only one measurement cycle is extended to 6 x 40ms. |
| Concurrent gaps framework | 6 x 40ms during whole 2560ms window |
| Assumption: 2.56s paging cycle and MUSIM MGRP is 2.56s, SMTC sampling interval is 40ms. |  |

Observation 2: LBT failure model in NR-U requirement design can also address collision between MUSIM gap and L3/L1 measurement occasion without above problem. Besides, it can better address aperiodic MUSIM gap.

Proposal 1: Frameworks of LBT failure in NR-U design can be used as a starting point when discussing NW A L3/L1 requirement impact due to MUSIM gaps.

# References

- R4-2303310, WF on NR Dual TxRx Multi-SIM, vivo

- R4-2303249, LS on priority levels for MUSIM gaps, vivo