3GPP TSG-RAN WG4 Meeting #106-bis-e									         R4-2304303

Electronic, April 17 – 26, 2023

Agenda item:	5.11.2.1

Source:	Apple

Title:	On BWP w/o restriction - RRM core requirements for option A

Document for:	Discussion

# Introduction

In RANP#99 a new work item on BWP without restriction was approved [1]. Objectives are duplicated here for information:

- For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to IoDT availability). - The interruption related requirements will be decided and specified in RAN4. The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

RAN4 TU starts from this meeting. In this contribution, we provide initial discussion on RRM requirements for option A.

# Discussion

Scope of option A is to study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc, for CSI-RS based BM/RLM/BFD. This aspect has been discussed for several meetings. The latest agreement can be found in the approved WF in RAN4#106 [2]:

#### If CSI-RS based RLM/BFD/BM requirements are complete for Option A)

<Agreement >:

- For the UE performing the BM/RLM/BFD based on CSI-RS within the active BWP (Option A), existing BM/RLM/BFD requirements defined in TS 38.133 is complete and no new or additional requirements are needed.

#### Issue 1-2: Whether timing requirements should be enhanced for the case when CD-SSB is outside active BWP

< Way forward >

- The existing UE transmission timing error requirements based on the SSB defined in clause 7.1 of TS 38.133 shall apply for the UE performing the BM/RLM/BFD based on CSI-RS within the active BWP (Option A), i.e., no additional timing requirements are needed.

- FFS if any clarifications on the existing requirements is needed, e.g., applicability of requirements, conditions of gap configuration etc.

As it can be observed, it has been agreed that for the UE performing the BM/RLM/BFD based on CSI-RS within the active BWP (Option A), existing BM/RLM/BFD requirements defined in TS 38.133 is complete and no new or additional requirements are needed. The only FFS is about if any clarifications on the existing requirements is needed, e.g., applicability of requirements, conditions of gap configuration etc, in the existing UE transmission timing error requirements based on the SSB. According to clause 7.1 of TS 38.133, one of the applicability conditions in existing Te requirement is the availability of SSB:

When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 40 ms.

Normally, if UE active BWP contains SSB from the serving cell, then UE has chance to use SSB to refine the DL timing so that the UL Tx timing can be guaranteed. The scenario we are facing here in option A is the case wherein UE active BWP doesn’t contain SSB. Network configures CSI-RS in the active BWP so that UE can perform BM/RLM/BFD on that BWP. Similar to RLM, RRM measurement on serving cell is also essential. In order to perform RRM measurement when active BWP doesn’t contain SSB, typically UE needs measurement gap. As optional features, RRM measurement on serving cell can be done via NCSG or NeedForGaps, if supported. Therefore, we think the availability of SSB in Te requirement condition can be implicitly met. Thus it seems no need to explicitly add more applicability of gap configuration.

Observation 1: for option A, UE anyway needs to measure serving cell, e.g. with legacy MG, NCSG or NeedForGaps. UE has chance to measure CD-SSB. No clarification is needed in existing Te requirements.

# Conclusion

In this contribution, we provide initial discussion on BWP without restriction – option A. After discussion, the following conclusions are provided:

Observation 1: for option A, UE anyway needs to measure serving cell, e.g. with legacy MG, NCSG or NeedForGaps. UE has chance to measure CD-SSB. No clarification is needed in existing Te requirements.

# References

- RP-230805, New WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, vivo

- R4-2303313, WF on BWP_withoutRestriction, vivo