3GPP RAN WG4 Meeting #106bis		R4-2304330

Electronic meeting, April 17th – 26th, 2023

Agenda item:	4.32.2

Source:	Apple, Globalstar

Title:	System parameters for the NTN L-/S-band

WI/SI:	NR_NTN_LSband

Release:	Rel-18

Document for:	Discussion

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments that use mixed L-/S-band pairing, whereupon the UL part is one the L-band, while the DL part is one the S-band. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI. So, during the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band where the UL is on the L-band and the DL is on the S-band [2].

During the RAN#106 meeting an initial discussion took place and some basic agreements for the system parameters were made, such as band definition [3]. However, not all the system related aspects were finalised and, thus, in this paper we present our further proposals for the system parameters of the new NTN band.

# 2	System parameters for the NTN L-/S-band

## 2.1	Channel bandwidth and asymmetric combinations

Referring again to the WI description [2], the objective is to enable the following channel bandwidths for the NTN band: 5, 10 and 15MHz. Even though the total bandwidth of the new band is 16.5MHz, the next lower standard channel bandwidth is 15MHz.

Proposal 1a:	Introduce 5, 10 and 15MHz channel bandwidths for the new NTN band.

In addition to defining 5, 10 and 15MHz channel bandwidths as the baseline symmetric combinations, it would be preferrable to introduce also asymmetric channel bandwidth combinations to allow for more flexible system deployments. In fact, somewhat similar to the design of the terrestrial bands n91-n94, a higher degree of the deployment flexibility can be achieved if the asymmetric channel combinations are accompanied with the variable Tx-Rx separation distance.

Proposal 1b:	Introduce 5+10MHz, 5+15MHz, and 10+15MHz UL/DL asymmetric channel combinations for the new NTN band.

Proposal 1c:	The Tx-RX separation distance is defined as 873.5 – 885 MHz for the new NTN band.

The corresponding changes for the baseline channels, asymmetric channel bandwidth combinations and the Tx-RX separation distance will look as follows (refer to [5] for the complete changes).

| NTN satellite band | SCS kHz | UE Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| n256 | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |
|  | 15 | 5 | 10 | 15 | 20 |
| n255 | 30 |  | 10 | 15 | 20 |
|  | 60 |  | 10 | 15 | 20 |
| [n254] | 15 | 5 | 10 | 15 |  |
|  | 30 |  | 10 | 15 |  |
|  | 60 |  | 10 | 15 |  |

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| [n254] | 5 | 10,15 | 0 |
|  | 10 | 15 | 0 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [15], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz |
| n255 | -101.5 MHz |
| [n254] | 873.5 – 885 MHz |

## 2.2	Channel raster and sync raster

The channel raster and the sync raster points can be defined based on the same principles that 3GPP has already taken for the two existing NTN bands. In fact, the existing NTN bands – n255 and n256 – follow the same principle as for the terrestrial bands. The channel raster points are defined in step of <20>, i.e. in step of 100kHz, so the same approach can be applied to the new NTN band [n254]. Similarly, the definition of the sync raster points can be done in the same way as for the FR1 NTN and terrestrial bands.

Proposal 2a:	Channel raster points are specified in steps of <20> with the UL channel raster points at 322000 – 325300 and the DL channel raster points at 496700 –500000.

Proposal 2b:	Sync raster points are specified in steps of <1> with the sync raster points at 6215 – 6244.

The corresponding changes in the specification for the channel raster and the sync raster points will look as presented below.

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| [n254] | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| [n254] | 15kHz | Case A | 6215 – <1> – 6244 |
| NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]. |  |  |  |

# 3	Conclusions

In this paper we have presented our further considerations on the band plan and system parameters for the new NTN band. The corresponding changes to TS 38.101-5 can be found in [5].

Proposal 1a:	Introduce 5, 10 and 15MHz channel bandwidths for the new NTN band.

Proposal 1b:	Introduce 5+10MHz, 5+15MHz, and 10+15MHz UL/DL asymmetric channel combinations for the new NTN band.

Proposal 1c:	The Tx-RX separation distance is defined as 873.5 – 885 MHz for the new NTN band.

Proposal 2a:	Channel raster points are specified in steps of <20> with the UL channel raster points at 322000 – 325300 and the DL channel raster points at 496700 –500000.

Proposal 2b:	Sync raster points are specified in steps of <1> with the sync raster points at 6215 – 6244.

# 4	References

RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

R4-2303531, "WF on NTN L-/S-band", Apple

R4-2304331, "RF requirements for the NTN L-/S-band", Apple, Globalstar

R4-2304332, "Introduction of the NTN L-/S-band", Apple, Globalstar