3GPP TSG-RAN4 Meeting #106-bis-e	R4-2304341

Online, 17th April – 26th April 2023

Agenda item:	4.10.2

Source:	Apple

Title:	MSD requirements for CA_n71A-n85A

WI/SI:	NR_CADC_R18_2BDL_xBUL-Core

Release:	Rel-18

Document for:	Approval

# Introduction

In RAN4#105 meeting, the CA_71A-85A topic was discussed an the following WF [1] was adopted:

- Issue: 1-1: 1UL CA_n71-n85 architecture and requirement

  - Agreement:

    - Dual triplexer approach can be used as a start with UL on each antenna.

    - n29 band protection level needs to be clarified as follows :

      - When the uplink is n71, the protection level of n29 is -38 dBm/MHz.

      - When the uplink is n85 there is no protection for n29.

- 3 antennas not precluded.

- Companies encouraged to bring proposals for MSD for UL n71 into DL n85, and UL n85 into DL n71.

- Companies encouraged to check potential MSD for non-CA n71 and n85 REFSENS.

  - This is because in case of triplexer the IL and isolations may be degraded compared to single band isolations.

As a result of the WF above, few companies provided their MSD analysis for the CA_71A-85A in the last meeting (RAN4#106) but the decision to add this MSD requirement in the specifications was postponed.

In this proposal, we present the results of our MSD analysis for this band combination. Our analysis will be based on the two-antenna architecture using 2 triplexers, one for the main path, and the other one for the diversity path. We will derive the cross-band MSD for both n71ULm85DL and n85ULn71DL.

# Discussion

  - Architecture

As can be seen on Figure 2.1-1 below, there is no frequency gap between the n71UL and n85UL. This makes is impossible to design a multiplexer to connect to route the two bands to a single antenna as it’s usually done for typical LB-HB CA combinations.

Figure 2.1-1: Spectrum allocation for the 600MHz-700MHz bands

There are two potential solutions that can be considered for this analysis as outlined in the noted in the WF: one solution that is based on 2-antenna architecture,  and another solution based on 3-antenna architecture. The 3-antenna architecture has the advantage of being based on existing filters so it can be implemented quickly for faster time to market. Additionally, because the 3-antenna solution uses narrowband antennas, their radiated performance can be easily optimized.

However, using three LB antennas could be a challenge for UE implementation due to the limited antenna volume. That makes the 3-antenna approach more suitable for FWA applications where antenna volume is less challenging. Our analysis is for UE applications and therefore we will consider the 2-antenna architecture. This architecture uses two triplexers as shown in Figures 2.1-2 and 2.1-3:

- One triplexer to route n71DL, n71UL, and n85DL to the main antenna.

- Another triplexer to route n71DL, n85UL, and n85DL to the diversity antenna.

Figure 2.1-2: Front-end architecture for the main path          Figure 2.1-3: Diversity path

For the n71ULn85DL case, we assumed a worst-case n71UL to n85DL triplexer isolation of 40dB so that the n71UL/n71DL isolation of 55dB required for the single-band operation is maintained. Antenna rejection of 35dB (from the diplexer) is considered. For the n85ULn71DL, 43dB of the n85UL/n71DL isolation is used with the antenna rejection (diplexer) of 40dB. Antenna isolation of 10dB is also assumed.

  - Effects of cross band interference

- For n71UL n85DL case, as depicted in Figure 2.2-1, the ACLR2 of the n71UL will be the source of interference in n85DL: As an example, when fTX = 690MHz, the ACLR2 will be at fACLR2 = 690 + 2*20 = 730MHz which lands in the n85DL with the assumption that the channel BW is CBW=20MHz which is the maximum BW for that band. The ACLR2 is dominated by IMD5.

Figure 2.2-1: n71 Tx effects on the n85 Rx

- For n85UL n71DL case, as depicted in Figure 2.2-2, the ACLR4 of the n85UL will be the source of interference in n71 DL: As an example, when fTX = 705MHz, the ACLR4 will be at fACLR2 = 705 - 4*15 = 645MHz which lands in the n71DL with the assumption that the channel BW is CBW=15MHz which is the maximum BW for that band. The ACLR4 is dominated by IMD9, which should be at significantly lower levels when compared to the IMD5 for the n71ULn85DL case.

Figure 2.2-2: n85 Tx effects on the n71 Rx

# MSD Analysis

The results of our analysis are summarized in the Figure 3-1 below.

Figure 3-1: Cross-band MSD analysis for the LB-LB CA_71A-85A combination

# Conclusion

In this discussion paper, we shared the results of our analysis of the cross-band MSD for CA_n71A-n85A (see Table 4-1 below). This analysis was based on a two-antenna architecture using two triplexers.

Proposal: Table 3-1 shows our proposed value for the n5 MSD for CA_n5n71A-n85A combination.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.5 | ACLR2 |
| n85 | n71 | 705.5 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 4.1 | ACLR4 |

Table 3-1: MSD due to cross band isolation for the CA_71A-85A combination

# 4	References

- R4-2220560 WF on 1UL CA_n71-n85 architecture and requirement, T-Mobile USA, RAN4#105