3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304356

# Oneline, April 17 – April 26, 2023

Agenda item:	5.31.3.2

Source:	Qualcomm Inc.

Title:	On NR SL RRM

Document for:	Discussion

# Introduction

In this contribution, we present our view on the scope of R18 SL enhancement.

# Discussion

We have the following observations on potential RAN4 impact from different SL procedures discussed in RAN1.

## RRM scope for SL-U

Proposal 1: The following legacy requirements are applicable to SL-U

- Interruptions: LBT procedure has no impact on the interruption lengths and applicability conditions

- Scheduling availability: scheduling availability is defined based on Tx switching time, and the same Tx switching time options apply to operating on unlicensed band.

- UE transmit timing: the transmit timing requirement is derived based on SLSS sampling rate and SCS, and the same requirements are applicable to unlicensed band operation given SCSs of the synchronization source and the DUT SL UE.

For initiation/cease of SLSS transmissions with SyncRef UE as synchronization reference source, we can use NR-U requirement methodology as a reference:

Proposal 2: Consider the following requirement for initiation/cease of SLSS transmissions with SyncRef UE as synchronization reference source

Extending the measurement period to 4+x S-SSB periods, x is the S-SSB periods in which the SLSS is not available due to LBT failures, and is capped by x_max.

Observation 1: The definition of S-SSB periods with SLSS unavailable also needs to be clarified, but the exact definition depends on how RAN1 designs the additional slots for SLSS transmission.

For requirement on UE searching for new detectable SyncRef UE when the UE is synchronized to a SyncRef UE that is synchronized to GNSS directly or in-directly, we can consider a similar extension as a starting point, but now there are two parameters to consider: SLSS Tx dropping rate and SLSS identification measurement time, and changes on the two values should be considered jointly to ensure that UE can measure in at least 3 S-SSB periods with SLSS available within the required measurement period.

Proposal 3: Modify the legacy requirement for UE searching for new detectable SyncRef UE when the UE is synchronized to a SyncRef UE that is synchronized to GNSS directly or in-directly to account for number of S-SSB periods with SLSS unavailable and ensure that UE can measure in at least 3 S-SSB periods with SLSS available within the required measurement period.

For cases other than above and synchronized direct to GNSS, the search requirements aims at merging synchronization clusters faster by the asynchronized search covering the entire S-SSB period.

Observation 2: We don’t expect the synchronization cluster merging as a common procedure in the target application scenarios for SL-U with good system performance improvement.

Based on the above observation, we have the following proposal:

Proposal 4: For other cases (not synchronize directly or indirectly to GNSS), maintain the data transmission dropping requirement and extend the search period requirement as:

The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331[2]. Tdetect,SyncRef UE_V2X is defined as (8+0.16z) seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the UE is allowed to drop at most an aggregated window of 480ms (6 % of 8s in legacy requirement) of its V2X data and SLSS transmissions during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

The value of z is determined by the number of S-SSB periods with SLSS unavailable.

We have the following observations for L1 SL-RSRP related requirements.

Observation 3: Since the L1 SL-RSRP measurement may be applicable to multiple slots in multiple consecutive slot transmission (MCSt) scheme, there is potential impact on RAN4 L1 SL-RSRP depending on RAN1 procedure design.

Observation 4: UE may need to measure L1 SL-RSRP on additional PSCCH locations when half slot transmission is introduced, and therefore potential RAN4 impact can be discussed.

# Conclusions

Proposal 1: The following legacy requirements are applicable to SL-U

- Interruptions: LBT procedure has no impact on the interruption lengths and applicability conditions

- Scheduling availability: scheduling availability is defined based on Tx switching time, and the same Tx switching time options apply to operating on unlicensed band.

- UE transmit timing: the transmit timing requirement is derived based on SLSS sampling rate and SCS, and the same requirements are applicable to unlicensed band operation given SCSs of the synchronization source and the DUT SL UE.

Proposal 2: Consider the following requirement for initiation/cease of SLSS transmissions with SyncRef UE as synchronization reference source

Extending the measurement period to 4+x S-SSB periods, x is the S-SSB periods in which the SLSS is not available due to LBT failures, and is capped by x_max.

Observation 1: The definition of S-SSB periods with SLSS unavailable also needs to be clarified, but the exact definition depends on how RAN1 designs the additional slots for SLSS transmission.

Proposal 3: Modify the legacy requirement for UE searching for new detectable SyncRef UE when the UE is synchronized to a SyncRef UE that is synchronized to GNSS directly or in-directly to account for number of S-SSB periods with SLSS unavailable and ensure that UE can measure in at least 3 S-SSB periods with SLSS available within the required measurement period.

Observation 2: We don’t expect the synchronization cluster merging as a common procedure in the target application scenarios for SL-U with good system performance improvement.

Proposal 4: For other cases (not synchronize directly or indirectly to GNSS), maintain the data transmission dropping requirement and extend the search period requirement as:

The UE shall be able to identify newly detectable intra-frequency SyncRef UE within Tdetect,SyncRef UE_V2X seconds if the SyncRef UE meets the selection / reselection criterion defined in TS 38.331[2]. Tdetect,SyncRef UE_V2X is defined as (8+0.16z) seconds at S-SSB Ês/Iot ≥ 0 dB, provided that the UE is allowed to drop at most an aggregated window of 480ms (6 % of 8s in legacy requirement) of its V2X data and SLSS transmissions during Tdetect,SyncRef UE_V2X for the purpose of selection / reselection to the SyncRef UE.

The value of z is determined by the number of S-SSB periods with SLSS unavailable.

Observation 3: Since the L1 SL-RSRP measurement may be applicable to multiple slots in multiple consecutive slot transmission (MCSt) scheme, there is potential impact on RAN4 L1 SL-RSRP depending on RAN1 procedure design.

Observation 4: UE may need to measure L1 SL-RSRP on additional PSCCH locations when half slot transmission is introduced, and therefore potential RAN4 impact can be discussed.