3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304359

# Oneline, April 17 – April 26, 2023

Agenda item:	5.19.1.1

Source: 	Qualcomm Incorporated

Title: 	MU-MIMO enhancement simulation assumptions and network assistant signaling

Document for:	Approval

# Introduction

In this contribution, we provide our view on the R18 demod enhancement on MU-MIMO.

# Discussion

## Implementation feasibility of R-ML receiver for MU-MIMO and network assistant signaling

The necessary information to enable R-ML receiver is discussed in the previous meeting. Considering the overhead, we believe the MCS index table via RRC signaling is a beneficial signaling which can reduce the search space for modulation order detection with a very low overhead.

Proposal 1: Introduce the RRC based signaling to inform UE the MCS index table(s) used for PDSCH of the co-scheduled UEs.

For co-scheduled UEs in different CDM groups, detecting their existence and modulation orders becomes prohibitively expensive when the PRG allocation misalignment across different CDM groups. We have the following observation:

Observation 1: When PRG allocation is aligned across different CDM groups, the complexity of co-scheduled UE detection and the modulation order detection is reduced by at least x times, x is the PRG size.

Therefore, to enable R-ML receiver in the real deployment, we have the following proposal:

Proposal 2: Introduce the signaling to inform UE whether the PRG allocation is aligned across the co-scheduled UEs in different CDM groups. The signaling format depends on how static this information is in the real deployment.

We also consider DMRS power boosting for the co-scheduled UE is a necessary information, and therefore we have the following proposal:

Proposal 3: Introduce the signaling to inform UE numbers of CDM groups without data of the co-scheduled UEs, or at least inform UE whether numbers of CDM groups without data are aligned across all co-scheduled UEs.

PDSCH allocation alignment across co-scheduled UEs has a significant impact on the R-ML receiver:

Observation 2: The co-scheduled UE detection can’t be performed in a per-symbol basis due to the availability of DMRS and complexity constraints, and therefore, the interference in R-ML receiver applied uniformly to all the PDSCH symbols. When the interference presence is not uniform in a slot, R-ML receiver tries to cancel a non-existent interference, and end up introduce additional noise to fail the decoding of the symbol.

Therefore, we have the following proposal:

Proposal 4: Introduce the following network assistant signaling to enable the correct interference cancellation when implementing R-ML receiver:

- Whether all the serving PDSCH symbols are interfered by the same set of co-scheduled UEs, if not which serving PDSCH symbols are interfered by the same set of co-scheduled UEs

- Whether the interference signal contains one or more PT-RS or CSI-RS resources transmitted for the co-scheduled UEs

- Whether scrambling sequences are aligned between the target UE and all the co-scheduled UEs

Regarding the modulation order, in the previous meeting, many network vendors and operators mentioned the flexibility of scheduling more than one co-scheduled UEs (besides target UE) or having different modulation orders on multiple co-scheduled UEs is inevitable and may happen frequently. In our opinion, aligned modulation order on the (interfering) co-scheduled UE is beneficial to UE, but if this is not a common case that can be ensure by network vendors and operators, we need to discuss how beneficial is the signaling.

Proposal 5: The alignment of modulation order across all the (interfering) co-scheduled UEs is an important information for R-ML receiver, and RAN4 can study how beneficial it is to introduce the signaling to inform UE whether the modulation orders are aligned.

For the existence of co-scheduled UEs, we have the following proposal:

Proposal 6: Informing UE whether the co-scheduled UE exists across the entire bandwidth is critical for UE to decide whether to enable R-ML or use legacy receiver. However, the existence of co-scheduled UEs on each port and RE signaling requires huge overhead and may not be practical.

Proposal 7: Network should ensure that DMRS sequence and seed are aligned across co-scheduled UEs.

## Analysis for comparison between E-LMMSE-IRC receiver and LMMSE-IRC receiver

Consider the MU-MIMO system model:

rk, l= H1k,ld1k,l+j=2NUEHjk,ldjk,l+n(k,l)

The LMMSE-IRC receiver (baseline) detect signal d1k,l by first estimating the noise including interfering signal and the Gaussian noise:

rIRCk,l=rk, l-H1k,ld1k,l=j=2NUEHjk,ldjk,l+n(k,l)

rIRCk,lrIRCk,lH=rk, l-H1k,ld1k,lrk, l-H1k,ld1k,lH=rk, l-j=1NUEHjk,ldjk,l+j=2NUEHjk,ldjk,lrk, l-j=1NUEHjk,ldjk,l+j=2NUEHjk,ldjk,lH=rk, l-j=1NUEHjk,ldjk,lrk, l-j=1NUEHjk,ldjk,lH+2rk, l-j=1NUEHjk,ldjk,lj=2NUEHjk,ldjk,lH+j=2NUEHjk,ldjk,lj=2NUEHjk,ldjk,lH

Assume the signal vectors across different UEs are independent, then when we average over signal vector realizations,

Ej=2NUEHjk,ldjk,lj=2NUEHjk,ldjk,lH=j=2NUEPjHjk,lHjHk,l

Where Pj=trace(Edjk,ldjk,lH), i.e., the average power of transmission to UE j.

Assume the signal vectors of all UEs are independent with noise, then when we average over signal vector and noise realizations

Erk, l-j=1NUEHjk,ldjk,lj=2NUEHjk,ldjk,lH=En(k,l)j=2NUEHjk,ldjk,lH=Ej=2NUEn(k,l)djk,lHHjk,lH=0

Therefore, by averaging over signal vector and noise realizations, we have (use ~ to denote the averaging)

rIRCk,lrIRCk,lH~rk, l-j=1NUEHjk,ldjk,lrk, l-j=1NUEHjk,ldjk,lH+j=2NUEPjHjk,lHjHk,l

The receiver weight matrix is

RIRC= P1H1k,lH1Hk,l+1Nrek,l∈DMRS of NUErIRCk,lrIRCk,lH~ P1H1k,lH1Hk,l+1Nrek,l∈DMRS of NUErk, l-j=1NUEHjk,ldjk,lrk, l-j=1NUEHjk,ldjk,lH=P1H1k,lH1Hk,l+j=2NUEPjHjk,lHjHk,l+1Nrek,l∈DMRS of NUEr(k,l)r(k,l)H=RE-MMSE

Where RE-MMSE is from the formulation agreed in the previous meeting.

Therefore, we conclude:

Observation 3: LMMSE-IRC and E-LMMSE-IRC are statistically equivalent and can be proved mathematically. Given that the channel estimation is done across multiple REs and the simulations are ran in a long enough duration to have converged throughput, we expect the E-LMMSE-IRC receiver to have exactly the same performance as LMMSE-IRC.

Proposal 8: Do not consider additional requirement for E-LMMSE-IRC receiver.

## Advanced MU-MIMO Receiver Simulations and Requirement Recommendations

Proposal 9: When defining the requirement, the precoding matrices across co-scheduled UEs should be orthogonal given that it is a simple enhancement from the network to achieve a better performance in MU-MIMO scenarios.

Proposal 10: Based on our results, MCS 13 2+2 in TDL-A channel with 16QAM interference is a good representative scenario to verify performance gain by R-ML receiver.

# Conclusion

Proposal 1: Introduce the RRC based signaling to inform UE the MCS index table(s) used for PDSCH of the co-scheduled UEs.

Observation 1: When PRG allocation is aligned across different CDM groups, the complexity of co-scheduled UE detection and the modulation order detection is reduced by at least x times, x is the PRG size.

Proposal 2: Introduce the signaling to inform UE whether the PRG allocation is aligned across the co-scheduled UEs in different CDM groups. The signaling format depends on how static this information is in the real deployment.

Proposal 3: Introduce the signaling to inform UE numbers of CDM groups without data of the co-scheduled UEs, or at least inform UE whether numbers of CDM groups without data are aligned across all co-scheduled UEs.

Observation 2: The co-scheduled UE detection can’t be performed in a per-symbol basis due to the availability of DMRS and complexity constraints, and therefore, the interference in R-ML receiver applied uniformly to all the PDSCH symbols. When the interference presence is not uniform in a slot, R-ML receiver tries to cancel a non-existent interference, and end up introduce additional noise to fail the decoding of the symbol.

Proposal 4: Introduce the following network assistant signaling to enable the correct interference cancellation when implementing R-ML receiver:

- Whether all the serving PDSCH symbols are interfered by the same set of co-scheduled UEs, if not which serving PDSCH symbols are interfered by the same set of co-scheduled UEs

- Whether the interference signal contains one or more PT-RS or CSI-RS resources transmitted for the co-scheduled UEs

- Whether scrambling sequences are aligned between the target UE and all the co-scheduled UEs

Proposal 5: The alignment of modulation order across all the (interfering) co-scheduled UEs is an important information for R-ML receiver, and RAN4 can study how beneficial it is to introduce the signaling to inform UE whether the modulation orders are aligned.

Proposal 6: Informing UE whether the co-scheduled UE exists across the entire bandwidth is critical for UE to decide whether to enable R-ML or use legacy receiver. However, the existence of co-scheduled UEs on each port and RE signaling requires huge overhead and may not be practical.

Proposal 7: Network should ensure that DMRS sequence and seed are aligned across co-scheduled UEs.

Observation 3: LMMSE-IRC and E-LMMSE-IRC are statistically equivalent and can be proved mathematically. Given that the channel estimation is done across multiple REs and the simulations are ran in a long enough duration to have converged throughput, we expect the E-LMMSE-IRC receiver to have exactly the same performance as LMMSE-IRC.

Proposal 8: Do not consider additional requirement for E-LMMSE-IRC receiver.

Proposal 9: When defining the requirement, the precoding matrices across co-scheduled UEs should be orthogonal given that it is a simple enhancement from the network to achieve a better performance in MU-MIMO scenarios.

Proposal 10: Based on our results, MCS 13 2+2 in TDL-A channel with 16QAM interference is a good representative scenario to verify performance gain by R-ML receiver.

# Reference