3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2304405

Online, April 17 - April 26, 2023

Source:	China Telecom

Title:	Discussion on UE configured with two serving cells and each cell with one SUL band

Agenda Item:	8.2

Document for:	Discussion

- Introduction

The following proposal was endorsed in RAN #99. The contribution provides our views on the RAN task.

- UL/SUL indicator field is excluded from a DCI format 0_X.

- Task RAN4 to assess the additional, if any, RAN4 specification impact and UE implementation impact for a UE configured with two serving cells, each with SUL; report to RAN#100 with the goal of striving for potential normative work supporting the case where a UE is configured with two serving cells, each with SUL

  - E.g., whether back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier could be supported without any switching period, or

  - E.g., whether it is only feasible to support such configuration in the UL Tx switching framework with UE capability based switching period

  - Example band combinations are referred to in RP-223553 (RP-230719)

  - Further check the status in RAN#100

- Discussion

Firstly, it is known that the new Rel-18 WID on NR CA band combinations with two SUL cells was approved in RAN #98, and the first big CR for the WI was agreed in RAN4 #106 [1]. So, it is clear that the band configurations with two serving cells and each cell with one SUL band have already been supported from RAN4 perspective.

Observation 1: With the big CR for NR CA band combinations with two SUL cells agreed in RAN4 #106, it is clear that the band configurations with two serving cells and each cell with one SUL band have already been supported from RAN4 perspective.

Secondly, regarding the first sub-bullet from the RAN task, to support the back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier without any switching period, three additional uplink configurations need to be added on top of the RAN4 agreed CR. Taking CA_n41A-n95A_n79A-n98A as example, the three UL CA configurations in red need to be added in RAN4 specification.

| SUL band combination with CA | Uplink CA configuration or SUL configuration |
| --- | --- |
| CA_n41A-n95A_n79A-n98A | SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A CA_n41A-n98A CA_n95A-n79A CA_n95A-n98A |

Considering the UE implementation, we don’t see any UE implementation issue since the number of bands in uplink configuration is still 2.

Proposal 1: To support the back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier without any switching period,

- From RAN4 specification perspective, three additional uplink configurations need to be added on top of the RAN4 agreed CR. Taking CA_n41A-n95A_n79A-n98A as example, the three UL CA configurations in red need to be added.

| SUL band combination with CA | Uplink CA configuration or SUL configuration |
| --- | --- |
| CA_n41A-n95A_n79A-n98A | SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A CA_n41A-n98A CA_n95A-n79A CA_n95A-n98A |

- From UE implementation perspective, there in no issue since the number of bands in uplink configuration is still 2.

Thirdly, regarding the second sub-bullet from the RAN task, we think it is feasible to support such configuration in the Rel-18 UL Tx switching framework with UE capability based switching period, and without any additional RAN4 specification impact.

Considering the UE implementation, we don’t see any additional UE implementation issue as well.

Proposal 2: It is feasible to support such configuration in the Rel-18 UL Tx switching framework with UE capability based switching period, and without any additional RAN4 specification impact. In additional, there is no UE implementation issue.

- Conclusion

The contribution discussed the band configuration with two serving cells and each cell with one SUL band, with the following observation and proposals:

Observation 1: With the big CR for NR CA band combinations with two SUL cells agreed in RAN4 #106, it is clear that the band configurations with two serving cells and each cell with one SUL band have already been supported from RAN4 perspective.

Proposal 1: To support the back-to-back transmissions between two SUL carriers and back-to-back transmissions between SUL carrier and non-corresponding NUL carrier without any switching period,

- From RAN4 specification perspective, three additional uplink configurations need to be added on top of the RAN4 agreed CR. Taking CA_n41A-n95A_n79A-n98A as example, the three UL CA configurations in red need to be added.

| SUL band combination with CA | Uplink CA configuration or SUL configuration |
| --- | --- |
| CA_n41A-n95A_n79A-n98A | SUL_n41A-n95A SUL_n79A-n98A CA_n41A-n79A CA_n41A-n98A CA_n95A-n79A CA_n95A-n98A |

- From UE implementation perspective, there in no issue since the number of bands in uplink configuration is still 2.

Proposal 2: It is feasible to support such configuration in the Rel-18 UL Tx switching framework with UE capability based switching period, and without any additional RAN4 specification impact. In additional, there is no UE implementation issue.

- References

- R4-2300811, Big CR for NR CA band combinations with two SUL cells, RAN4 #106, CMCC, Feb 2023.