3GPP TSG-RAN4 Meeting #106bis-e	                                                            R4-2304427

Online, April 17 – April 26, 2023

Title: 	Further response on enhanced cell reselection requirements in NTN

Source: 	CATT

Agenda item:	7.1.2

Document for:	Approval

- Introduction

In RAN4#105 meeting, RAN4 sent LS [1] to RAN2 to clarify the capability description for enhanced cell reselection requirements in NTN. And in this meeting, we received the response from RAN2 [2] about the modifications made in RAN2 spec and asking for further check and clarification on the requirements in RRC_INACTIVE state from RAN4.

This document discuss the response LS from RAN2 and present our understandings and proposals based on which a draft LS is provided in Annex.

- Discussion

The LS [1] clarified to RAN2 that when UE doesn’t support enhanced cell reselection requirements, the non-HST NTN requirements should apply. Based on the LS, RAN2 has agreed the CR to correct the description as follows and ask RAN4 to check if the modification is correct.

Enhanced RRM requirements for measurements in IDLE and INACTIVE modes It is optional for UE to support enhanced RRM requirements for measurements for NTN bands (FR1 only and FDD only) in RRC_IDLE/RRC_INACTIVE as specified in TS 38.133 [5]. If UE does not support this feature, otherNTN measurement requirements (as specified in TS 38.133 [5], section 4.2C.2) are applied for both LEO and GEO.

We understand this modification is correct and can reflect RAN4’s request since the indicated requirements in section 4.2C.2 mean the non-HST NTN requirements. And based on this correction, the FG 25-4 in the feature list needs to be updated to align with the agreed CR as pointed by RAN2.

Proposal 1: RAN2 CR on the capability description for enhanced cell reselection requirements in NTN is correct.

Proposal 2: Update FG 25-4 in the feature list to align with the RAN2 CR.

In addition, although the UE capability for enhanced cell reselection requirements is defined for IDLE/INACTIVE state, there are no requirements defined for RRC_INACTIVE state in RAN4. So RAN2 ask RAN4 to clarify whether the requirements also apply to RRC_INACTIVE state. In our understanding, the same requirements should apply to RRC_INACTIVE state and RAN4 can introduce the requirements for RRC_INACTIVE state in R17 TEI stage.

Proposal 3: The same cell reselection requirements apply to RRC_INACTIVE state in NTN and RAN4 to introduce the requirements in R17 TEI.

Based on the discussion and proposals above, we provide a draft LS to RAN2 in Annex.

- Conclusion

This document discussed the response LS from RAN2 on enhanced cell reselection requirements in NTN and presented the following proposals:

Proposal 1: RAN2 CR on the capability description for enhanced cell reselection requirements in NTN is correct.

Proposal 2: Update FG 25-4 in the feature list to align with the RAN2 CR.

Proposal 3: The same cell reselection requirements apply to RRC_INACTIVE state in NTN and RAN4 to introduce the requirements in R17 TEI.

- Reference

- R4-2220427, LS on capability description for enhanced cell reselection requirements in NTN, Nokia

- R4-2304012 (R2-2301966), Response LS on enhanced cell reselection requirements in NTN, Nokia

Appendix Draft LS to RAN2

3GPP TSG-RAN4 Meeting #106bis-e	                                                            R4-23XXXX

Online, April 17 – April 26, 2023

Title:	Further response on enhanced cell reselection requirements in NTN

Response to:	R4-2304012 (R2-2301966)

Release:	Rel-17

Work Item:	NR_NTN_Solutions_Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name:                  Qiuge Guo

E-mail Address:   guoqiuge@CATT.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 thanks RAN2 for the response LS (R2-2301966) on enhanced cell reselection requirements in NTN and the CR. RAN4 has discussed the issues in the response LS and has the following agreements.

- RAN2 CR on the capability description for enhanced cell reselection requirements in NTN is correct. - RAN4 updated the FG 25-4 in the feature list to align with the RAN2 CR. - The same cell reselection requirements apply to RRC_INACTIVE state in NTN and RAN4 will introduce the requirements in R17 TEI.

2. Actions:

To RAN WG2:

RAN4 kindly asks RAN2 to take the above analysis and recommendation into account.

3. Date of Next TSG-RAN Meetings:

TSG RAN WG4 Meeting #107		May 22 – May 26, 2023		        Incheon, KR

TSG RAN WG4 Meeting #108		August 21 – August 25, 2023		Toulouse	, FR