3GPP TSG-RAN WG4 Meeting # 106bis-e                                                     R4-2304433			R4-22xxxxx

# Online, April 17 – April 26, 2023

Title: 	Analysis on Local area BS feasibility for duplex evolution

Source: 	CATT

Agenda item:	5.20.2.2.1

Document for:	Discussion

- Background

This contribution provides our analysis for the self-interference of local area BS. The analysis is conducted using FR1 as the example; FR2 can be analyzed using the similar methodology when the assumptions are fixed.

- Discussion

  - The whole SI contribution

For Local Area BS, the SI analysis assumptions are summarized in table1.

Table 1: SI analysis assumptions for Local area BS

| Parameters | Local Area BS |
| --- | --- |
| Reference sensitivity level degradation due to SI | 1dB |
| Channel bandwidth | 100MHz |
| Subband configuration | {DUD} |
| DL subband width BWDL | 40MHz*2 |
| UL subband width BWUL | 20MHz |
| Tx output power over whole channel PTx | 24dBm |
| Adjacent subband Leakage Power Ratio | 45dB |
| Noise Figure | 13dB |

Reference sensitivity level degradation due to SI is 1dB, interfering signal mean power can be calculated as in table 2.

Table 2 : Interfering signal mean power of receiver

| Parameters | Value |
| --- | --- |
| Rx subband width BWUL (MHZ) | 20 |
| Noise figure NF(dB) | 13 |
| Noise floor Pnoise floor (dBm/ BWUL) (Note1) | -88.0 |
| Reference sensitivity level degradation Drsl (dB) | 1 |
| Receiver total noise power Ptotal noise (dBm/ BWUL) (Note2) | -87.0 |
| UL subband total interfering signal mean power Ptotal interference (dBm/ BWUL) (Note3) | -93.8 |
| Note1: Pnoise floor= -174dBm/Hz+10*log10(BWUL*106) +NF Note2: Ptotal noise = Pnoise floor+ Drsl Note3: Ptotal interference= Ptotal noise - Pnoise floor |  |

From above calculation, it can be seen that the if 1 dB sensitivity degradation is the target, the total interferences signal mean power is -93.8dBm. The total interferences of SI include Tx leakage and Rx path contribution, then each contribution is assumed to be -96.8dBm in the following separate analysis.

Observation 1: The Tx leakage and Rx contribution requirements to the SI can be divided to -96.8dBm each if each of them contributes to half of noise.

  - Tx leakage

Assuming Tx leakage contribution is -96.8dBm.

Table 3: RSIC requirement for Tx leakage of SI

| Parameters | Value |
| --- | --- |
| Tx subband BWDL (MHZ) | 40 |
| Rx subband BWUL (MHZ) | 20 |
| Tx out power over whole channel PTx（dBm) | 24 |
| Tx leakage contribution Pinterference Tx leakage (dBm/ BWUL) | -96.8 |
| Overall RSIC requirement for Tx leakage RSICTx leakage (dB) (Note1) | 117.8 |
| Note1: RSICTx leakage=PTx-10*log10(BWDL/BWUL)-Pinterference Tx leakage |  |

Observation 2: The overall RSIC requirement for Tx leakage should be at least 117.8 dB for local area SBFD BS.

  - Rx path contribution

Assuming Rx path contribution is -96.8dBm.

- Rx blocking in Tx subband

If assume Rx blocking in Tx subband is -35dBm for Local Area BS, RSIC requirement, ACS and Rx intermodulation requirements are analysed as follows.

Table 4 : RSIC requirement for Rx blocking in Tx subband of SI

| Parameters | Value |
| --- | --- |
| Tx subband BWDL (MHZ) | 40 |
| Rx subband BWUL (MHZ) | 20 |
| Tx out power over whole channel PTx（dBm) | 24 |
| Rx blocking in Tx subband PRBTxSB (dBm) | -35 |
| RSIC requirement for Rx blocking in Tx subband issue RSICRBTxSB (dB) (Note1) | 59 |
| Note1: RSICRBTxSB =PTx- PRBTxSB |  |

- Adjacent Channel Selectivity (ACS)

Assuming ACS contributes half of the Rx path contribution, then ACS requirement should be 64.8dB.

ACS= PRBTxSB - (Pinterference Rx path-3dB)

Pinterference Rx path is Rx path contribution, i.e. -96.8dBm.

Observation 3: If Rx blocking in Tx subband is -35dBm, ACS requirement should be 64.8dB, which is very challenging.

- Rx intermodulation

Assuming Rx intermodulation contribution is half of the Rx path contribution, i.e. -99.8dBm，if Rx blocking in Tx subband is -35dBm, IM3 should be below -99.8 dBm,the required IIP3 is -2.6dBm.

PIM3= Pinterference Rx path-3dB

IIP3= PRBTxSB +( PRBTxSB -PIM3)/2

Observation 4: If IM3 contribution is -99.8dBm for local area BS SBFD Rx noise, IIP3 should be at least -2.6 dBm, which is very challenging.

If Rx blocking in Tx subband is -35dBm, ACS and IIP3 requirements are very challenging. Refer to TS 38.104, assume ACS and Rx intermodulation interference signals power are both -44dBm:

- ACS: 55.8dB

- IIP3: -16.0dBm

- RSIC requirement for Rx blocking in Tx subband: 68dB

Observation 5: If Rx blocking in Tx subband is -44dBm, ACS requirement should be 55.8dB.

Observation 6: If Rx blocking in Tx subband is -44dBm, IIP3 should be -16dBm for Local area BS with SBFD capability.

Observation 7: If Rx blocking in Tx subband is -44dBm, RSIC requirement should be 68dB for Local area BS with SBFD capability.

  - Analysis using the framework

Table 5 is the analysis using the RSIC analysis framework [2].

Table 5 : Solutions to Tx leakage and Rx path issues of SI

| Item |  | Parameters and calculations | example |
| --- | --- | --- | --- |
| General assumption |  | BS total Tx power @ DL SB (dBm) ① | 24 |
|  |  | SBFD configuration | DUD |
|  |  | UL subband width (MHz) ② | 20 |
|  |  | DL subband width (MHz) ③ (Note1) | 40 |
|  |  | Guard band assumption |  |
|  |  | BS NF (dB) ④ | 13 |
| Tx leakage contribution |  | Tx Frequency isolation capability (dB) ⑤ | 45 |
|  |  | DL subband RF filter suppression capability (dB) ⑥ | 0 |
|  |  | Antenna isolation capability (dB) ⑦ | 70 |
|  |  | Tx Beam nulling/isolation capability (dB) ⑧ | NA |
|  |  | DL EIRP impact due to beam nulling in TX sub-band | —— |
|  |  | Digital IC for Tx leakage capability (dB) ⑨ | 3 |
|  |  | Overall RSIC for Tx leakage (dB) ⑩ |  |
|  |  | ⑩=⑤+⑥+⑦+⑨ | 118 |
|  |  | Tx leakage contribution to SI (dBm/UL subband) ⑪ |  |
|  |  | ⑪=①-10*log10(③②)-⑩ | -97.01 |
| Rx path noise contribution | Blocking analysis | Rx beam nulling/isolation capability (dB) ⑫ | NA |
|  |  | RX sensitivity degradation caused by RX beam nulling | —— |
|  |  | UL subband RF filter suppression capability (dB) ⑬ | 0 |
|  |  | Overall RSIC for blocking capability (dB) ⑭ |  |
|  |  | ⑭=⑦+⑬ | 70 |
|  |  | Tx SB interference signal level at Rx path (dBm) ⑮ |  |
|  |  | ⑮=①-⑭ | -46 |
|  | Rx IMD | Rx IIP3 capability (dBm) ⑯ | -16 |
|  |  | Rx IM3 contribution (dBm) ⑰ |  |
|  |  | ⑰=⑮-2*(⑯-⑮) | -106 |
|  | Rx ACS | ACS capability (dB) ⑱ | 55 |
|  |  | Rx ACS contribution (dBm) ⑲ |  |
|  |  | ⑲=⑮-⑱ | -101 |
|  |  | Any other RX impacts if significant (dBm) ⑳(Note 2) | -127 |
|  | Rx path overall noise contribution (dBm) ㉑ |  |  |
|  | ㉑=10*log10(10⑰10+10⑲10+10⑳10) |  | -99.80 |
| Total SI contribution (dBm) ㉒ |  |  |  |
| ㉒=10*log10(10⑪10+10㉑10) |  |  | -95.17 |
| Noise floor (dBm/UL subband) ㉓ |  |  |  |
| ㉓=-174+10*log10②*106+④ |  |  | -87.99 |
| Overall REFSENS degradation (dB) ㉔ |  |  |  |
| ㉔=10*log1010㉒10+10㉓10-㉓ |  |  | 0.76 |
| Note 1：③ is single DL subband width |  |  |  |
| Note 2: If ⑳ is very lower than ⑰ and ⑲, the value of it inserted in the table must be at least 10dB lower than ⑰ and ⑲ |  |  |  |

Observation 8: From the analysis in table5, the following capabilities are needed,

overall RSIC for Tx leakage capability is 118dB

overall RSIC for blocking capability is 70dB

ACS is 55dBc

IIP3 is -16dBm

The capabilities in observation 8 are improved compared with legacy LA BS, but it’s feasible from implementation point of view.

- Summary

In this contribution, we provide the analysis for LA BS SI. We have the following observations.

Observation 1: The Tx leakage and Rx contribution requirements to the SI can be divided to -96.8dBm each if each of them contributes to half of noise.

Observation 2: The overall RSIC requirement for Tx leakage should be at least 117.8 dB for Local area SBFD BS.

Observation 3: If Rx blocking in Tx subband is -35dBm, ACS requirement should be 64.8dB, which is very challenging.

Observation 4: If IM3 contribution is -99.8dBm for Local area BS SBFD Rx noise, IIP3 should be at least -2.6 dBm, which is very challenging.

Observation 5: If Rx blocking in Tx subband is -44dBm, ACS requirement should be 55.8dB.

Observation 6: If Rx blocking in Tx subband is -44dBm, IIP3 should be -16dBm for Local area BS with SBFD capability.

Observation 7: If Rx blocking in Tx subband is -44dBm, RSIC requirement should be 68dB for Local area BS with SBFD capability.

Observation 8: From the analysis in table5, the following capabilities are needed,

overall RSIC for Tx leakage capability is 118dB

overall RSIC for blocking capability is 70dB

ACS is 55dBc

IIP3 is -16dBm

The capabilities in observation 8 are improved compared with legacy LA BS, but it’s feasible from implementation point of view.

Reference

[1]R4-2300545, “Further discussion on BS feasibility study and impact on RF requirements for duplex evolution”, RAN4#106