3GPP TSG-RAN WG4 Meeting # 106bis-e                                                     R4- 2304434			R4-22xxxxx

# Online, April 17 – April 26, 2023

Title: 	SBFD adjacent channel co-existence simulation results

Source: 	CATT

Agenda item:	5.20.2.1

Document for:	Discussion

- Background

This contribution provides the SBFD adjacent channel co-existence simulation results according to the update of the simulation assumption in last RAN4 meeting [3].

- Discussion

  - SBFD adjacent channel co-existence simulation scenarios

SBFD adjacent channel co-existence simulation calibration scenarios are summarized in table 1, SBFD subband configuration is {DU}.

Table 1: SBFD adjacent channel co-existence simulation calibration scenarios

| FR1 SBFD Urban Macro UL | Victim: SBFD (DU) SINR baseline: No system in adjacent channel SINR with ACI: NR TDD DL in adjacent channel Priority: High |
| --- | --- |
| FR1 SBFD Urban Macro DL | Victim: SBFD (DU) SINR baseline: No system in adjacent channel SINR with ACI: NR TDD DL in adjacent channel Priority: High |
| FR1 legacy TDD Urban Macro UL | Victim: NR TDD UL SINR baseline: NR TDD UL in adjacent channel SINR with ACI: SBFD (DU) in adjacent channel Priority: Low |
| FR1 legacy TDD Urban Macro DL | Victim: NR TDD DL SINR baseline: NR TDD DL in adjacent channel SINR with ACI: SBFD (DU) in adjacent channel Priority: High |
| FR2 SBFD Urban Macro UL | Victim: SBFD (DU) SINR baseline: No system in adjacent channel SINR with ACI: NR TDD DL in adjacent channel Priority: High |
| FR2 SBFD Urban Macro DL | Victim: SBFD (DU) SINR baseline: No system in adjacent channel SINR with ACI: NR TDD DL in adjacent channel Priority: High |
| FR2 legacy TDD Urban Macro UL | Victim: NR TDD UL SINR baseline: NR TDD UL in adjacent channel SINR with ACI: SBFD (DU) in adjacent channel Priority: Low |
| FR2 legacy TDD Urban Macro DL | Victim: NR TDD DL SINR baseline: NR TDD DL in adjacent channel SINR with ACI: SBFD (DU) in adjacent channel Priority: High |

  - FR1

2.2.1 FR1 SBFD Urban Macro UL

Table 2: FR1 SBFD Urban Macro UL SINR and Throughput

| FR1 SBFD Urban Macro UL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | -10.18 | 12.25 | 0.00 | 1.66 |
| Baseline + ACI | -12.29 | 11.75 | 0.00 | 1.60 |
| ((Baseline + ACI) - Baseline)/Baseline | __ | __ |  | 0.0381 |

Figure1: FR1 SBFD Urban Macro UL SINR and Throughput

The simulation results show 50% throughput degrades3.81%, 5% throughput of baseline and “baseline+ACI” are both 0.

2.2.2 FR1 SBFD Urban Macro DL

Table 3: FR1 SBFD Urban Macro DL SINR and Throughput

| FR1 SBFD Urban Macro DL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | 8.05 | 24.14 | 1.73 | 4.81 |
| Baseline + ACI | 6.72 | 23.67 | 1.51 | 4.72 |
| ((Baseline + ACI) - Baseline)/Baseline | —— | —— | 0.1292 | 0.0190 |

Figure2: FR1 SBFD Urban Macro DL SINR and Throughput

The simulation results show 50% throughput degrades 1.90%,  5% throughput degrades 12.92%.

2.2.3 FR1 legacy TDD Urban Macro UL

Table 4: FR1 legacy TDD Urban Macro UL SINR and Throughput

| FR1 legacy TDD Urban Macro UL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | -9.63 | 12.61 | 0.06 | 1.71 |
| Baseline + ACI | -10.71 | 12.18 | 0.00 | 1.65 |
| ((Baseline + ACI) - Baseline)/Baseline | 0.1124 | 0.0337 | 1.0000 | 0.0313 |

Figure3: FR1 legacy TDD Urban Macro UL SINR and Throughput

The simulation results show 50% throughput degrades 3.13%,  5% throughput degrades 100%.

2.2.4 FR1 legacy TDD Urban Macro DL

Table 5: FR1 legacy TDD Urban Macro DL SINR and Throughput

| FR1 legacy TDD Urban Macro DL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | 5.81 | 22.58 | 1.36 | 4.50 |
| Baseline + ACI | 4.13 | 22.01 | 1.11 | 4.39 |
| ((Baseline + ACI) - Baseline)/Baseline | —— | —— | 0.1866 | 0.0249 |

Figure4: FR1 legacy TDD Urban Macro DL SINR and Throughput

The simulation results show 50% throughput degrades 2.49%,  5% throughput degrades18.66%.

  - FR2

2.3.1 FR2 SBFD Urban Macro UL

Table 6: FR2 SBFD Urban Macro UL SINR and Throughput

| FR2 SBFD Urban Macro UL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | -7.38 | 12.15 | 0.10 | 1.65 |
| Baseline + ACI | -7.86 | 11.65 | 0.09 | 1.59 |
| ((Baseline + ACI) - Baseline)/Baseline | 0.0657 | 0.0411 | 0.0980 | 0.0378 |

Figure5: FR2 SBFD Urban Macro UL SINR and Throughput

The simulation results show 50% throughput degrades 3.78%,  5% throughput degrades9.80%.

2.3.2 FR2 SBFD Urban Macro DL

Table 7: FR2 SBFD Urban Macro DL SINR and Throughput

| FR2 SBFD Urban Macro DL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | 10.71 | 25.34 | 2.21 | 5.05 |
| Baseline + ACI | 9.87 | 24.52 | 2.05 | 4.89 |
| ((Baseline + ACI) - Baseline)/Baseline | 0.0788 | 0.0324 | 0.0697 | 0.0323 |

Figure6: FR2 SBFD Urban Macro DL SINR and Throughput

The simulation results show 50% throughput degrades 3.23%,  5% throughput degrades 6.97%.

2.3.3 FR2 legacy TDD Urban Macro UL

Table 8: FR2 legacy TDD Urban Macro UL SINR and Throughput

| FR2 legacy TDD Urban Macro UL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | -9.64 | 11.90 | 0.06 | 1.62 |
| Baseline + ACI | -9.88 | 11.76 | 0.06 | 1.60 |
| ((Baseline + ACI) - Baseline)/Baseline | —— | —— | 0.0521 | 0.0113 |

Figure7: FR2 legacy TDD Urban Macro UL SINR and Throughput

The simulation results show 50% throughput degrades 1.13%,  5% throughput degrades5.21%.

2.3.4 FR2 legacy TDD Urban Macro DL

Table 9: FR2 legacy TDD Urban Macro DL SINR and Throughput

| FR2 legacy TDD Urban Macro DL |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5% SINR | 50% SINR | 5% thr (bps/Hz) | 50% thr (bps/Hz) |
| Baseline | 10.46 | 25.69 | 2.16 | 5.12 |
| Baseline + ACI | 9.66 | 24.88 | 2.02 | 4.96 |
| ((Baseline + ACI) - Baseline)/Baseline | —— | —— | 0.0665 | 0.0314 |

Figure8: FR2 legacy TDD Urban Macro DL SINR and Throughput

The simulation results show 50% throughput degrades 3.14%,  5% throughput degrades 6.65%.

- Conclusion

Table 10: SBFD adjacent channel co-existence simulation results

|  | 5% throughput degradation (%) | 50% throughput degradation (%) |
| --- | --- | --- |
| FR1 SBFD Urban Macro UL |  | 3.81 |
| FR1 SBFD Urban Macro DL | 12.92 | 1.90 |
| FR1 legacy TDD Urban Macro UL | 100.00 | 3.13 |
| FR1 legacy TDD Urban Macro DL | 18.66 | 2.49 |
| FR2 SBFD Urban Macro UL | 9.80 | 3.78 |
| FR2 SBFD Urban Macro DL | 6.97 | 3.23 |
| FR2 legacy TDD Urban Macro UL | 5.21 | 1.13 |
| FR2 legacy TDD Urban Macro DL | 6.65 | 3.14 |

This contribution summarizes the simulation results from our company. The simulation results show 50% throughput degradation is within the 5% evaluation criteria. 5% throughput of baseline and “baseline+ACI” are both 0 in FR1 SBFD Urban Macro UL scenario.

Reference

[1] R4-2217466, “WF for adjacent channel co-existence evaluation of SBFD operation”,  RAN4#104-bis-e

[2] R4-2220247, “Simulation assumptions for SBFD adjacent channel co-existence study”,  RAN4#105

[3] R4-2302888, “WF for SBFD adjacent channel co-existence study”, RAN4#106