3GPP TSG-RAN WG4 #106bis-e	R4-2304455

Electronic Meeting, April 17th – 26th, 2023

Agenda item:	5.4.2.1

Source: 	Qualcomm Incorporated

Title: 	Considerations on CA_n5-n105

Document for:	Approval

# Introduction

Considerations on CA_n5-n105 are provided in this contribution.

# Discussion

## The objectives of SI

The objectives of SI are as follows:

- Investigate the feasibility and solutions to enable simultaneous transmission on two UL bands with 2Tx and simultaneous reception on two or three bands for the following sub-1GHz band combinations for smart phone form factor

Table 1: Summary of band combinations considered in the SI

| Configuration | Uplink Configuration | fallback configurations (Status) | Supported operators |
| --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | DL_n5A-n105A_UL_n5A_BCS0 (New) DL_n5A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n28A-n105A | CA_n28A-n105A | DL_n28A-n105A_UL_n28A_BCS0 (New) DL_n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n26A-n28A | CA_n26A-n28A | DL_n26A-n28A_UL_n28A_BCS0 (Completed in RAN4#106) DL_n26A-n28A_UL_n26A_BCS0 (New) | Telstra |
| CA_n5A-n28A-n105A1 | CA_n5A-n28A | DL_n5A-n28A_UL_n5A-n28A_BCS0 (Ongoing in a separate WI) DL_n5A-n28A-n105A_UL_n5A_BCS0 (New) DL_n5A-n28A-n105A_UL_n28A_BCS0 (New) | Spark NZ |
| CA_n5A-n28A-n105A1 | CA_n5A-n105A | DL_n5A-n105A_UL_n5A-n105A_BCS0 (New) DL_n5A-n28A-n105A_UL_n5A_BCS0 (New) DL_n5A-n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |
| CA_n5A-n28A-n105A1 | CA_n28A-n105A | DL_n28A-n105A_UL_n28A-n105A_BCS0 (New) DL_n5A-n28A-n105A_UL_n28A_BCS0 (New) DL_n5A-n28A-n105A_UL_n105A_BCS0 (New) | Spark NZ |

Table 2: Bandwidth combination set for each band combination

| - NR CA configuration | - Uplink CA configuration or single uplink carrier | - NR Band | - Channel bandwidth (MHz) | - Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n105A | CA_n5A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n28A-n105A | CA_n28A-n105A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |
| CA_n26A-n28A | CA_n26A-n28A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
| CA_n5A-n28A-n105A1 | CA_n5A-n28A CA_n5A-n105A CA_n28A-n105A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

  - The following aspects need be studied

    - UE RF architecture including n-plexing, PA

    - Study feasibility of low band wideband antenna and multiple low band narrowband antennas.

    - Evaluate the MSD requirements if needed.

    - Evaluate if UE complexity can be reduced based on the frequency range and BCS proposed by operators.

  - Power class 3 (PC3) is considered in this study

- Identify potential impacts to relevant RAN4 requirements.

NOTE 1: The study of 3 band combination can only start after completion of 2 bands fallbacks

NOTE 2: Check at RAN4#107 whether CA_n5A-n28A-n105A could be included in the SI based on study progress for the fall back combinations listed in Table 1.

## RF architecture considerations

The frequency ranges for the superset of this CA combination under study is shown below for illustration.

Figure 1 CA_n5A-n28A-n105A bands and frequency ranges

n105 and n5 have certainly enough frequency separation to be implemented using a quadplexer

Figure 2 Quadplexer implementation for CA_n5-n105 with two antennas

Problems with this architecture are related to antenna BW rations and the fact that both UL’s in UL CA would be transmitted from same antenna.

| Antenna | F_Low | F_High | BW Ratio |
| --- | --- | --- | --- |
| Main TRX | 612 | 894 | 37.5% |
| Diversity TRX | 612 | 894 | 37.5% |

Figure 3 Antenna BW Ratio's for Quadplexer solution with two antennas

Looking at figure 3, the antenna BW ratios are extremely high, much higher than in CA_n28-n105

Observation 1: The antenna BW ratios are so high in Quadplexer solution with two antennas that the feasibility should be justified before making requirements

Another option to implement two antenna solution is to use two triplexers.

Figure 4 Two-Triplexer solution for CA_n5-n105 with two antennas

Benefit of this is that the UL’s can be transmitted from different antennas, helping to optimise the TRP for each UL. However, similar to the Quadplexer case above, the antenna BW’s ratios are extremely high.

| Antenna | F_Low | F_High | BW Ratio |
| --- | --- | --- | --- |
| Main TRX | 612 | 894 | 37.5% |
| Diversity TRX | 612 | 894 | 37.5% |

Figure 5 Antenna BW Ratio's for Two-Triplexer solution with two antennas

Observation 2: The antenna BW ratios are so high in Two-Triplexer solution with two antennas that the feasibility should be justified before making requirements

Three-antenna solution would consist of n105 TRX, n5 TRX, and n5-n105 DRX each in own antenna.

Figure 6 Three-antenna solution for CA_n5-n105

Observation 3: Three-antenna solution is one option for CA_n5A-n105 RFFE Architecture

Again, the UL’s would be transmitted from different antennas. The antenna BW ratios in TX capable antennas would be naturally equal to the antennas used in devices today as Main TRX and Diversity TRX have only one band. The antenna BW ration in DRX antenna is still very high, potentially degrading OTA performance to some extent.

| Antenna | F_Low | F_High | BW Ratio |
| --- | --- | --- | --- |
| Main TRX | 612 | 703 | 13.8% |
| Diversity TRX | 824 | 894 | 8.1% |
| Diversity RX | 612 | 894 | 37.5% |

Figure 7 Antenna BW Ratios for Three-antenna

Three-antenna solution seems easiest solution from antenna BW ratio perspective if 3 LB antennas are supported by the device.

Observation 4: The antenna BW ratios for TRX antennas are normal, only the DRX antenna has extremely high BW Ratio with three antennas

## Intermodulation aspects

This combination does not have any IMD’s to own 2UL72DL bands.

Observation 5: CA_n5-n105 does not have any IMD’s in 2UL/2DL CA

# Conclusions

Initial considerations of CA_n5-n105 was provided with the following observations

Observation 1: The antenna BW ratios are so high in Quadplexer solution with two antennas that the feasibility should be justified before making requirements

Observation 2: The antenna BW ratios are so high in Two-Triplexer solution with two antennas that the feasibility should be justified before making requirements

Observation 3: Three-antenna solution is one option for CA_n5A-n105 RFFE Architecture

Observation 4: The antenna BW ratios for TRX antennas are normal, only the DRX antenna has extremely high BW Ratio with three antennas

Observation 5: CA_n5-n105 does not have any IMD’s in 2UL/2DL CA

# Reference

- RP-230789, “Study on enhancement for sub-1GHz NR band combinations”, Spark NZ, Huawei, HiSilicon