3GPP TSG-RAN WG4 Meeting #106-bis-e					R4-2304472

Online, February 27 – March 3, 2023

Source: 	China Unicom

Title:	Discussion on reference sensitivity degradation requirements for FDD PC2 HPUE

Agenda Item:	4.22.2

Document for:	Discussion

1	Introduction

In RAN4-106 meeting, WF on FDD PC2 HPUE [1] was agreed to further discuss reference sensitivity degradation requirements based on companies contributions. This paper collected some analyses from [1], targeting at facilitating the discussion for finalizing RSD requirements for FDD PC2 HPUE.

2	Discussion

Taking RSD values for n8 PC2 as an example;

For 1Tx architecture:

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | Skyworks(R4-2300652) | 0.3 | 0.4 | 0.7 | 1.9 |  |  | 2.6 |  |  |  |
|  | Apple(R4-2300362) | 0 | 1.2 | 0.9 | 1.9 | N/A | N/A | 4.4 | N/A | N/A | N/A |
|  | Huawei, HiSilicon(R4- 2300715) | 1.2 | 1.2 | 1.4 | 3.3 | - | - | 3.8 | - | - | - |
|  | Murata(R4-2302031) | 0.8 | 0.8 | 0.5 | 4.9 | - | - | 6.8 | - | - | - |
|  | Qualcomm(R4-2302709) | 0.8 | 0.6 | 0.7 | 1.8 | - | - | 1.8 |  |  |  |
| Average |  | 0.6 | 0.8 | 0.8 | 2.8 |  |  | 3.9 |  |  |  |

For 2Tx architecture:

| Operating Band | Source | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | Skyworks(R4-2300652) | 0.8 | 1.0 | 1.5 | 5.1 |  |  | 6.6 |  |  |  |
|  | Apple(R4-2300362) | 0 | 0 | 0.9 | 6.2 | N/A | N/A | 8.1 | N/A | N/A | N/A |
|  | Huawei, HiSilicon(R4- 2300715) | 3.1 | 3.2 | 3.8 | 6.2 | - | - | 7.1 | - | - | - |
|  | Murata(R4-2302031) | 1.3 | 1.3 | 1.4 | 5.5 | - | - | 7.7 | - | - | - |
|  | Qualcomm(R4-2302709) | 1.5 | 1.5 | 2.2 | 6.5 | - | - | 5.9 |  |  |  |
|  | MTK (R4-2219869) | 0 | 0 | 1 | 5.4 | - | - | 7.2 | - | - | - |
|  | ZTE, CU (R4-2215893) | 0.5 | 0.6 | 1.2 | 5.0 | - | - | 6.7 | - | - | - |
| Average |  | 1.0 | 1.1 | 1.7 | 5.7 |  |  | 7.0 |  |  |  |

It can be observed that the RSD values for smaller channel bandwidth (i.e. for <=20MHz for n8) are closer according to companies analyses, but the difference between values for 35MHz bandwidth is larger, and comments were made that it might not be convincing to calculate the average values for this case. Similar issues occur for HPUE for CA with PC2 on FDD carrier.

Observation: According to analyses from previous meetings for certain FDD bands, the RSD values for smaller channel bandwidth (e.g. for <=20MHz for n8) are closer, but the difference between values for higher bandwidth is larger.

Proposal: To figure out the reasons for large value differences, and further discuss in this meeting on how to proceed to finalize RSD values for FDD PC2 bands, as well as MSD for HPUE for CA with PC2 on FDD carrier.

3	Conclusion

In this paper, the background for RSD evaluations for FDD PC2 HPUE was discussed. And the following observation and proposal are made:

Observation: According to analyses from previous meetings for certain FDD bands, the RSD values for smaller channel bandwidth (e.g. for <=20MHz for n8) are closer, but the difference between values for higher bandwidth is larger.

Proposal: To figure out the reasons for large value differences, and further discuss in this meeting on how to proceed to finalize RSD values for FDD PC2 bands, as well as MSD for HPUE for CA with PC2 on FDD carrier.

References

- R4-2303458 WF on FDD PC2 HPUE, RAN4#106	China Unicom, Skyworks, Huawei, HiSilicon