3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304491

# Electronic Meeting, April 17 – April 26, 2023

Title: 	Discussion on new IoT NTN FDD band (L+S band) UE RF requirements

Agenda Item:	6.6.3

Source: 	MediaTek Inc.

Document for:	Approval

Introduction

Regarding the new approved WI of “New spectrum WID: Introduction of new IoT NTN FDD band (L+S band)” in RAN-Plenary#99 meeting, to define the UE RF TX and RX requirements for the new IoT NTN band (L+S band) is needed. In the contribution, the consideration for UE RF TX and RX requirements are provided.

RF requirements

- 2.1	General view for band agnostic UE RF requirements

The new IoT NTN FDD band (L+S band) spectrum WI [1] was approved.

First, regarding UE RF, the objective of the WI core part is show below for reference.

- Specify the following new FDD frequency bands for NB-IoT/eMTC NTN operation:

  - 1610 – 1626.5 MHz in UL, and 2483.5 – 2500 MHz in DL

- Support UE power class 3 (+23dBm) and power class 5 (+20dBm)

This band will be Release-independent from Rel-17 for the UE.

For the eMTC/NB-IoT UE RF requirements, we distinguish band agnostic requirements from TS 36.102. In the table below, we provide comments on reusing these requirements if possible.

Table 2.1-1

| Requirement name | Band-agnostic? | Reuse 36.102 requirement? | Comments |
| --- | --- | --- | --- |
| Clause 6: Transmitter Characteristics |  |  |  |
| UE maximum output power | Yes | Yes | For category M1 and NB1/NB2, 23dBm with +/-2dB tolerance |
| UE maximum output power reduction | Yes | Yes | Different MPR tables for category M1 and NB1/NB2 separately. |
| UE additional maximum output power reduction | No |  |  |
| Spurious emissions for UE co-existence | No |  |  |
| Additional spurious emissions | No |  |  |
| 7: Receiver characteristics |  |  |  |
| Minimum requirements (QPSK) | No |  |  |
| In-band blocking | No | Yes if there is no specific concern | Different band group may have different Different In-band blocking tables for category M1 and NB1/NB2 separately. |
| Out-of-band blocking | No | See comments | Different Out-of-band requirements for b255 and b256; whether to leverage requirement from b255 or b256 needs further discussion. |
| Narrow band blocking | No | Yes if there is no specific concern |  |
| RX spurious emission | No | Yes if there is no specific concern |  |

Conclusion

In the contribution, the potential UE RF requirements are provided.

Observation 1: Regarding the new IoT NTN FDD band (L+S band) UE RF requirements, to distinguish band-agnostic UE RF requirements from TS 36.102 is provided in Table 2.1-1.

Proposal 1: To agree the proposal in the Table 2.1-1 if there is no specific concern, or at least use them as the starting point for further discussions on new IoT NTN FDD band (L+S band) UE RF requirements.

- References

- RP-230792, " New WID: FDD band (L+S band) for IoT NTN operation ", MediaTek, Apple, Globalstar, Kepler communication, Qualcomm, ZTE