3GPP TSG-RAN WG4 Meeting #106bis-e	R4- 2304518

Online, April 17 – April 26, 2023

Source: 	Huawei, HiSilicon

Title:	PC2 A-MPR for band n8 NS_43

Agenda Item:	4.22.2

WI Code:	HPUE_NR_FR1_FDD_R18

Document for:	Approval

# 1	Introduction

The PC2 A-MPR simulation results for band n8 NS_43 have been presented in [1]. Further study is carried out and updated proposals on the A-MPR requirements to be specified are shown in the sequel.

# 2	Discussion

Additional simulation results are obtained for NS_18 PC2 1Tx with BW=10MHz and SCS=30kHz, which are shown in Table 1 below. The figures in the 2nd column show the differences of the A-MPR simulation results between PC2 and PC3, and the figures in the 3rd column plot only the PC2 A-MPR, for which 0dB is assigned if A-MPR ≤ PC2 MPR.

Table 1: A-MPR simulation results for NS_43 with BW=10MHz, SCS=30kHz

| BW | DIFF A-MPR (PC2-PC3) | PC2 A-MPR |
| --- | --- | --- |
| 10MHz |  |  |

It can be seen from the results in Table 1 that some A-MPR is needed for PC2 NS_18 for BW=10MHz with SCS=30kHz, although the PC3 A-MPR in the current specification is only allowed for SCS=15kHz.

The existing PC3 A-MPR requirements in TS 38.101-1 are duplicated below.

#### 6.2.3.6	A-MPR for NS_43 and NS_43U

Table 6.2.3.6-1: A-MPR regions for NS_43

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 15 | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 40 | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 45 | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | < 1.8 MHz /12/SCS > 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.6-2. NOTE 2: 15 kHz SCS unless otherwise stated NOTE 3: Void |  |  |  |  |  |  |  |

Table 6.2.3.6-2: A-MPR for NS_43

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | N/A | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A | ≤ 9 |
|  | QPSK | ≤ 2 |  |  |  |  |  | ≤ 2.5 |  |  |  | ≤ 9 |
|  | 16 QAM |  |  |  |  |  |  |  |  | ≤ 2.5 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 2.5 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |
| CP-OFDM | QPSK | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 4 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |

It can be seen that: for BW=5MHz, A1 applies; for BW=10MHz, A2 is for Pi/2 BPSK, A3 for 64QAM, and A4 for QPSK; for BW=15MHz, A6 applies.

Based on the simulation results [1] for BW=5MHz, it’s proposed to modify the A-MPR values for A1 as follows.

Proposal 1: Define the 1Tx PC2 A-MPR requirements for BW=5MHz as follows:

| Modulation/Waveform |  | A1 (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | [1] | N/A |
|  | QPSK | ≤ [2+1.5] |  |
|  | 16 QAM | ≤ [3+0.5] |  |
|  | 64 QAM | ≤ [3+0.5] |  |
|  | 256 QAM |  |  |
| CP-OFDM | QPSK | ≤ [3.5+0.5] |  |
|  | 16 QAM | ≤ [3.5+0.5] |  |
|  | 64 QAM | ≤ [4] |  |
|  | 256 QAM |  |  |

When defining the A-MPR regions for BW=10MHz, the current specification uses exact RB numbers for the limits of L_CRB, e.g. >40 or >45 in region A. As a result, the regions are not scalable if the SCS is changed from 15kHz to 30kHz.

Proposal 2: Convert the exact RB length limits into SCS-dependent values in the A-MPR table for 10MHz, e.g. 40 to 7.2MHz/12/SCS and 45 to 8.1MHz/12/SCS. And modify Note 2 to allow 30kHz SCS for PC2.

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 10 MHz | FC = 910 |  | > 7.2 MHz/12/SCS | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 8.1 MHz/12/SCS | A3 |  | > 7.2 MHz/12/SCS | A5 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.6-2. NOTE 2: Only 15 kHz SCS is applicable for power class 3; both 15kHz and 30kHz SCS are applicable for power class 2. NOTE 3: Void |  |  |  |  |  |  |  |

Based on the simulation results [1], it’s found that the existing regions needs to be expanded in addition to the increase of PC3 A-MPR values. More explicitly, the proposed changes are shown in Table 2.

Table 2: Comparison of PC2 and PC3 A-MPR for 10MHz

| Modulation | Waveform | PC3 A-MPR | Proposed new A-MPR for PC2 | Proposed expansion for PC2 |
| --- | --- | --- | --- | --- |
| QPSK | DFT-s-OFDM | A4, 2.5dB | A4, 3.5dB | A5, 5.5dB |
|  | CP-OFDM | A5, 4dB | A5, 6dB | A4, 4dB |
| 16QAM | DFT-s-OFDM | A5, 2.5dB | A5, 5dB | A4, 3.5dB |
|  | CP-OFDM | A5, 4dB | A5, 6dB | A4, 4.5dB |
| 64QAM | DFT-s-OFDM | A3, 2.5dB | A3, 5dB | A2, 4.5dB |
|  | CP-OFDM | A3, 4dB | A3, 6.5dB | A2, 5dB |

The A-MPR regions are illustrated in Figure 1 below. It can be seen that the region for A5 is contained in the region for A4; and the region for A3 is contained in the region for A2.

A2/A5

A3

A4

Figure 1: Illustration of the A-MPR regions for A2/A3/A4/A5 for BW=10MHz.

Proposal 3: Define the 1Tx PC2 A-MPR for 10MHz as shown in the table below:

| Modulation/Waveform |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [1.5] | N/A |  |  |  | N/A |  | N/A |
|  | QPSK |  |  |  |  | ≤ [2.5+1] |  | ≤ [2.5+3] |  |
|  | 16 QAM |  |  |  |  | ≤ [3.5] |  | ≤ [2.5+2.5] |  |
|  | 64 QAM | ≤ [4.5] |  | ≤ [2.5+2.5] |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK |  |  |  |  | ≤ [4] |  | ≤ [4+2] |  |
|  | 16 QAM |  |  |  |  | ≤ [4.5] |  | ≤ [4+2] |  |
|  | 64 QAM | ≤ [5] |  | ≤ [4+2.5] |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |

For BW=15MHz, three regions are defined in the existing PC3 A-MPR requirements, all having the A-MPR value of A6. However, it’s observed that RB allocations having RB_start=1.8MHz/12/SCS do not belong to any of the A-MPR regions. Hence, the following fix is proposed.

Proposal 4: For BW=15MHz, change the RB start in region A from RBstart< 1.8 MHz /12/SCS to RBstart≤ 1.8 MHz /12/SCS for both PC3 and PC2.

For 1Tx PC2, it’s found that the A-MPR value A6 for the existing regions needs to be increased. Additionally, new regions are needed to accommodate the expansion of the current regions, which will have the new A-MPR value of A7.

Proposal 5: For BW=15MHz 1Tx PC2, define the A-MPR regions and A-MPR values as follows.

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 15 MHz | FC = 907.5 | ≤ 1.8 MHz /12/SCS > 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
|  |  |  |  |  | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | < 7.2 MHz/12/SCS | A7 |
|  |  |  |  |  | ≥ 1.8 MHz/12/SCS ≤ 7.2 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 |

| Modulation/Waveform |  | A6 | A7 |  |
| --- | --- | --- | --- | --- |
|  |  | Outer / Inner (dB) | Outer (dB) | Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [9+3] | ≤ [4] | [3] |
|  | QPSK | ≤ [9+3] | ≤ [4] | [3] |
|  | 16 QAM | ≤ [9+3] | ≤ [5] | [4] |
|  | 64 QAM | ≤ [9+3] | ≤ [5.5] | [5.5] |
|  | 256 QAM | ≤ [9+3] | ≤ [6.5] | [5.5] |
| CP-OFDM | QPSK | ≤ [9+3] | ≤ [6] | [4.5] |
|  | 16 QAM | ≤ [9+3] | ≤ [6] | [5] |
|  | 64 QAM | ≤ [9+3] | ≤ [6.5] | [6] |
|  | 256 QAM | ≤ [9+3] | ≤ [7] | [7] |

Note that A7 is derived based on the simulation results as well as satisfies A7 ≤ PC3_MPR+3dB. Figure 3 illustrates the A-MPR regions for BW=15MHz.

Figure 3: Illustration of the A-MPR regions for A6/A7 for BW=15MHz.

A6

A7

A7

A6

A6

# 3	Conclusion

In this paper, we have presented our updated proposals on the 1Tx PC2 A-MPR requirements for NS_43.

Proposal 1: Define the 1Tx PC2 A-MPR requirements for BW=5MHz as follows:

| Modulation/Waveform |  | A1 (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | [1] | N/A |
|  | QPSK | ≤ [3.5] |  |
|  | 16 QAM | ≤ [3.5] |  |
|  | 64 QAM | ≤ [3.5] |  |
|  | 256 QAM |  |  |
| CP-OFDM | QPSK | ≤ [4] |  |
|  | 16 QAM | ≤ [4] |  |
|  | 64 QAM | ≤ [4] |  |
|  | 256 QAM |  |  |

Proposal 2: Convert the exact RB length limits into SCS-dependent values in the A-MPR table for 10MHz, e.g. 40 to 7.2MHz/12/SCS and 45 to 8.1MHz/12/SCS. And modify Note 2 to allow 30kHz SCS for PC2.

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 10 MHz | FC = 910 |  | > 7.2 MHz/12/SCS | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 8.1 MHz/12/SCS | A3 |  | > 7.2 MHz/12/SCS | A5 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.6-2. NOTE 2: Only 15 kHz SCS is applicable for power class 3; both 15kHz and 30kHz SCS are applicable for power class 2. NOTE 3: Void |  |  |  |  |  |  |  |

Proposal 3: Define the 1Tx PC2 A-MPR for 10MHz as shown in the table below:

| Modulation/Waveform |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [1.5] | N/A |  |  |  | N/A |  | N/A |
|  | QPSK |  |  |  |  | ≤ [2.5+1] |  | ≤ [2.5+3] |  |
|  | 16 QAM |  |  |  |  | ≤ [3.5] |  | ≤ [2.5+2.5] |  |
|  | 64 QAM | ≤ [4.5] |  | ≤ [2.5+2.5] |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK |  |  |  |  | ≤ [4] |  | ≤ [4+2] |  |
|  | 16 QAM |  |  |  |  | ≤ [4.5] |  | ≤ [4+2] |  |
|  | 64 QAM | ≤ [5] |  | ≤ [4+2.5] |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |

Proposal 4: For BW=15MHz, change the RB start in region A from RBstart< 1.8 MHz /12/SCS to RBstart≤ 1.8 MHz /12/SCS for both PC3 and PC2.

Proposal 5: For BW=15MHz 1Tx PC2, define the A-MPR regions and A-MPR values as follows.

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 15 MHz | FC = 907.5 | ≤ 1.8 MHz /12/SCS > 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
|  |  |  |  |  | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | < 7.2 MHz/12/SCS | A7 |
|  |  |  |  |  | ≥ 1.8 MHz/12/SCS ≤ 7.2 MHz/12/SCS | > 5.4 MHz/12/SCS | A7 |

| Modulation/Waveform |  | A6 | A7 |  |
| --- | --- | --- | --- | --- |
|  |  | Outer / Inner (dB) | Outer (dB) | Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [11] | ≤ [4] | [3] |
|  | QPSK | ≤ [11] | ≤ [4] | [3] |
|  | 16 QAM | ≤ [11] | ≤ [5] | [4] |
|  | 64 QAM | ≤ [11] | ≤ [5.5] | [5.5] |
|  | 256 QAM | ≤ [11] | ≤ [6.5] | [5.5] |
| CP-OFDM | QPSK | ≤ [11] | ≤ [6] | [4.5] |
|  | 16 QAM | ≤ [11] | ≤ [6] | [5] |
|  | 64 QAM | ≤ [11] | ≤ [6.5] | [6] |
|  | 256 QAM | ≤ [11] | ≤ [7] | [7] |

# References

- R4-2300716 PC2 A-MPR for band n8, Huawei, HiSilicon, RAN4#106