3GPP TSG-RAN WG4 Meeting #106bis-e	R4- 2304524

# Online, April 17 – April 26, 2023

Source:	Huawei, HiSilicon, BT plc

Title:	TP to TR 38.718-03-01 for CA_n1-n28-n46 and DC_n1-n28-n46

Agenda item:	4.11.2

WI code:	NR_CADC_R18_3BDL_xBUL-Core

Document for:	Approval

# Background

This contribution provides text proposal on the NR CA/DC band combination CA_n1-n28-n46 and DC_n1-n28-n46 as defined in the WID [1].

# Text Proposal

##### ---Start of changes---

## 5.x	CA_n1-n28-n46

### 5.x.1	Common for 1 band UL and 2 bands UL CA

### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n28-n46 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported channel bandwidths per CA configuration

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n28A-n46A | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n46 | 10, 20, 40, 60, 80 |  |
| CA_n1A-n28A-n46C | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n46 | CA_n46C_BCS0 |  |
| CA_n1A-n28A-n46D | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n46 | CA_n46D_BCS0 |  |
| CA_n1A-n28A-n46(2A) | CA_n1A-n28A CA_n1A-n46A CA_n28A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n46 | CA_n46(2A)_BCS0 |  |

#### 5.x.1.3	∆TIB,c and ∆RIB,c values

For CA_n1-n28-n46, the TIB,c and RIB,c values are are given in the tables below.

Table 5.x.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)8 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |  |
| CA_n1-n28-n46 | 0.3 | 0.6 | - |
| NOTE 8: “-” denotes ΔTIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.x.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)9 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |  |
| CA_n1-n28-n46 | - | - | 0.5 |
| NOTE 9: “-” denotes ΔRIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	Co-existence studies

It can be seen from the co-existence studies of the lower order CA configurations that the IMD products of the dual-UL may fall into the DL of the 3rd band in the following cases:

- IMD4 of band n1 UL and band n28 UL falls into band n46 DL

- IMD4 of band n1 UL and n46 UL falls into band n28 DL

- IMD5 of band n28 UL and n46 UL falls into band n1 DL

#### 5.x.2.2	REFSENs requirements

Due to the limitation of applicable channel bandwidths, it’s impossible to define the carrier frequencies for the test case of IMD5 of band n28 UL and n46 UL falling into band n1 DL. And the MSD for the NR-U band n46 is set to N/A.

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n28-n46 | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710 | 5 | 25 | 765 | N/A | FDD | N/A |
|  | n46 | 5215 | 20 | 100 | 5215 | N/A | TDD | IMD4 |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | FDD | N/A |
|  | n28 | 710 | 5 | 25 | 765 | 10.5 | FDD | IMD4 |
|  | n46 | 5160 | 20 | 100 | 5160 | N/A | TDD | N/A |

##### ---Next changes---

## 6.x	DC_n28-n46-n78

### 6.x.1	Configurations for DC_n1-n28-n46

Table 6.x.1-1: Inter-band NR DC configurations (three bands)

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n1A-n28A-n46A DC_n1A-n28A-n46C DC_n1A-n28A-n46D DC_n1A-n28A-n46(2A) | DC_n1A-n46A DC_n28A-n46A |

##### ---End of changes---

# Reference

[1]	RP-230084, “Revised WID on Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity  for 3 bands DL with x bands UL (x=1,2)”, ZTE Corporation