# 3GPP TSG-RAN WG4 Meeting # 106-bis-e                              R4-2304529

# Online, April 17 – April 21, 2023

# Source: 	Nokia, Nokia Shanghai Bell

# Title: 	Discussion on NCR EMC core requirements

# Agenda item:	5.29.3

# Document for:	Approval

- Introduction

One of the objectives of the RAN4#106-bis-e meeting target is to consider the effect of new features e.g., NCR-MT UL transmission, reference points NCR type 1-O and 1-H, for EMC core requirements. In this document, we give some discussions on the topic of EMC core requirements and share our views.

- Discussions

2.1 New features to under consideration

Changes to the TS38.106 specification [1] were considered at the RAN4#106 meeting [2]. New RF core requirements included specifications for NCR-MT UL transmission and NCR Type 1-O and Type 1-H reference points. This document discusses the impact of the new features on the core EMC requirements TS 38.114 specification [3].

Similar to the NCR type 2-O RF requirements (i.e., Over-the-Air (OTA) requirements), NCR Type 1-O RF requirements are taken account in the far field at the Radiated Interface Boundary (RIB). The type 2-O requirement is applied to FR2, and the type 1-O requirement is applied to FR1. Figure 1 illustrates the radiated reference points for NCR Type 1-O. The interface to the BS is referred to as the BS-side RIB and the interface to the UE is referred to as the UE-side RIB.

Figure 1: Radiated reference points for NCR Type 1-O

In [2] it is proposed that the two sets of requirements are needed for NCR Type 1-H (see Figure 2), similar to the BS Type 1-H requirements: Conducted requirements are to be applied for the TAB connectors and OTA requirements for the RIBs.

Figure 2: Radiated reference points for NCR Type 1-H

2.3 Changes to EMC core requirements

The scope of [3] specification is defined to be Electromagnetic Compatibility (EMC) requirements for the assessment of NR repeaters and ancillary equipment in Chapter 1. The scope specifies the applicable requirements, procedures, test conditions, performance assessment and performance criteria for NR repeaters and associated ancillary equipment in the categories of repeater type 1-C and repeater type 2-O.

Based on new features defined in section 2.1 we propose extending the scope of [3] specification to cover also categories repeater type 1-H and repeater type 1-C.

Observation 1: Repeater type 1-H and repeater type 1-O are missing from the scope of the [3] specification.

Proposal 1: We propose to extend the scope to include type 1-O repeaters as shown in Figure 1 and type 1-H repeaters as shown in Figure 2.

Chapter 3 of the [3] specification lists definitions, symbols and abbreviations. The terms NCR Type 1-O and NCR Type 1-H are not included in the list of definitions and NCR is not included in the list of abbreviations.

Observation 2: Terms of NCR type 1-O and NCR type 1-H are not included in the list of definitions, and NCR is not included in the list of abbreviations in the [3] specification.

Proposal 2: The terms NCR type 1-O and NCR type 1-H need to be included in the list of definitions, and NCR to the list of abbreviations in the [3] specification.

In Chapter 4.1 and 4.2 of [3] specification, test conditions are defined for NR repeater type 1-C (referring to TS 38.115-1 [4]) and repeater 2-O type (referring to TS 38.115-2 [5]) only. Test conditions are not defined for NR repeater type 1-O and repeater type 1-H. The wanted input signal level is not defined for new types needs to be added to the specification.

Observation 3: Test conditions are not defined for NR repeater type 1-O and repeater type 1-H in [3] specification. The wanted input signal level is not defined.

Proposal 3: Test conditions for NR repeaters type 1-O and 1-H are recommended to be defined in [3] specification. The definition of input signal level needs to be added: "The wanted input signal level shall be set to a level where the performance is not limited by the receiver noise floor or strong signal effects."

The exclusion band is defined in Chapter 4.4 of the [3] specification. As the NCR can operate access and backhaul links in different NR NCR operating bands, then the transmitter exclusion and receiver exclusion band for NCR must be applied separately for the access and backhaul links.

Observation 4: As the NCR can operate access and backhaul links in different NR NCR operating bands, then the transmitter exclusion and receiver exclusion band for NCR must be applied separately for the access and backhaul links.

Proposal 4: The exclusion bands need to be defined separately for the transmitter and receiver to the specification [3].

NR repeaters test configurations are defined for NR repeater type 1-C and NR repeater type 2-O in chapter 4.5 in the [3] specification. The specification does not define configurations for type 1-H and 1-O.

Observation 5: NR repeaters test configurations are not defined for NR repeater type 1-H and 1-O.

Proposal 5: NR repeaters test configurations are to be defined for NR repeater type 1-H and 1-O.

Performance assessments are defined in Chapter 5 in the [3] specification. Assessment of throughput of NCR-MT is not defined in the specification.

Observation 6: Assessment of throughput of NCR-MT is not defined in the specification [3].

Proposal 6: Assessment of throughput of NCR-MT needs to be defined in the specification [3].

Performance criteria for continuous phenomena for NR repeaters is defined in chapter 6.1 in the [3] specification. The performance criteria need to be defined to NCR-MT similarly as in 3GPP TS38.175 specification [6].

Table 6.1-1: FR1 performance criteria for continuous phenomena for NCR

| NR channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate for NCR-MT | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 10, 15 | 30 | G-FR1-xx-xx in annex x.x in TS 38.106 [1] | Throughput > 95 %, no loss of service |
| 10, 15 | 60 | G-FR1-xx-xx in annex x.x in TS 38.106 [1] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-xx-xx in annex x.x in TS 38.106 [1] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-xx-xx in annex x.x in TS 38.106 [1] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Table 6.1-2: FR2 performance criteria for continuous phenomena for NCR

| NR channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate for IAB-MT | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-xx-xx in annex x.x in TS 38.106 [1] | Throughput > 95 %, no loss of service |
| 50 | 120 | G-FR2-xx-xx in annex x.x in TS 38.106 [1] |  |
| 100, 200, 400 | 120 | G-FR2-xx-xx in annex x.x in TS 38.106 [1] |  |
| NR channel bandwidth (MHz) | Sub-carrier spacing (kHz) | G-FR2-xx-xx in annex x.x in TS 38.106 [1] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Observation 7: Performance criteria is not defined for NCR-MT in the [3] specification.

Proposal 7: Performance criteria is to be defined for NCR-MT in the [3] specification.

In [3] specification, the applicability overview for emissions is defined in Table 7.1-1. It defines the applicability of radiated emissions for repeater type 1-C. The applicability of radiated emissions is not defined for 1-H type.

Observation 8: The applicability of radiated emissions is not defined for NR repeater type 1-H in the [3] specification.

Proposal 8: The applicability of radiated emissions shall be defined for NR repeater type 1-H in the [3] specification.

[3] specification defines test configurations, radiated emissions (repeater) and limits for emissions in Chapters 8.1, 8.2.1 and 8.2.1.3. It defines configurations, applicability and limits for repeater type 1-C and 2-O. The configurations, applicability and limits of emissions are not defined for NR repeater 1-H type and repeater type 1-O.

Observation 9: The configurations, applicability and limits of emissions are not defined for NR repeater type 1-H and repeater type 1-O in the [3] specification.

Proposal 9: The configurations, applicability and limits of emissions are to be defined for NR repeater type 1-H and repeater type 1-O in the [3] specification.

[3] specification defines test configurations for immunity testing in Chapter 9.1. It defines configurations for repeater type 1-C and 2-O. The immunity test configurations are not defined for NR repeater 1-H type and repeater type 1-O.

Observation 10: The immunity test configurations are not defined for NR repeater type 1-H and repeater type 1-O in the [3] specification.

Proposal 10: The immunity test configurations shall be defined for NR repeater type 1-H and repeater type 1-O in the [3] specification.

Chapter 9.2.2 of the [3] specification defines test method and level for RF electromagnetic field. It defines test method and level for repeater type 1-C and 2-O. Test method and level for RF electromagnetic field are not defined for NR repeater 1-H type and repeater type 1-O.

Observation 11: Test method and level for RF electromagnetic field are not defined for NR repeater type 1-H and repeater type 1-O in [3] specification.

Proposal 11: Test method and level for RF electromagnetic field are to be defined for NR repeater type 1-H and repeater type 1-O in [3] specification.

- Conclusion

In this paper, we provide some discussion for the EMC core requirement from the NCR type aspects. The conclusions are shown below:

Observation 1: Repeater type 1-H and repeater type 1-O are missing from the scope of the [3] specification.

Proposal 1: We propose to extend the scope to include type 1-O repeaters as shown in Figure 1 and type 1-H repeaters as shown in Figure 2.

Observation 2: Terms of NCR type 1-O and NCR type 1-H are not included in the list of definitions, and NCR is not included in the list of abbreviations in the [3] specification.

Proposal 2: The terms NCR type 1-O and NCR type 1-H need to be included in the list of definitions, and NCR to the list of abbreviations in the [3] specification.

Observation 3: Test conditions are not defined for NR repeater type 1-O and repeater type 1-H in [3] specification. The wanted input signal level is not defined.

Proposal 3: Test conditions for NR repeaters type 1-O and 1-H are recommended to be defined in [3] specification. The definition of input signal level needs to be added: "The wanted input signal level shall be set to a level where the performance is not limited by the receiver noise floor or strong signal effects."

Observation 4: As the NCR can operate access and backhaul links in different NR NCR operating bands, then the transmitter exclusion and receiver exclusion band for NCR must be applied separately for the access and backhaul links.

Proposal 4: The exclusion bands need to be defined separately for the transmitter and receiver to the specification [3].

Observation 5: NR repeaters test configurations are not defined for NR repeater type 1-H and 1-O.

Proposal 5: NR repeaters test configurations are to be defined for NR repeater type 1-H and 1-O.

Observation 6: Assessment of throughput of NCR-MT is not defined in the specification [3].

Proposal 6: Assessment of throughput of NCR-MT need to be defined in the specification [3].

Observation 7: Performance criteria is not defined for NCR-MT in the [3] specification.

Proposal 7: Performance criteria is to be defined for NCR-MT in the [3] specification.

Observation 8: The applicability of radiated emissions is not defined for NR repeater type 1-H in the [3] specification.

Proposal 8: The applicability of radiated emissions shall be defined for NR repeater type 1-H in the [3] specification.

Observation 9: The configurations, applicability and limits of emissions are not defined for NR repeater type 1-H and repeater type 1-O in the [3] specification.

Proposal 9: The configurations, applicability and limits of emissions are to be defined for NR repeater type 1-H and repeater type 1-O in the [3] specification.

Observation 10: The immunity test configurations are not defined for NR repeater type 1-H and repeater type 1-O in the [3] specification.

Proposal 10: The immunity test configurations shall be defined for NR repeater type 1-H and repeater type 1-O in the [3] specification.

Observation 11: Test method and level for RF electromagnetic field are not defined for NR repeater type 1-H and repeater type 1-O in [3] specification.

Proposal 11: Test method and level for RF electromagnetic field are to be defined for NR repeater type 1-H and repeater type 1-O in [3] specification.

- Reference

- 3GPP TS 38.106, 5G; NR; Repeater radio transmission and reception

- R4-2300694 Discussion on RAN1 LS reply on NCR-MT transmission and Beam correspondence

- 3GPP TS38.114, NR; Repeaters ElectroMagnetic Compatibility (EMC)

- 3GPP TS 38.115-1: "NR; Repeater conformance testing - Part 1: Conducted conformance testing"

- 3GPP TS 38.115-2: "NR; Repeater conformance testing - Part 1: Radiated conformance testing"

- 3GPP TS38.175: “NR; Integrated access and backhaul ElectroMagnetic Compatibility (EMC)”