3GPP TSG-RAN WG4 Meeting #106bis-e 	R4-2304534

Electronic Meeting, April 17 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	RF requirements for Network energy savings for NR

Agenda item:	5.35.2	BS and UE RF requirements	[Netw_Energy_NR-Core]

Document for:	Approval

Introduction

At RAN#98 meeting, the work item Network energy savings (NES) for NR was approved to specify the selected network energy saving techniques in various domains [1]. One of the objectives for RAN4 is to specify the corresponding RRM/RF core requirements, if necessary.

In this contribution we provide initial considerations on possible impact to BS and UE RF requirements.

Discussion

General impact to BS and UE RF requirements

According to the WID, a list of network energy saving features will be discussed in RAN1, RAN2, and RAN4. Even on the topics led by RAN1 and RAN2, the corresponding RRM/RF core requirements need to be specified in RAN4.

### 4.1 Objective of SI or Core part WI or Testing part WI The objectives of the work item are the following: - Specify SSB-less SCell operation for inter-band CA for FR1 and co-located cells, if found feasible by RAN4 study, where a UE measures SSB transmitted on PCell or another SCell for an SCell’s time/frequency synchronization (including downlink AGC), and L1/L3 measurements, including potential enhancement on SCell activation procedures if necessary [RAN4, RAN2] - Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided. - Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements - Specify mechanism(s) to prevent legacy UEs camping on cells adopting the Rel-18 NES techniques, if necessary [RAN2] - Specify CHO procedure enhancement(s) in case source/target cell is in NES mode [RAN2] - Specify inter-node beam activation and enhancements on restricting paging in a limited area [RAN3]. - Specify the corresponding RRM/RF core requirements, if necessary, for the above features [RAN4]

As we discuss in our contribution [2] it is important to prioritize the discussion on SSB-less SCell operation for inter-band CA for FR1 and co-located cell.

RAN4 prioritization the discussion on SSB-less SCell operation for inter-band CA for FR1 and co-located cell would be beneficial to RAN4 works.

The RRM/RF requirements for the other network energy saving features e.g., cell DTX/DRX, spatial and power domain techniques can be discussed after there is conclusion from RAN1/RAN2.

RAN1/RAN2 conclusions are needed to discuss RRM/RF potential impact on some NES features e.g., cell DTX/DRX, spatial and power domain techniques.

SSB-less operation impact

SSB-less SCell operation for inter-band CA for FR1 and co-located cell may have some impact to carrier aggregation band combinations that are defined in UE core specifications TS 38.101-1/2/3. The feasible set of PCells (or other SCells) that SCell may perform SSB-less operation may have different frequency offsets, as shown in Figure 1.

Figure 1: Candidate PCells for SSB-less operation with the SCell.

The time/frequency synchronization offsets may also depend on how far apart the bands of the SCell and PCell or other SCell are located in the channel raster. That is the selection of the PCell from which the SSBs can be used for the synchronization of the SCell may impact the accuracy of SCell’s time/frequency synchronization.

However, as we discuss in our contribution [3] RAN4 should perform feasibility studies including simulation based on agreed assumptions and metrics. Thus, some impact to RF requirements may be possible both in BS RF core specification TS 38.104 and UE RF core specification TS 38.101-1, 38.101-2 and 38.101-3.

At least SSB-less operation may have impact to BS and/or UE RF requirements.

Proposal 1: RAN4 RF core requirements can be specified when NES techniques would find feasible.

Beam management impact

WID [1] includes following:

- Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2]

Spatial domain energy saving enhancements will require specific beam management for efficient use of energy. Such kind of changes in antenna arrangements of BS with an active antenna system may based on some muting patterns of antenna elements. Introducing different transceiver different muting patterns may have impact to some RF core requirements.

Introducing beam management technique may require updates in some RF core requirements.

Also in conformance test specification, some additional manufacturer declarations may be needed for at least BS test specifications TS 38.141-1 and TS38.141-2.

Beam management techniques introduced for NES will require updates in manufacturer declarations in BS conformance specification.

Conclusion

In this contribution we provide initial considerations on possible impact to BS and UE RF requirements.  The following Observations and Proposal were made:

Observation 1: RAN4 prioritization the discussion on SSB-less SCell operation for inter-band CA for FR1 and co-located cell would be beneficial to RAN4 works.

Observation 2: RAN1/RAN2 conclusions are needed to discuss RRM/RF potential impact on some NES features e.g., cell DTX/DRX, spatial and power domain techniques.

Observation 3: At least SSB-less operation may have impact to BS and/or UE RF requirements.

Proposal 1: RAN4 RF core requirements can be specified when NES techniques would find feasible.

Observation 4: Introducing beam management technique may require updates in some RF core requirements.

Observation 5: Beam management techniques introduced for NES will require updates in manufacturer declarations in BS conformance specification.

- References

- RP-223540, Network energy savings for NR, Huawei

- R4-2304180, SSB-less operation for FR1 inter-band co-located CA, Nokia, Nokia Shanghai Bell

- R4-2304181, Details of the feasibility study of SSB-less SCell operation for FR1 inter-band co-located CA, Nokia, Nokia Shanghai Bell