3GPP TSG-RAN WG4 Meeting #106bis-e 	R4-2304535

Electronic Meeting, April 17 – 26, 2023

Source:	Nokia, Nokia Shanghai Bell

Title:	Clarifications on RAN1 LS Reply

Agenda item:	5.20.1 General and work plan [FS_NR_duplex_evo]

Document for:	Approval

Introduction

This contribution discusses following topic:

- The LS reply to the LS received from RAN1 in [1]

Discussion

Reply LS to R1-2302087

Interference suppression capability of co-site gNB-gNB adjacent channel CLI

In RAN1 #112, RAN1 sent an LS to RAN4 in [1]. The first topic discussed in this LS consisted of the gNB-gNB adjacent-channel CLI model, and RAN1 inquired on the value of the interference suppression capability of co-site inter-sector co-channel inter-subband CLI, αadj, co-site-x as follows:

Agreement-3 For SLS in RAN1, for co-site gNB-gNB adjacent-channel CLI modelling, reuse similar method as co-site inter-sector co-channel inter-subband CLI modeling as follows. Iadj, co-siteper-RB=Iadj, co-site-sector-1per-RB+Iadj, co-site-sector-2per-RB+Iadj, co-site-sector-3per-RB Iadj,co-site-sector-xper-RB=Ptxadj,sector-x, per-RBαadj,co-site-x*Nused-DL-RBadj, sector-xNDLRBadj - Ptxadj, sector-x, per-RB is DL Tx power of sector x per RB (in linear scale), Ptxadj,sector-x, per-RB=Ptxadj,sector-x, max/NDLRBadj. - Ptxadj,sector-x, max is the maximum DL Tx Power of sector x in adjacent channel (in linear scale). - NDLRBadj is the total number of DL RBs in adjacent channel. - Nused-DL-RBadj,sector-x is the number of DL RBs allocated for DL transmission of sector x in adjacent channel. - αadj, co-site-x is the interference suppression capability of co-site inter-sector co-channel inter-subband CLI between the aggressor sector x and the victim sector. - 10*log10αadj,co-site-x=spatial isolationdB+10*log1011ACLRBS+1ACSBS+… - FFS the concrete value of αadj, co-site-x - ACLRBS and ACSBS are in linear scale. Send an LS to RAN4 to inquire on the value of αadj, co-site-x.

First of all, we believe that there is a typo in the RAN1 LS above. In the LS, αadj, co-site-x is defined as interference suppression capability of co-site inter-sector co-channel inter-subband CLI. However, we believe that the LS is discussing the interference suppression capability of the co-site inter-sector gNB-gNB adjacent-channel CLI.

In RAN4 #106, RAN4 confirmed RAN1's understanding on the co-site inter-sector co-channel inter-subband CLI modeling (LS Reply in R4-2302885). Additionally, RAN4 advised RAN1 to use the piece-wise noise figure model in the simulations. It is our understanding that similar approach can be applied the co-site inter-sector gNB-gNB adjacent channel interference model.  In RAN4 #106, the following has been agreed about the co-channel model:

Answer from RAN4: RAN4 confirms RAN1 the co-site inter-sector co-channel inter-subband CLI modelling carried out in RAN1, except: - The noise figure model in the above answer shall also be used in the simulation. - Regarding spatial isolation values, the following values have been proposed for macro BS in RAN4: - FR1: 62-93dB with 75dB being typical values. - FR2: 75-98dB with 88dB being typical values. - Some companies have proposed that isolating materials could be added between sectors to increase the isolation. RAN4 has not yet discussed the details of what kind of materials and the building practice or whether such approaches can be applied to outdoor sites. Further improvement over the spatial isolation is FFS. - Additionally, RAN4 has not yet precluded possible improvements on receiver performance compared to baseline gNB ACS. The ACLR/ACS values for FR1 and FR2 are shown in the table below. Range; ACLR [dB]; ACS [dB]. FR-1; 45; 46. FR-2; 28; 24.

Given the above, we propose that the suppression capabilities for the co-site inter-sector gNB-gNB adjacent channel also are dependent on the spatial isolation and the ACLR and ACS parameters. Additionally, RAN1 should consider the co-site inter-sector gNB-gNB adjacent interference as input for the recently agreed piece-wise noise figure model.

Reply to RAN1 that in addition to the model proposed for the co-site inter-sector gNB-gNB adjacent-channel CLI modeling proposed in R1-2302087, the noise figure model proposed by RAN4 in R4-2302885, should also be considered.

Regarding the ACLR and ACS values, we propose that the same values are used in the co-channel and in the adjacent channel cases: the values for the co-channel inter-subband CLI model were derived from the adjacent channel values, so in our view it makes sense to maintain these.

Reply to RAN1 that ACLR and ACS values agreed for the co-site inter-sector co-channel inter-subband CLI can be reused for the co-site gNB-gNB adjacent channel case.

Regarding the spatial isolation, in multi-operator co-site deployments in TR 38.828, where results of the coexistence evaluation for CLI are presented, the isolation between co-located base stations (zero grid shift) is 30 dB in FR1 and 50-70 dB in FR2. These assumptions are the basis of the transmitter intermodulation and co-location blocking requirements for the specifications  [3]. In the last RAN4 meeting, the spatial isolation between sectors of the same operator was considered to be in the range of 62-93 dB (FR1) and 75-98 dB (FR2).

Assumptions made for SBFD co-site inter-sector co-channel inter-subband are much higher than assumption used on current co-location BS isolation assumption of 30 dB.

RAN4 to discuss the range and typical values of co-site (zero grid shift) spatial isolation in light of the assumptions of the current requirements of 30 dB in FR1 and 50 dB for FR2 for co-site gNB-gNB adjacent channel.

Reply to RAN1 that the spatial isolation values agreed for co-site inter-sector gNB-gNB adjacent channel CLI, the following are proposed for macro BS:

- FR1: 30 - 62 dB with TBD being typical values.

- FR2: 50 - 75 dB-with TBD being typical values.

In addition to question about αadj, co-site-x in RAN1’s LS, there is another aspect to consider. The LS reply sent by RAN4 in R4-2302885 mentions that RAN4 has not yet precluded possible improvements on receiver performance when using isolating materials between sectors. If used in sites of the same operator, these isolators can be thoroughly studied. The isolators’ design can be optimized for specific antenna configurations. The deployment of such isolators with the intention of improving isolation towards the other operators’ antennas would need coordination between the different operators, which might be challenging to achieve in a real deployment. Therefore, we propose to not consider any other improvement on receiver performance by using isolating materials between co-site inter-operator deployments.

For the co-site gNB-gNB adjacent channel, no improvement over the spatial isolation due to isolating materials is considered.

UE-UE co-channel channel model

Agreement-4 For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for UE-UE co-channel channel model, the power of UE-UE co-channel inter-subband CLI experienced by the victim UE on each receiver chain at DL RB n can be modelled as IUE-UE-CLIB'→B,per-RBn=IUE-UE,IBEn*CLUE B'→UE B+PtxUE B'*CLUE B'→UE B*1ICSUE*1NULRB where - IUE-UE-CLIB'→B,per-RBn is the power of UE-UE co-channel inter-subband CLI from aggressor UE B' to victim UE B on each receiver chain at one DL RB n (linear value). - PtxUE B' is UL transmission power of UE B' across all transmit chains over the allocated UL RBs (linear value) - CLUE B'→UE B is the coupling loss between UE B' and UE B (linear value), accounting for analog beamforming at the aggressor UE and victim UE - NULRB is the total number of UL RBs in the UL subband - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input. - 10*log10IUE-UE,IBEn=maxPtxUE B',per-RB-30dB,10*log10IGeneral+IIQfor FR1maxPtxUE B',per-RB-25dB,10*log10IGeneral+IIQfor FR2-1, wherein, - For SBFD Subband configuration with {DUD} pattern, IIQ can be ignored - 10*log10IGeneral=PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-3-5⋅ΔRB-1LCRB,-57dBm+10log10SCS/15kHz-PtxUE B',per-RBfor FR1PtxUE B',per-RB+max-25-10⋅log10(NRB/LCRB),20⋅log10EVM-5⋅ΔRB-1LCRB,-55.1dBm-PtxUE B',per-RBfor FR2-1 - PtxUE B',per-RB is UL transmission power of UE B' across all transmit chains per RB (linear value). PtxUE B',per-RB=PtxUE B'/ LCRB, and LCRB is the number of UL RBs allocated for UL transmission of UE B'. - NRB is the Transmission Bandwidth Configuration, referring to Table 5.3.2-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1. - NRB=273 for FR1 with 100MHz transmission bandwidth and 30kHz SCS - NRB=132 for FR2-1 with 200MHz transmission bandwidth and 120kHz SCS - ΔRB is the starting frequency offset between the allocated UL RBs and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated UL RBs) - EVM is the limit specified in Table 6.4.2.1-1 in TS 38.101-1 for FR1 and in TS 38.101-2 for FR2-1 for the modulation format used in the allocated RBs. Send an LS to RAN4 to inform them of the above agreement and to check if the RAN1 agreement is inline with RAN4’s understanding.

RAN1’s working assumptions for the modelling of UE-UE co-channel inter-subband CLI at a given DL RB n is essentially modelled as the sum of the aggressor UE leakage component and the victim UE receiver selectivity component. For the UE leakage component, RAN1 adopts the UE in-band emissions model defined in TS 38.101-1 and TS 38.101-2 for FR1 and FR2 UEs respectively. It is noted that the carrier leakage component of the in-band emissions model is not considered, as already suggested by RAN4 in the LS response R4-2220243. Similarly, the IQ image component can also be omitted DUD cases since the image frequencies of the allocated UL RBs with respect to the middle of the operating channel are expected to overlap with the UL subband and therefore won’t affect the reception of a DL UE. For the UE selectivity component, RAN1 leaves up to RAN4 to decide on the in-channel selectivity value. The UE adjacent channel selectivity (ACS) typically assumes analog and/or digital channel filter that provides suppression of the interfering signal. For in-channel selectivity (ICS) such filter might not be available, hence the selectivity could be worse. Nevertheless, in our view, RAN1 should adopt the ACS assumptions for the ICS model. Thus, RAN4 suggests the following UE ICS values:

| Frequency range | ICS [dB] |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 23 |

RAN4 agrees with the RAN1’s model for the UE-UE co-channel inter-subband CLI when only large-scale fading is considered.

Working assumption: For SLS in RAN1, if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the UE-UE co-channel inter-subband CLI signal across all Rx chains at DL RB n at victim UE can be modeled as: IInter-UE-CLIn=Ileakagen+Iselectivity, where, - Ileakagen=HCLIn W yn is the first part of UE-UE co-channel inter-subband CLI across all Rx chains at DL RB n, caused by power leakage at aggressor UE, - HCLIn is the NR×NT channel matrix between aggressor UE and victim UE at DL RB n, the beamforming of the aggressor UE and the victim UE can be taken into account by HCLIn - NR is the number of Rx chains and NT is the number of Tx chains - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1. - yn=y0n,y1n,…,yRnT, - ykn~N0,IUE-UE,IBEn, k=0,1,…,R-1, is modelled as white Gaussian noise - IUE-UE,IBEn has the same meaning as in the agreement for the case only large-scale fading is modelled - Iselectivity is modelled as frequency flat Iselectivity=z0n,z1n,…,zNR-1nT - zkn~N0,σz,n2, k=0,1,…,NR-1, is modelled as white Gaussian noise, σz,n2=PblockerICSUE×NULRB - Pblocker=1NR*m∈Used UL RBsHCLImWsm2 - HCLIm is the NR×NT channel matrix between aggressor UE and victim UE at UL RB m, the analog beams of the aggressor UE and the victim gNB can be taken into account by HCLIm, - W is the NT×R normalized wideband UL digital precoder of the aggressor UE, WF=1 - sm is the symbol transmitted at UL RB m at aggressor UE with transmission power for each layer as PtxUE B',per-RB. - PtxUE B',per-RB has the same meaning as in the agreement for the case only large-scale fading is modelled - NULRB is the total number of UL RBs in the UL subbands, - ICSUE is in linear scale. For the value of ICSUE, it is up to RAN4. Companies can report the value used in their simulation before receiving RAN4’s further input. Send an LS to RAN4 to inform them of the above agreement and to check if the RAN1 agreement is inline with RAN4’s understanding.

Finally, the RAN1 LS also contained a working assumption about the UE-UE co-channel model when both large-scale and small-scale fading is considered. In general, this working assumption can be confirmed. However, we believe that there are some aspects to be clarified.

First, it seems that there is a typo in ykn~N0,IUE-UE,IBEn, k=0,1,…,R-1. We believe that R should be replaced by NR which is defined as the number of Rx chains.

Secondly, when HCLIm is defined in Iselectivity, the LS states that “… the analog beams of the aggressor UE and the victim gNB”, we believe that this should be “victim UE”.

Reply to RAN1 that if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the working assumption on the model on UE-UE co-channel inter-subband CLI is inline with RAN4’s understanding.

Finally, in the same way a NF model was defined to account for the non-linearities at the gNB receiver, as pointed out in our companion contribution [3], we believe that the noise figure model should be considered at the UE side.

Reply to RAN1 that the UE NF proposed in [3] model should be considered

Conclusion

In this paper, we discussed the LS Reply to RAN1. The following observations and proposals were discussed:

Proposal 1: Reply to RAN1 that in addition to the model proposed for the co-site inter-sector gNB-gNB adjacent-channel CLI modeling proposed in R1-2302087, the noise figure model proposed by RAN4 in R4-2302885, should also be considered.

Proposal 2: Reply to RAN1 that ACLR and ACS values agreed for the co-site inter-sector co-channel inter-subband CLI can be reused for the co-site gNB-gNB adjacent channel case.

Observation 1: Assumptions made for SBFD co-site inter-sector co-channel inter-subband are much higher than assumption used on current co-location BS isolation assumption of 30 dB.

Proposal 3: RAN4 to discuss the range and typical values of co-site (zero grid shift) spatial isolation in light of the assumptions of the current requirements of 30 dB in FR1 and 50 dB for FR2 for co-site gNB-gNB adjacent channel.

Proposal 4: Reply to RAN1 that the spatial isolation values agreed for co-site inter-sector gNB-gNB adjacent channel CLI, the following are proposed for  macro BS:

	FR1: 30 - 62 dB with TBD being typical values.

	FR2: 50 - 75 dB-with TBD being typical values.

Proposal 5: For the co-site gNB-gNB adjacent channel, no improvement over the spatial isolation due to isolating materials is considered.

Proposal 6: RAN4 agrees with the RAN1’s model for the UE-UE co-channel inter-subband CLI when only large-scale fading is considered.

Proposal 7: Reply to RAN1 that if both large-scale and small-scale fading are modelled for UE-UE co-channel channel model, the working assumption on the model on UE-UE co-channel inter-subband CLI is inline with RAN4’s understanding.

Proposal 8: Reply to RAN1 that the UE NF proposed in [3] model should be considered

- References

- R1-2302087, LS on interference modelling for duplex evolution, RAN1 #112

- TR 38.828 - Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR

- R4-2304538, Considerations on feasibility of UE aspects, Nokia, Nokia Shanghai Bell