3GPP TSG-RAN WG1 Meeting #106bis-e	R4-2304544

Electronic meeting, 17th – 26th April 2023

Agenda Item:	5.20.2.2.1

Source:	Ericsson

Title:	TP to TR 38.858 section 10.2 Feasibility of FR1 Wide Area BS aspects

Document for:	Approval

# 1	Introduction

During RAN4#106, a structure for the RAN4 part of TR 38.858 was agreed. Although several details remain open for the feasibility study and the co-existence simulations need to be carried out, nonetheless initial draft text can be provided for some sections.

Section editors were agreed for each section. It was also agreed that each company should submit text for each of the sections, and then that the editor would merge the texts.

In this TP, we provide some text for the section on FR1 feasibility. The TP presents proposals or wide area, medium range and local area, although we appreciate that the editors for these sections differ.

# 3	Text Proposal

## 10.2 Feasibility of FR1 Wide Area BS aspects

### 10.2.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

| FR1 (or FR2-1) |  |  |  | Ericsson |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS |  |  |
| BS TX Power dBPTX = ① dBm |  |  |  | 53 dBm |  |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc |  |  |
|  |  | Frequency isolation techniques used |  | Digital filtering, CFR, DPD |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 70 dBc |  |  |
|  |  | Spatial isolation techniques used |  | A combination of spatial isolation, chokes, absorption, mushroom EBG. 80dB can be achieved over a bandwidth of 100-200MHz and for certain beam directions. However, since the isolation needs to be achieved across the whole band and considering that depending on beam direction the isolation in fact varies from around 55dB to more than 80dB, 70dB is adopted as an average antenna isolation over a reasonable bandwidth. |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 dBc Note that the TX beam nulling reduces the variation due to beam direction, and hence spatial isolation + TX nulling can be assumed to be 80dB for most directions. |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Varies up to a maximum 3-5dB EIRP loss in DL, depending on TX beam direction |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -72 dBm |  |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc |  |  |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc |  |  |
|  |  | RF IC techniques and other tech. (before LNA) |  | Analogue interference cancellation incurs RX sensitivity loss due to insertion and also severe limitations on sub-band pre-coding and multi-carrier operation. Also, high routing complexity with large number of TX and RX. Filtering prior to the LNA would imply the need for the BS hardware to be specifically tuned to the SBFD carrier and no multi-carrier possibilities. Filter would be bulky to integrate into an AAS. Insertion loss would degrade sensitivity. Filtering in-between LNA stages could increase linearity, but still a large number of filters to incorporate multi-carrier configurations would not be feasible. Loss of integration would cause increases in size, energy etc. |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | >=5dBc if e.g. filtering or analogue IC would be applied. |  |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -27 dBm |  |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |  |  |
|  |  | Frequency isolation techniques |  | The RX input level is -27 dBm, and hence the receiver is in high non-linearity; no possibility for interference mitigation as part of the digital receive combining algorithms. Increasing IIP3 towards levels above +5dBm would necessitate designs with low levels of integration, increased energy consumption, thermal issues, size etc. |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -32dBm (Minimum for RAN4 requirement) -22dBm (Realistic for AAS) -10dBm (optimistic for AAS) |  |  |
|  |  |  | Rx IM3 contribution (dBm) | Even without ADC overload: -17 dBm (RAN4 minimum receiver) -37 dBm (Realistic) -61 dBm (Optimistic) |  |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | ADC could be overloaded; this can be mitigated with filtering prior to ADC except for direct conversion architectures. |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | Receiver in high non-linearity |  |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | RX processing does not mitigate analogue non-linearity |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | Receiver saturated |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | Digital IC not possible due to receiver non-linearity and would anyhow be highly complex due to large number of TX/RX for wide area. |  |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | Transmitter: 125 dB Receiver: N/A due to receiver saturation |  |  |
| Noise floor ⑩dBm |  |  |  | -96 dBm/CBW |  |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -102 dBm |  |  |
| Required RSIC budget (①-⑪dBc) |  |  |  | 155 dBc |  |  |
| SBFD configuration |  |  |  | 40-20-40 MHz |  |  |
| Guardband assumption (if exist) |  |  |  | 5 PRB. |  |  |
| bandwidth over which suppression is achieved |  |  |  | >300 MHz |  |  |
| Others |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |

### 10.2.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and analysis results.

### 10.2.3	Co-channel inter-sub-band inter-site interference analysis

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

For inter-site interference, it is assumed that the transmitter can suppress interference with at least the ACLR, and the receiver selectivity will be at least equivalent to the ACS.

Inter-site power, in particular in the transmit sub-band will be part of the total receive power at the receiver input and may have some impact to the receiver blocking / non-linearity effects.

### 10.2.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

## 10.3 Feasibility of FR1 Medium Range BS aspects

### 10.3.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results.

| FR1 (or FR2-1) |  |  |  | Ericsson |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Medium Range (3GPP minimum requirements) | Medium range (Realistic) | Medium Range (Optimistic RX) | Medium Range (Realistic, lower power) |
| BS TX Power dBPTX = ① dBm |  |  |  | 38 dBm | 38 dBm | 38 dBm | 35 dBm |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc | 45 dBc | 45 dBc |
|  |  | Frequency isolation techniques used |  | Digital filtering, CFR, DPD |  |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 65-70 dBc | 65-70 dBc | 65-70 dBc | 65-70 dBc |
|  |  | Spatial isolation techniques used |  | A combination of spatial isolation, chokes, absorption, mushroom EBG. 80dB can be achieved over a bandwidth of 100-200MHz and for certain beam directions. However, since the isolation needs to be achieved across the whole band and considering that depending on beam direction the isolation in fact varies from around 55dB to more than 80dB, 65-70dB is adopted as an average antenna isolation over a reasonable bandwidth. |  |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 10 dBc Note that the TX beam nulling reduces the variation due to beam direction, and hence spatial isolation + TX nulling can be assumed to be 80dB for most directions. |  |  |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | Varies up to a maximum 3-5dB EIRP loss in DL, depending on TX beam direction |  |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -87 dBm | -87 dBm | - 87 dBm | -90 dBm |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc | 0 dBc | 0 dBc | 0 dBc |
|  |  | RF IC techniques and other tech. (before LNA) |  | Analogue interference cancellation incurs RX sensitivity loss due to insertion and also severe limitations on sub-band pre-coding and multi-carrier operation. Also, high routing complexity with large number of TX and RX. Filtering prior to the LNA would imply the need for the BS hardware to be specifically tuned to the SBFD carrier and no multi-carrier possibilities. Filter would be bulky to integrate into an AAS. Insertion loss would degrade sensitivity. Filtering in-between LNA stages could increase linearity, but still a large number of filters to incorporate multi-carrier configurations would not be feasible. Loss of integration would cause increases in size, energy etc. |  |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | >=5dBc if e.g. filtering or analogue IC would be applied. |  |  |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -42 dBm | -42 dBm | -42 dBm | -45 dBm |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |  |  |  |
|  |  | Frequency isolation techniques |  | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. For RX, the 3rd column represents improved receiver linearity in the analogue domain. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. It is assumed that inter-sub band RX ACS is very large due to time alignment and achieving orthogonality between the TX and RX signals in the digital domain. |  |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -27.6 dBm | -17.6 dBm | -13 dBm | -17.6 dBm |
|  |  |  | Rx IM3 contribution (dBm) | -70.8 dBm | -90.8 dBm | -100 dBm | -100 dBm |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | No significant issues for medium range BS power level other than mentioned above. Phase noise reciprocal mixing is not significant for this frequency range and power levels. |  |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -70.8 dBm | -90.8 dBm | -100 dBm | -100 dBm |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 10-15 dBc (Transmitter) |  |  |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 109 dBc | 128 dBc | 135 dBc | 134 dBc |
| Noise floor ⑩dBm |  |  |  | -90 dBm/CBW | -90 dBm/CBW | -90 dBm/CBW | -90 dBm/CBW |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -96 dBm | -96 dBm | -96 dBm | -96 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | 134 dBc | 134 dBc | 134 dBc | 131 dBc |
| SBFD configuration |  |  |  | 40-20-40 |  |  |  |
| Guardband assumption (if exist) |  |  |  | 5 PRB |  |  |  |
| bandwidth over which suppression is achieved |  |  |  | <300MHz |  |  |  |
| Others |  |  |  | The conclusion does not take into account interference increase due to scattering effects, or the possibility for receiver algorihms to mitigate scattering, inter-sector and inter-site and self-interference (reference scenarios needed). |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |  |

### 10.3.2	Co-channel inter-sub-band co-site inter-sector interference analysis

Editor's note: This section captures the typical assumption of RF requirements and analysis results

### 10.3.3	Co-channel inter-sub-band inter-site interference analysis

Editor's note: This section captures the CLI modeling. As approved previously, ACLR and ACS value can be reused.

For inter-site interference, it is assumed that the transmitter can suppress interference with at least the ACLR, and the receiver selectivity will be at least equivalent to the ACS.

Inter-site power, in particular in the transmit sub-band will be part of the total receive power at the receiver input and may have some impact to the receiver blocking / non-linearity effects.

### 10.3.4	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

## 10.4 Feasibility of FR1 Local Area BS aspects

### 10.4.1	Self-interference analysis

Editor's note: This section captures the typical assumption based on which the RSIC capability is derived and analysis results

| FR1 (or FR2-1) |  |  |  | Ericsson (preliminary) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Local Area BS (3GPP minimum) | Local Area BS (Realistic RX) |  |
| BS TX Power dBPTX = ① dBm |  |  |  | 24 dBm | 24 dBm |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 45 dBc | 45 dBc |  |
|  |  | Frequency isolation techniques used |  | Digital filtering, CFR, DPD |  |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 70 dBc | 70 dBc |  |
|  |  | Spatial isolation techniques used |  | Physical distance, isolation structures |  |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 0 dBc | 0 dBc |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | TX beam nulling not assumed due to array size |  |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | -91 dBm | -91 dBm |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0 dBc | 0 dBc |  |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0 dBc | 0 dBc |  |
|  |  | RF IC techniques and other tech. (before LNA) |  | Analogue IC could be considered for this case, but is restrictive on pre-coding and multi-carrier. Digital IC has instead been assumed. |  |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | 0 dBc | 0 dBc |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  | -46 dBm | -46 dBm |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc |  |  |
|  |  | Frequency isolation techniques |  | Digital IC of TX. The impact of scattering / reflection in the environment has not been considered. It is assumed that RX ACS is very large due to time alignment and achieving orthogonality between the TX and RX signals in the digital domain. |  |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | -24.6 dBm | -14 dBm |  |
|  |  |  | Rx IM3 contribution (dBm) | -88.8 dBm | -110 dBm |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | No other significant impacts other than those mentioned above |  |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | -88.8 dBm | -110 dBm |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | TBC dBc RX beam nulling is in effect part of the digital baseband combining. Digital baseband combining may improve self-interference suppression, effectively at the cost of suppression of other interferers and RX beamforming gain. Reference scenarios are needed to assess the overall potential from RX baseband combining. |  |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  |  |  |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 15 dBc on transmitter |  |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 112 dBc | 124 dBc |  |
| Noise floor ⑩dBm |  |  |  | -87 dBm/CBW | -87 dBm/CBW |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | -93 dBm | -93 dBm |  |
| Required RSIC budget (①-⑪dBc) |  |  |  | 117 dBc | 117 dBc |  |
| SBFD configuration |  |  |  | 40-20-40, see annex |  |  |
| Guardband assumption (if exist) |  |  |  | 5 PRB, see annex |  |  |
| bandwidth over which suppression is achieved |  |  |  | <300MHz |  |  |
| Others |  |  |  | The conclusion does not take into account interference increase due to scattering effects, or the possibility for receiver algorihms to mitigate scattering, inter-sector and inter-site and self-interference (reference scenarios needed). |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |

### 10.4.2	Co-channel inter-sub-band inter-site interference analysis

This section captures the typical assumption of RF requirements and analysis results

### 10.4.3	Summary

Editor's note: This section captures the conclusion of BS SBFD feasibility.

For inter-site interference, it is assumed that the transmitter can suppress interference with at least the ACLR, and the receiver selectivity will be at least equivalent to the ACS.

Inter-site power, in particular in the transmit sub-band will be part of the total receive power at the receiver input and may have some impact to the receiver blocking / non-linearity effects.