3GPP TSG-RAN4 Meeting #106-bis-e	R4-2304567

# Online, 17th – 26th April 2023

Source:				Ericsson

Title:					NTN enhancements – System parameters

Agenda item:		5.27.1.1

Document for:		Approval

# Introduction

This document discusses the system parameters and bands definition in the scope of the NTN enhancements WI ([1]), based on the agreed Way Forward [2] from last RAN4#106 meeting.

# Discussion

## Bands definition

The following bands were agreed in last RAN4#106 meeting:

Table 1: NTN Ka-bands definition

| NTN operating band | UL Earth-to-Space | DL Space-to-Earth |
| --- | --- | --- |
| n5121 | 27.5 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5112 | 28.35 - 30.0 GHz | 17.3 - 20.2 GHz |
| n5103 | 27.5 - 28.35 GHz | 17.3 - 20.2 GHz |
| Note 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 and ECC Decision (13)01. Note 2: This band is applicable in the USA subject to FCC 47 CFR part 25. Note 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). |  |  |

## NTN FR range

In last RAN4#106 meeting, RAN4 had some discussion related to the frequency range definition for NTN satellite operations and the following way forward was captured ([2]):

There is indeed a need to define (or redefine) the frequency range where the SAN and NTN satellite UE RF requirements  will be applicable. Also, it was agreed that, whatever frequency range is defined in the scope of this WI, this range will only be applicable to NTN satellite specifications, i.e. TS 38.108, TS 38.181, TS 38.101-5 and TS 38.521-5. And, to not impact other RAN WGs, the new frequency range(s) will be regarded as a TN FR2 in those WGs, as stated in the note below option 2’s table.

Considering the new frequency range(s)’s definition will not be spread outside TS 38.108, TS 38.181, TS 38.101-5 and TS 38.521-5, it might be better to define a frequency range specific to NTN satellite that covers the considered band for NTN satellite and for which the RF requirements specified in those specifications are applicable.

We would make the following proposal:

Proposal1: Define below frequency range restricted to the scope of NTN satellite specifications:

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR2-NTN | 17300-30000 MHz |

Observation1: NTN includes satellite and HAPS operations, it should be clarified this frequency range is for satellite operations only.

Observation2: This frequency range boundaries could be updated later when introducing a new NTN satellite band not included in this range and for which the specified RF requirements are also applicable.

## Channel arrangement

### Channel raster

Based on the agreed NTN satellite bands definition, the NR-ARFCN values shall be calculated based on the following Table 2 and formula:

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 2: NR-ARFCN parameters for the global frequency raster (extract from TS 38.104)

|  | Range of frequencies (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- | --- |
| Downlink (17.3-20.2 GHz) | 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |
| Uplink (27.5-30.0 GHz) | 24250 – 100000 | 60 | 24250.08 | 2016667 | 2016667 – 3279165 |

We then make the following proposal:

Proposal2: Specify the following channel raster NREF values for bands n512, n511 and n510.

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | [60] | 2070833 – <1> – 2112499 | 1553334 – <1> – 1746666 |
|  | [120] | 2070833 – <2> – 2112499 | 1553335 – <2> – 1746665 |
| n511 | [60] | 2084999 – <1> – 2112499 | 1553334 – <1> – 1746666 |
|  | [120] | 2084999 – <2> – 2112499 | 1746665 – <2> – 1746665 |
| n510 | [60] | 2070833 – <1> – 2084999 | 1553334 – <1> – 1746666 |
|  | [120] | 2070833 – <2> – 2084999 | 1746665 – <2> – 1746665 |

Note that 60 and 120 kHz SCS are kept in [] as RAN4 RRM has not made yet any conclusion on the feasibility or not of those sub-carriers spacing for the considered bands ([3]).

### Synchronization raster

The three new NTN bands are specified with the same frequency range in downlink 17.3-20.2 GHz. They will then all have the same synchronization raster definition.

The GSCN values shall be calculated based on the following Table 3:

Table 3: GSCN parameters for the global frequency raster (extract from TS 38.104)

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 3000 – 24250 | 3000 MHz + N * 1.44 MHz, N = 0:14756 | 7499 + N | 7499 – 22255 |

We then make the following proposal:

Proposal3: Specify n512, n511 and n510 synchronization raster entries according to following table:

| NTN operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17441 – <1> – 19432 |
|  | 240 kHz | Case E | 17454 – <2> – 19420 |
| n511 | 120 kHz | Case D | 17441 – <1> – 19432 |
|  | 240 kHz | Case E | 17454 – <2> – 19420 |
| n510 | 120 kHz | Case D | 17441 – <1> – 19432 |
|  | 240 kHz | Case E | 17454 – <2> – 19420 |

# Conclusion

In this contribution, we discussed the system parameters to be used for the NTN satellite bands agreed in last RAN4 meeting. We made the following proposals and observations:

Proposal1: Define below frequency range restricted to the scope of NTN satellite specifications:

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR2-NTN | 17300-30000 MHz |

Observation1: NTN includes satellite and HAPS operations, it should be clarified this frequency range is for satellite operations only.

Observation2: This frequency range boundaries could be updated later when introducing a new NTN satellite band not included in this range and for which the specified RF requirements are also applicable.

Proposal2: Specify the following channel raster NREF values for bands n512, n511 and n510.

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | [60] | 2070833 – <1> – 2112499 | 1553334 – <1> – 1746666 |
|  | [120] | 2070833 – <2> – 2112499 | 1553335 – <2> – 1746665 |
| n511 | [60] | 2084999 – <1> – 2112499 | 1553334 – <1> – 1746666 |
|  | [120] | 2084999 – <2> – 2112499 | 1746665 – <2> – 1746665 |
| n510 | [60] | 2070833 – <1> – 2084999 | 1553334 – <1> – 1746666 |
|  | [120] | 2070833 – <2> – 2084999 | 1746665 – <2> – 1746665 |

Proposal3: Specify n512, n511 and n510 synchronization raster entries according to following table:

| NTN operating band | SS Block SCS | SS Block pattern | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17441 – <1> – 19432 |
|  | 240 kHz | Case E | 17454 – <2> – 19420 |
| n511 | 120 kHz | Case D | 17441 – <1> – 19432 |
|  | 240 kHz | Case E | 17454 – <2> – 19420 |
| n510 | 120 kHz | Case D | 17441 – <1> – 19432 |
|  | 240 kHz | Case E | 17454 – <2> – 19420 |

# Reference

- RP-230809, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2302998, WF for system parameters of above 10 GHz NTN band, Thales

- R4-2301465, NTN enhancements – System parameters and bands definition, Ericsson