3GPP TSG-RAN WG4 Meeting #106-bis-e 	R4-2304596

E-meeting, April 17th – 26th, 2023

Agenda Item: 5.11.2.3

Source: MediaTek inc.

Title: Discussion on BWP operation without BW restrictions – Option C

Document for: Discussion and decision

# Introduction

RAN plenary discussed the topic BWP operation without restrictions in Meeting RAN#99 and defined a new WI with objectives captured below [1]:

## Objective ### Objective Core part WI ### The work item includes following objectives: - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1)

Therefore, in this contribution paper, we provide analysis and discussion on the completeness of Option C (NCD-SSB).

# Discussion on RRM requirements impact/workload in RAN4

The existing RAN4 requirements specify that the UE is not required to perform BM/RLM/BFD outside the active BWP. However, in the previous RAN plenary #99, a new WI was introduced to allow the UE to perform L1 (BM/RLM/BFD) measurements based on NCD-SSB. Such scenario was discussed back in Rel-17 RedCap WI and it was agreed that existing UE requirements based on CD-SSB can be reused for 2Rx RedCap UE with NCD-SSB. Besides, the timing requirements can be directly applied with the use of NCD-SSB as captured in Rel-17 RedCap. This has been captured in TS 38.133 for RLM, BFD and timing requirements as follows:

## 8.1B Radio Link Monitoring for RedCap ### 8.1B.1 Introduction The requirements in clause 8.1B apply for radio link monitoring on UE with reduced capabilites: - PCell in SA NR operation mode, The UE shall monitor the downlink radio link quality based on the reference signal configured as RLM-RS resource(s) in order to detect the downlink radio link quality of the PCell as specified in TS 38.213 [3]. The configured RLM-RS resources can be all SSBs, or all CSI-RSs, or a mix of SSBs and CSI-RSs. UE is not required to perform RLM outside the active DL BWP. The SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified.

## 8.5B Link Recovery Procedures for Redcap ### 8.5B.1 Introduction ## The Redcap UE shall assess the downlink radio link quality of a serving cell based on the reference signal in the set as specified in TS 38.213 [3] in order to detect beam failure on PCell in SA. The SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified.

## 7.1A UE transmit timing for RedCap ### 7.1A.2 Requirements The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1A.2-1. This requirement applies: - when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission. - when it is the transmission for PUSCH on CG resources for SDT in RRC_Inactive. The UE shall meet the Te requirement for an initial transmission provided that at least one SSB (CD-SSB or NCD-SSB) is available at the UE for acquiring the frame timing of the reference cell during the last 160 ms on the condition that: - the SSB is within the UE’s active BWP, or - the SSB is not within the UE’s active BWP, and the measurement gap is configured.

Now, for the case of active BWP without CD-SSB but with the NCD-SSB is contained within the active BWP, the existing requirements in RAN4 for BM/RLM/BFD measurements can be directly applied with no changes.

- the requirements in sections 8.1, 8.5, and 7.1 for RLM, BFD and UE transmit timing are applicable with NCD-SSB in active BWP with no changes required.

However, RAN4 should further study the impact on L3 measurements. For example, whether the intra-frequency L3 measurement should also be done on the same NCD-SSB or UE still needs to go back to CD-SSB for L3 measurements. In the latter case, RAN4 may have to open the discussions on how the measurement is done, e.g., via MG, NCSG or measured with/without interruption. In our view, it is better to align the SSB used for L1 measurement and intra-frequency L3 measurements. This simplified not only the UE behaviour but also the spec change (the workload). RAN4 can check whether a LS is needed to inform RAN1 and RAN2 for potential changes in their specs.

- For UE supporting Option C, FFS whether the UE is expected to perform intra-frequency L3 measurements on the same NCD-SSB. FFS whether an LS is needed to RAN1 and RAN2.

# Summary

In this contribution, we have the following proposal:

Proposal 1: the requirements in sections 8.1, 8.5, and 7.1 for RLM, BFD and UE transmit timing are applicable with NCD-SSB in active BWP with no changes required.

Proposal 2: For UE supporting Option C, FFS whether the UE is expected to perform intra-frequency L3 measurements on the same NCD-SSB. FFS whether an LS is needed to RAN1 and RAN2.

# References

- RP-230805, “New WID: Complete the specification support for BandWidth Part operation without restriction in NR”, Vodafone, vivo, Rotterdam, Netherlands, March 2023.