3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304612

# Online, April 17 – April 26, 2023

Source:	LG Electronics

Title:	UE Tx requirement for inter-band CA/EN-DC PC1.5 with 3Tx

Agenda item:	4.29.3.1

Document for:	Discussion

Introduction

In this paper, we provide our views on UE maximum output power and configured transmitted power for inter-band CA/EN-DC PC1.5 with 3Tx.

Discussion

Based on the WID[2],  inter-band CA/EN-DC power class 2 and power class 1.5 are targeted.

- CA power class or EN-DC power class is PC2 - PC3 FDD band 1Tx + PC2 TDD band 2Tx (UL MIMO and TxD) - PC3 FDD band 1Tx + PC3 TDD band 2Tx (UL MIMO) - PC3 TDD band 1Tx + PC2 TDD band 2Tx (UL MIMO) - CA power class or EN-DC power class is PC1.5 - PC3 FDD band 1Tx + PC1.5 TDD band 2Tx (UL MIMO and TxD)

According to the existing requirements, power class 1.5 is not specified for inter-band CA/EN-DC.

Intuitionally, there can be issues for UE maximum output power and Configured output power for the inter-band CA/EN-DC PC1.5 with 3Tx due to the difference of supported power class.

UE maximum output power (inter-band CA/EN-DC PC1.5 with 3Tx)

For the existing inter-band CA,

PC2-related UE capabilities apply to the requirement of UE maximum output power, such as,

- maxUplinkDutyCycle-interBandCA-PC2

- maxUplinkDutyCycle-PC2-FR1,.

For the existing inter-band EN-DC,

PC2-related UE capabilities apply to the requirement of UE maximum output power, such as,

- maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 for TDD-TDD EN-DC

- maxUplinkDutyCycle-FDD-TDD-EN-DC1/2 for FDD-TDD EN-DC

For inter-band CA/EN-DC PC1.5 with 3Tx,

whether these PC2-related UE capabilities can apply to or not should be discussed. If not applied, corresponding UE capabilities need to be considered.

Proposal 1: Study whether the existing UE capabilities can be applied or new UE capabilities are needed for inter-band CA/EN-DC PC1.5 with 3TX.

Configured transmitted power (inter-band CA/EN-DC PC1.5 with 3Tx)

For the existing inter-band CA, the configured transmitted power is specified by considering up to PC2.

- ΔPPowerClass,CA = 3 dB for a power class 2 capable UE

- 0dB otherwise

For the existing inter-band EN-DC, the configured transmitted power is specified by considering up to PC2.

- ΔPPowerClass,EN-DC = 3 dB for a power class 2 capable UE

- 0dB otherwise

For inter-band CA/EN-DC PC1.5 with 3Tx, the configured transmitted power needs to be defined by considering up to PC1.5.  For example,

- ΔPPowerClass,EN-DC = 6 dB for a power class 1.5 capable UE

- ΔPPowerClass,EN-DC = 3 dB for a power class 2 capable UE

- 0dB otherwise

Proposal 2: Define configured transmitted power by considering up to PC1.5. for inter-band CA/EN-DC PC1.5 with 3Tx.

Conclusion

In this contribution, we provided our view on UE maximum output power and configured transmitted power for inter-band CA/EN-DC PC1.5 with 3Tx. The followings are proposed.

Proposal 1: Study whether the existing UE capabilities can be applied or new UE capabilities are needed for inter-band CA/EN-DC PC1.5 with 3TX.

Proposal 2: Define configured transmitted power by considering up to PC1.5. for inter-band CA/EN-DC PC1.5 with 3Tx.

- References

[1] R4-2303523, “WF on 3Tx requirements”, Oppo

[2] RP-230161, “Revised WID for Low NR band 4Rx and 3Tx”, Oppo