3GPP TSG-RAN WG4 Meeting # 106 bis-e									    		                 R4-2304615

Electronic Meeting, 17th – 26th April, 2023

Agenda item:			5.27.2

Source:	Samsung

Title:	Simulation assumptions and preliminary co-existence study for above 10GHz NTN co-existence study

Document for:	Discussion

- Introduction

According to the Rel-18 NR NTN enhancement WID, RAN4 has been mandated to study and identify NTN example band in above 10 GHz.

The following assumptions are taken a baseline for this work:

- GSO and NGSO (e.g. LEO, MEO, HEO) based satellite access to be considered

- ESIM scenarios for NGSO in Ka band are not considered in this WI.

- Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified

- Regarding mobile VSAT, three types of terminals and scenarios exist; airborne, maritime and land based ESIM. Which type(s) to be specified depends on the outcome of the regulation analysis and co-existence study.

- FDD mode is assumed for satellite operation above 10 GHz, while TDD mode is assumed for terrestrial operation in FR2

- The ITU-R harmonized Ka band will serve as reference

- Co-existence between overlapping NTN and TN band portions is out of scope of this work item. This aspect will be captured in the specification.

For co-existence studies, following considerations should be taken into account:

- Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow.

- For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands.

From the RAN4 #104-bis-e to 106 meeting, agreements on some assumptions have been achieved. And the remaining issues have been discussed in this paper.

- Discussion

Network layout model

In orde to evaluate the worst situation of each scenario, to drop the NTN UE inside of TN network would much better, which has been reflected in the WF[1] annex 2. On the other hand, the deployment rules in annex 1 is the baisc deployment rules for NTN coexistance study.

Proposal 1: Consider the deployment an observation rules as listed in Annex 1 (when NTN UEs are dropped at TN cluster edges) and Annex 2 (when NTN UEs are dropped in TN) in the coexistence study.

Simulation parameters

NTN parameters

NTN UE parameters

To assume fixed tilt values for the NTN UE is a simplifed way for the coexisetence study. For the NTN UE in the same serving area of a NTN SAN, the up-tilt of the Ues would be similar no matter the connected satellite is a GEO or LEO satellite, because of the height of the satellite is much larger than the covered area diameter. Therefore, the different of up-tilt of each NTN UE in this area can be ignored, but random tilt value is not reasonable. At least one snapshot during  the simulation, the up-tilt of NTN UE should be similar or same. Another reasonable assumption for the tilt value is to separate the LEO and GEO NTN to model the tilt: For the GEO, the tilt value would be a fixed value.

Proposal 2: Propose following table as the horizontal boresight and vertical tilt assumptions of NTN UEs.

|  | GEO | LEO |
| --- | --- | --- |
| Antenna vertical tilt | 20 degree above horizontal | 30 degree above horizontal |
| Antenna horizontal boresight | Following Serving satellite or random between (0-360) | Following Serving satellite or random between (0-360) |

Proposal 3: Propose following table as the Parabolic Antenna for NTN UE assumption

| Characteristics |  |
| --- | --- |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Efficiency |  |
| Rx Antenna gain | 39.7 dBi |
| Noise figure | 1.2 dB |
| Rx Feeder loss | -0.5dB |
| Antenna temperature | 150 K |
| Sky temperature | N/A |
| Ground temperature | N/A |
| G/T figure of merit | 15.35 (dB/K) (Note 2) |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 43.2 dBi |
| Output loss | N/A |
| EIRP | 45.2 dBW |
| UE height | FFS |
|  | NOTE 1: VSAT terminal characteristics could be implemented with phased array antenna NOTE 2: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) Where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain) - Noise Figure: NF in dB |

Proposal 4: Propose following table as the Phased Array Antenna for NTN UE assumptions

| Characteristics | ESIM (TR 38.821) | ESIM (TR 38.821),CATT | ESIM (ITU) |
| --- | --- | --- | --- |
| Antenna type and configuration | Directional (M,N,P,Mg,Ng) = (128/64,128/32,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=65/90 deg) | Directional (M,N,P,Mg,Ng) = (128,128,2,1,1); (dV,dH) = (0.5, 0.5)λ with directional antenna element (HPBW=90 deg) | Antenna pattern: S.580-6 |
| Polarisation | Linear: +/-45°X-pol | Linear: +/-45°X-pol |  |
| Element gain including loss | 5.5dBi | 3 dBi per element |  |
| Rx Antenna gain | 45.3dBi/ 3dBi per elemant | 45dBi(per polarization) | 45 dBi |
| Antenna temperature | 150 K | 150 K |  |
| Noise figure | 1.2 dB/ 11/13dB | 1.2 dB/ 11/13dB |  |
| Tx transmit power | Transmit power density（dBW/Hz）-46.3/ 10*log10(64*32)+7 dBm) | 10*log10(128*128)+7 dBm | Transmit power density（dBW/Hz）-46.3 |
| Tx antenna gain | 45.3dBi (per polarization)/ 3 dBi per element | 45dBi(per polarization) | 45 dBi |

Calibration

In order to obtain believable study results, calibration is necessary for this Ka-band NTN coexistence study. From the 2GHz study experience [2] the coupling loss and geometry SINR are the two basic values that should be calibrated in order to algin the simulators. Therefore:

Proposal 5: Propose RAN4 to start the calibration for the Ka-band NTN coexistence study. Coupling loss and geometry SINR at CDF percentile 5%, 50% and 95% in different scenarios should be calibrated.

Co-existence study

This study provided a preliminary simulation result for the NTN UL interfered with TN DL in the worst situation. Simulation assumption is based on the latest agreement of RAN4. The result shown below:

For the average throughput loss, 7dB ACIR is needed to achieve the protection criteria.

For the 5% edge UE throughput loss, 27dB ACIR is needed to achieve the protection criteria.

- Conclusion

Proposal 1: Consider the deployment an observation rules as listed in Annex 1 (when NTN UEs are dropped at TN cluster edges) and Annex 2 (when NTN UEs are dropped in TN) in the coexistence study.

Proposal 2: Propose following table as the horizontal boresight and vertical tilt assumptions of NTN UEs.

Proposal 3: Propose following table as the Parabolic Antenna for NTN UE assumption

Proposal 4: Propose following table as the Phased Array Antenna for NTN UE assumptions

Proposal 5: Propose RAN4 to start the calibration for the Ka-band NTN coexistence study. Coupling loss and geometry SINR at CDF percentile 5%, 50% and 95% in different scenarios should be calibrated.

- Reference

[1] R4-2302878 WF for above 10GHz NTN-TN co-existence study

[2] TR 38.863 Non-terrestrial networks (NTN) related RF and co-existence aspects

- Annex – Coexistance study assumptions

- Scenario NTN UL interfere with TN UL

- NTN Satellite parameters

| Satellite orbit |  | GEO |
| --- | --- | --- |
| Satellite altitude |  | 35786 km |
| Satellite EIRP density | Ka-band (i.e. 20 GHz for DL) | 40 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi |
| Channel bandwidth |  | 200 |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  | 0.1765 deg |
| Satellite beam diameter |  | 110 km |
| G/T | Ka-band (i.e. 30 GHz for UL) | 28 dB K-1 |
| Satellite Rx max Gain |  | 58.5 dBi |

| Parameters | NTN | Remark |
| --- | --- | --- |
| Carrier frequency | 30GHz(UL)/20GHz(DL) |  |
| The number of active UE (UL) | 9 UEs and 2RBs per UE for GEO and LEO1 |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 2.6.2 |  |
| NTN satellite Noise figure in dB | See Table 2.3.1-3 |  |
| Handover margin | 3dB |  |
| Note 1: UEs are equally splitted inside the channel bandwidth into ACIR 3 regions. Scheduled PRB position for UE1 per satellite beam should be also fully aligned to simulate the worst case for co-channel interference and this is also aligned with full buffer case. |  |  |

| Satellite | GEO |
| --- | --- |
| G/T (dB K-1) | 28 |
| G_Rx (dBi) | 58.5 |
| NF (dB) | 5.9 |

- NTN UE parameters

| Characteristics | VSAT (Note 2) |
| --- | --- |
| Frequency band | Ka band(i.e. 30 GHz UL and 20 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | circular |
| Rx Antenna gain | 39.7 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 43.2 dBi |

| Carrier frequency in GHz | 30 |
| --- | --- |
| Transmission bandwidth in MHz | 200 |
| Environment | Urban macro |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance in meter | 200m |
| Number of micro BSs per macro cell |  |
| System loading and activity | See Annex 2 |
| Network location | See Annex 2 |
| DL subcarrier spacing | 120kHz |
| UL | OFDMA |
| DL power control | No |
| UL power control | See Section 2.7.1, 2.7.2 |
| Frequency reuse | 1 |
| Number of scheduled UE per cell (DL) | Same as the number of BS beam |
| Number of scheduled UE per cell (UL) | Same as the number of BS beam |
| UE antenna height in meter | Same as 3D-UMi in TR 36.873 |
| UE TX power in dBm | -40 to 23 |
| UE antenna gain in dBi | 0 |
| Building penetration loss | In path loss model, TR 38.901 |
| Cell selection margin in dB | 3 |
| BS-MS min distance in meters | 35 |
| BS noise figure in dB | 11 |
| UE noise figure in dB | 9 |
| BS-UE path-loss model | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |
| Link-level performance model | See Section 2.10 |
| UE distribution | Uniform |
| Evaluation metrics | See Section 2.9 |

|  | Urban Macro |
| --- | --- |
| ISD in meters | 200 |
| BS Antenna height in meters | 25 |
| UE Outdoor/indoor | 100% Outdoor |
| UE height in meter | 1.5 |

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 28dB |
|  | ACS | 24dB |
| UE | ACLR | 17dB |
|  | ACS | 22dB |

- TN BS and UE antenna and beam forming pattern modelling

Refer to TR 38.803

- Propagation model

38.901-UMa

- Transmission power control model

  - TN UL TPC

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

  - NTN UL TPC

For calibration purpose, reuse the same TN TPC for NTN with SNR target 15dB.

For the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted but needs to revise CLx-ile to align with UE UL power control parameters used in TR38.821. The CLx-ile value should be adapted for rural scenario.

  - DL TPC

For downlink scenario, no power control scheme is applied.

  - Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where,

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

- Performance metric

For NR,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

For NTN,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

- Throughput ~ SNR mapping

Adopt Section 5.2.7 of TR 38.803 as the SINR-Throughput performance metrics