3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2304617

Electronic Meeting, April 17 – April 26, 2023

Agenda Item:	5.26.2.2

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for MUSIM gaps collision handling

Document for:	Discussion

# Introduction

In NR Rel-17 specification, RAN4 has introduced gap patterns particularly for MUSIM purpose. However, their corresponding RRM requirements were not specified. In the last meeting, RAN4 has discussed the RRM requirements for MUSIM gaps collision handling as in the WF [1] which are classified under three cases:

- Collision between different MUSIM gaps

- Collision between MUSIM and legacy gaps

- Collision between MUSIM gaps and NW A signals

In this paper, the requirements for handling MUSIM gap collisions for the above cases are further discussed and our view is also provided.

# Discussion

## MUSIM gap priority configuration

Based on the current signaling framework, UE can request from NW A up to four MUSIM gap patterns (three periodic and one aperiodic as defined in Table 9.1.10-1 of 38.133 [2]). In RAN4#105 meeting, it was agreed to introduce different priority for each periodic MUSIM gaps as shown below:

RAN4#105 Agreement: - Agreements - Introduction of priorities for MUSIM gaps - Each periodic MUSIM gap can be assigned with a different priority - FFS whether aperiodic MUSIM gap shall be assigned with a priority level - FFS on relation between MUSIM priority level and priority levels for other MGs

Also, in the last meeting (RAN4#106), RAN4 made the following agreement:

Issue 2-1-2: Priority/usage indication on MUSIM gaps from UE side - Agreements - UE can optionally indicate its preferred priority for all or a subset MUSIM gaps - It is up to NW A on how to use this information Recommended WF - Focus and discuss how UE “UE can optionally indicate its preferred priority for all or a subset MUSIM gaps”

Since RAN4 has already agreed to introduce different priority for each periodic MUSIM gap, and UE can indicate its preferred priority for these gaps, then this can be designed simply by introducing a new parameter in the assistance-information that is used by the UE to request MUSIM gap configurations from NW A. This means UE can request (offset, MGL, MGRP and MUSIMGapPriority) from NW A, as shown in Figure 1, then NW A can accept the request when the offset, MGL and MGRP can be configured as requested by the UE. The requested MUSIMGapPriority for MUSIM can be designed using 2-bis to allow the UE suggesting priorities for 3 periodic MUSIM gaps.

Figure 1: Assistance information for MUSM gap.

Proposal 1: introduce a new parameter (MUSIMGapPriority) in the assistance-information along with the existing parameters used by the UE to request MUSIM gap configuration from NW A (i.e., offset, MGL, MGRP and MUSIMGapPriority).

Proposal 2: The requested priority (MUSIMGapPriority) for MUSIM can be designed using 2-bis to allow the UE suggesting priorities for 3 periodic MUSIM gaps.

Other remaining issues from the WF [1] are captured below:

Issue 2-1-4: Constraints on MUSIM gap priority configuration from NW A - Proposals - When MUSIM gaps’ priority are up to NW-A configuration - P1: NW A maintaining the same relative priorities requested by the UE (Qualcomm vivo MTK) - P1a: If UE requests two MUSIM gaps with the same priority X and if the network configures both gaps, then both gaps must be assigned a common priority X’. X’ may or may not be equal to X. (Qualcomm) - P1b: If UE requests MUSIM gap1 with priority X1 and MUSIM gap2 with priority X2, where X1 > X2, and if network A configures both gaps, then both gaps must be assigned priorities X1’ and X2’ such that X1’ > X2’. X1’ may or may not be equal to X1. X2’ may or may not be equal to X2. (Qualcomm) - P3: NW A could allocate higher priority for MUSIM gaps with longer MGRP (vivo) - P4: NW A treat the MUSIM gaps with the highest/second highest priority indicated by UE as aperiodic MUSIM gap or MUSIM gap for paging purpose (implicitly indicated); NW A could configure relative higher priority for these MUSIM gaps (vivo) - P5: MUSIM paging gap and aperiodic gap can have higher priority than NW-A’s MGs (Ericsson) - P6: 1 single priority applicable for all periodic MUSIM gaps. 1 priority for each aperiodic MUSIM gap. Aperiodic MUSIM gaps can be assigned with different priorities to the priority of the periodic MUSIM gaps (Nokia) Recommended WF Continue discussion

For Issue 2-1-4, although it is up to NW A how to configure the priority levels for the requested MUSIM gaps, the relative order of priority requested by the UE should be maintained as shown in Figure 2. We tend to believe this is a middle ground to reserve some configuration flexibility for network A and to minimize the changes to UE’s original request.

Figure 2: MUSIM gaps priority configuration with respect to MGs in NW A.

Proposal 3: NW A maintaining the same relative priorities requested by the UE.

Issue 2-1-5: Priority setting for aperiodic MUSIM gaps - Proposals - P1: When collides with legacy measurement gaps or MUSIM gaps, aperiodic gap shall be kept (Apple ZTE oppo vivo Huawei Ericsson) - P2: Prefer to allocate priority level for aperiodic MUSIM gap (Charter xiaomi ZTE vivo Qualcomm Nokia) - P3: No need to assign priority of aperiodic MUSIM gap (Apple Huawei Ericsson ZTE) - P4: It is not mandatory to assign priority for an aperiodic MUSIM gap and the highest priority is assumed by default (oppo MTK)

For Issue 2-1-5, aperiodic MUSIM gap is a one-shot gap in NW B and it should have some sort of high priority. Therefore, in our view, aperiodic MUSIM gap can be prioritized by default whenever colliding with other gaps. This means there is no need to introduce priority level for aperiodic MUSIM gap (both NW and UE can have the same understanding on how to handle aperiodic MUSIM gap).

Proposal 4: Aperiodic MUSIM gap can be prioritized by default and no need to introduce priority level for it.

## Case 1: On collision between different MUSIM gaps

For collision between different MUSIM gaps, the following issues are captured below:

Issue 2-2-1: Definition of the collision between different MUSIM gaps - Proposals - Option 1: The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap when priority rules are used to handle the collision between MUSIM gaps (Apple CMCC ZTE oppo xiaomi vivo MTK) - Option 2: No definition for collisions between MUSIM gaps is needed. (Huawei Nokia) - Option 3: No collisions between MUSIM gaps that have the same priority level (Qualcomm). Recommended WF Continue discussion

The same principle used to define the collision in Rel-17 between two concurrent gaps is also applicable to define the collision between different MUSIM gaps (i.e., support Option 1).

Proposal 5: The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap.

Issue 2-2-2: Solutions for collision between different MUSIM gaps - Proposals - Option 1: Priority based solution is used for collision between different MUSIM gaps (Apple oppo vivo MTK) - Option 1a: Priority based solution is used for collision between different MUSIM gaps, if multiple MUSIM gaps are assigned different priority levels (Qualcomm) - Option 2: Keep solution (keep all collided MUSIM gap) is used when different MUSIM gaps collide (Huawei) - Option 2a: Keep solution is used under particular conditions (xiaomi vivo oppo Ericsson Qualcomm) - Option 3: Consider combine both option 1 and 2 as the solution (ZTE) - Option 3a (ZTE): - The aperiodic gap has higher priority than other periodic gaps, the priority handling rule shall be used if it collides with the periodic gaps (except the paging gap). - The paging gap should not be dropped, the kept/merged solution is used if the second gap is paging gap. - Otherwise, the priority handling rule will be used among MUSIM gaps. - Option 4: Collision between periodic and aperiodic MUSIM gaps are handled by priorities (Nokia) Recommended WF Continue discussion

For Issue 2-2-2, priority-based solution in Option 1 should be sufficient to handle collisions between different MUSIM gaps. It is also the essential mechanism to handle the collision between MUSIM gaps and non-MUSIM gaps. It is a single and simple solution to resolve the collision of different scenarios. All other potential enhancement should only be discussed on top of Option 1. In other words, RAN4 should only introduce other solutions if RAN4 identifies an important scenario that cannot be handled by Option 1.

On the other hand, the use of keep solution in Option 2/3 could lead to significant performance degradation in NW A since long MGL gaps could result in as explained below.

Assume a UE is configured with 3 parodic MUSIM gaps (as defined in Table 9.1.10-1 of 38.133) such as:

- Id#0: MGL = 6ms, MGRP = 40ms

- Id#12: MGL = 10ms, MGRP = 80ms

- Id#13: MGL = 20ms, MGRP = 160ms

If no dropping is applied for MUSIM gaps collision as proposed in Option 2, the scenario shown in Figure 3 could occur.

Figure 3: Collision between different MSUIM gaps with no dropping rule.

As it can be noticed from Figure 2, the usage of the time resources for MUSIM purpose could jump up to 40% if MUSIM gaps are not dropped (where less than 15% could be the usage if priority rule is introduced among MUSIM gaps). This 40% of time occupancy by MUSIM is considered high especially when NW B is in IDLE/INACTIVE state and NW A is in the CONNECTED state. In addition, the MGL of the MUSIM gap resulting in from this approach could be doubled (e.g., from 20ms MGL to 46ms MGL), which increases the interruption time impact on NW A.

For Option 3, it is not straightforward as it can increase UE complexity to apply different rules based on different scenarios (in addition to the same concerns on Option 2).

Proposal 6: Priority based solution should be sufficient to handle collisions between different MUSIM gaps.

## Case 2: On collision between MUSIM and legacy gaps

Collision between MUSIM gaps and legacy gaps can be discussed in two cases:

- Collision between MUSIM gap and Type-1 MG

- Collision between MUSIM gap and Type-2 MG

Type-1 MG is the legacy gap configured via GapConfig without suffix, and Type-2 MG is the legacy gaps configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17. Note that Type-1 MG has no priority nor association, whereas Type-2 MG has both.

For the collision between MUSIM gap and Type-2 MG, RAN4 has already agreed that (in R4-2214349 of RAN4 #104) priority-based gap collision handling introduced in concurrent gaps design can be used as a baseline for collisions between MUSIM gap and Type -2 MG. We believe this solution is sufficient and no need to discuss other solutions on top of it.

On the other hand, the collision between MUSIM gap and Type-1 MG is still an open issue as captured below in the WF:

Issue 2-3-2: Solutions for collision between MUSIM gap and Type-1 MG or gap configured without priority - Proposals - P1: When a MUSIM gap collides with a legacy MG, requirements shall not apply if any one of the collided gaps is not assigned a priority. (Huawei vivo Nokia) - P2: MUSIM gaps are assumed to have higher priority than a Type-1 MG when either MUSIM gaps or Type-1 MG (or both) are not assigned priorities by the network. (Qualcomm) - P3: Collision is be handled based on the MGRP of the collided gaps (Ericsson MTK) - P3-1: Prioritize the gap with longer MGRP for the following MUSIM collision scenarios (Ericsson) - Any of the collision gaps is Type-1 MG; - NW-A doesn’t configure a priority associated with any of the collision gaps - P4: The sharing rule solution could be considered. (xiaomi) Recommended WF FFS on solutions for this issue.

Collisions between MUSIM gaps and Type-1 MG cannot be handled in the same way as the collision with Type-2 MG since Type-1 MG is a single gap and has no priority level. Therefore, three possible solutions can be adopted to handle this collision:

- Sol #1: Always prioritize MUSIM gaps over Type-1 MG.

- Sol #2: Always prioritize Type-1 MG over MUSIM gaps.

- Sol #3: Fair solution based on MUSIM gaps and Type-1 MG configurations.

Sol #1 and Sol #2 do not provide fairness in handling the collision between MUSIM gaps and Type-1 MG. One can always argue that MUSIM gap can be more important than Type-1 MG (e.g. receiving paging in NW B), or vice versa, i.e., arguing that Type-1 MG can be more important than MUSIM gap (e.g., critical measurements in NW A). Therefore, we propose a fair solution (Sol #3) to handle such collision without being biased to any party.

Sol #3 handles the collision between MUSIM gaps and Type-1 MG based on the configured periodicity (MGRP) of the collided gaps. The collided gap which has larger MGRP should be kept and the one with shorter MGRP can be dropped. This solution provides more fairness on which gap to keep/drop based on their frequent opportunities rather than applying a default rule. If both gaps’ MGRPs are the same, then prioritize MUSIM gap only if it is configured with the highest priority level (P1); otherwise prioritize Type-1 MG. In addition, when aperiodic MUSIM gap collide with Type-1 MG, prioritize aperiodic MUSIM gap by default since it is a single-shot gap.

Proposal 7: Collision between periodic MUSIM gaps and Type-1 MG can be handled based on the MGRP of the collided gaps (i.e., rather than applying default prioritization), which can provide more fairness.

Proposal 8: Collision between aperiodic MUSIM gap and Type-1 MG can be handled by prioritizing aperiodic MUSIM gap by default.

## Case 3: On collision between MUSIM gaps and NW A signals

The related issues for MUSIM gap collision with other signals are captured below:

Issue 2-4-2: Priority of MUSIM against SMTC for L3/ L1 measurement - Proposals - P1: MUSIM gaps have higher priority when colliding with SMTC/SSB for L3/L1 measurement (collisions between L3/L1 measurement resources and MUSIM gaps are handled in the same way as collisions between L3/L1 measurement resources and measurement gaps) (Apple xiaomi oppo vivo Huawei Ericsson MTK) - P2: RAN4 shall strike for optimization between MUSIM gaps and SMTC/L1 in NW A. (Apple) - P3: RAN4 to consider other options than only having a fixed MUSIM priority over SMTC, and other L3/ L1 measurement resources (Nokia, Ericsson) Recommended WF Continue discussion

For Issue 2-4-2, we support P1. The same principle of having high priority for MG against SMTC and other L1 measurements can be applied for MUSIM gaps as well.

Proposal 9: MUSIM gaps have higher priority when colliding with SMTC/SSB for L3/L1 measurement (collisions between L3/L1 measurement resources and MUSIM gaps are handled in the same way as collisions between L3/L1 measurement resources and measurement gaps).

Issue 2-4-3: Collision between SMTC and MUSIM gaps for handover and Scell activation - Proposals Recommended WF - Agreements: - FFS on collision between SMTC and MUSIM gaps for handover and Scell activation Continue discussion

For Issue 2-4-3, any collision between other RRM procedures and MUSIM gaps can be handled in the same way as collisions between RRM procedures and legacy MG, i.e., no joint requirements to be specified by default.

Proposal 10: Collisions between other RRM procedures and MUSIM gaps are handled in the same way as collisions between RRM procedures and legacy MG, i.e., no joint requirements to be specified by default.

# Summary

In this contribution we have discussed the RRM requirements for MUSIM gap collision handling mechanisms against legacy MGs, other MUSIM gaps and other signals. The following proposals were approached:

Proposal 1: introduce a new parameter (MUSIMGapPriority) in the assistance-information along with the existing parameters used by the UE to request MUSIM gap configuration from NW A (i.e., offset, MGL, MGRP and MUSIMGapPriority).

Proposal 2: The requested priority (MUSIMGapPriority) for MUSIM can be designed using 2-bis to allow the UE suggesting priorities for 3 periodic MUSIM gaps.

Proposal 3: NW A maintaining the same relative priorities requested by the UE.

Proposal 4: Aperiodic MUSIM gap can be prioritized by default and no need to introduce priority level for it.

Proposal 5: The gap proximity condition for the Rel-17 concurrent gap collision should be reused for the collision between different MUSIM gap.

Proposal 6: Priority based solution should be sufficient to handle collisions between different MUSIM gaps.

Proposal 7: Collision between periodic MUSIM gaps and Type-1 MG can be handled based on the MGRP of the collided gaps (i.e., rather than applying default prioritization), which can provide more fairness.

Proposal 8: Collision between aperiodic MUSIM gap and Type-1 MG can be handled by prioritizing aperiodic MUSIM gap by default.

Proposal 9: MUSIM gaps have higher priority when colliding with SMTC/SSB for L3/L1 measurement (collisions between L3/L1 measurement resources and MUSIM gaps are handled in the same way as collisions between L3/L1 measurement resources and measurement gaps).

Proposal 10: Collisions between other RRM procedures and MUSIM gaps are handled in the same way as collisions between RRM procedures and legacy MG, i.e., no joint requirements to be specified by default.

# References

- R4-2303248, WF on NR Dual TxRx Multi-SIM, vivo.

- 3GPP TS 38.133, V17.7.0, “NR; Requirements for support of radio resource management”.