3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304636

Online, April 17 – April 26, 2023

Agenda Item:	5.14.4.1

Source: 	CMCC

Title: 			 Discussion on general RRM requirements for ATG

Document for:	Discussion

- Introduction

In RAN4#106 meeting, some general issues were discussed, the WF has been approved in [1]. In this contribution, we further provide our views on theses open general issues based on current progress.

- General

Band table

- RAN4 to revisit the discussions on need for separate frequency band grouping table after REFSENS has been agreed for ATG bands

In last meeting, following agreement has been achieved in RF session [2]:

Agreements - The REFSENS requirements specified in TS 38.101-1 for each band can be reused for ATG UE

Since RAN4 has agreed to reuse the legacy REFSENS requirements for ATG UE for each band, the legacy frequency band grouping table can be also reused.

Proposal 1: Reuse the legacy frequency band grouping table for R18 ATG.

Issue 1-1-5: How to involve ATG RRM core requirements in TS38.133

- Option 1: ATG specific RRM requirements are defined in separate sections in specification.

- Option 2: Add ATG related requirements in the current corresponding section, similar as HST.

After four meetings of ATG impaction discussion, RAN4 has basically recognized the differences between ATG and fully ground based systems, as listed in the table below:

| Requirement | Item | Comments |
| --- | --- | --- |
| [Frequency bands grouping] | [NR operating bands in FR1] | TBD |
| [Cell re-selection] | [Cell re-selection mechanism] | TBD |
|  | [Cell re-selection measurement requirements] | TBD [signalling, associated UE capabilities and how to switch between the two sets of requirements] |
| [Conditional handover] | [Conditional handover mechanism] | TBD |
|  | [Conditional handover requirements] | TBD |
| UE transmit timing | Initial transmit timing requirements Te | Consider UE pre-compensation timing error |
|  | Gradual timing adjustment | TBD |
|  | Timing advance adjustment delay requirement | The koffset mechanism need to be introduced |
| [Measurement mechanism] |  | TBD |
| [RLM/BFD] |  | TBD [whether the existing relaxed RLM/BFD can be applied based on the assistance information] |
| NR intra-frequency measurements |  | The assumption that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 is not valid in ATG scenario. |
| [Scheduling restriction] |  | TBD |
| [Pre-configured measurement gap] |  | TBD |

Option 1 is suitable for the requirements that have been greatly updated compared to the legacy, including:

- Timing requirements

- Cell reselection requirements (if location-based cell reselection is introduced)

- Conditional handover (if location-based CHO is introduced)

Option 2 is suitable for the requirements which could reuse most of legacy requirements, including:

- Cell reselection requirements (if location-based cell reselection is NOT introduced)

- Measurement requirements (Assumption for derive SSB-IndexFromCell)

- Scheduling restriction

Therefore, if location-based cell reselection and location-based CHO mechanism are not introduced, then we support to use Option 2. Otherwise, Option 1 is more suitable.

Proposal 2: If location-based cell reselection and location-based CHO mechanism are not introduced, then we support to add ATG related requirements in the current corresponding section, similar as HST. Otherwise, define ATG specific RRM requirements in separate sections is more suitable.

- Conclusion

Based on the discussion above, we bring the following proposals:

Proposal 1: Reuse the legacy frequency band grouping table for R18 ATG.

Proposal 2: If location-based cell reselection and location-based CHO mechanism are not introduced, then we support to add ATG related requirements in the current corresponding section, similar as HST. Otherwise, define ATG specific RRM requirements in separate sections is more suitable.

- Reference

[1] R4-2303226, WF on NR ATG RRM requirements, CMCC

[2] R4-2303645, WF on ATG UE RF requirements, HW