3GPP TSG-RAN WG4 Meeting # 106bis-e	R4-2304644

Online, April 17 – April 26, 2023

Agenda Item:	5.19.2

Source: 	CMCC

Title: 			 Simulation results for ATP demodulation requirements

Document for:	Discussion

- Introduction

After RAN4#106 meeting, companies are encouraged to bring evaluation results with and without retransmission. [1] We update our simulation results without HARQ, and further provide our simulation results with HARQ.

For HARQ enabled test cases, we use the latest simulation assumption as follows:

For retransmission number 4 including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; for precoder, follow the latest UE reported PMI whose rank is same as the initial transmission.

- Requirement specification

Simulation results

Our simulation results of FR1 cases were provided in the table below, BLER represent the overall BLER:

| FR1 FDD 2T2R without HARQ |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | RANK | CQI | BLER | TP |  | TP | SNR |
| 0 | 1 | 2 | 0.074 | 2.810 |  | 10% | 4.5 |
| 2 | 1 | 3 | 0.126 | 4.395 |  | 15% | 8.3 |
| 4 | 1 | 4 | 0.191 | 6.066 |  | 20% | 11.2 |
| 6 | 1 | 5 | 0.250 | 7.757 |  | 25% | 13.9 |
| 8 | 1 | 5 | 0.239 | 9.344 |  | 30% | 15.9 |
| 10 | 1 | 6 | 0.230 | 11.438 |  | 35% | 17.6 |
| 12 | 2 | 6 | 0.215 | 13.866 |  | 40% | 19.8 |
| 14 | 2 | 7 | 0.210 | 16.273 |  |  |  |
| 16 | 2 | 7 | 0.217 | 19.493 |  |  |  |
| 18 | 2 | 8 | 0.169 | 23.471 |  |  |  |
| 20 | 2 | 9 | 0.149 | 26.076 |  |  |  |

| FR1 FDD 2T2R with HARQ |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | RANK | CQI | BLER | TP |  | TP | SNR |
| 0 | 1 | 2 | 0.086 | 3.025 |  | 10% | 4 |
| 2 | 1 | 3 | 0.124 | 4.689 |  | 15% | 7.8 |
| 4 | 1 | 4 | 0.198 | 6.443 |  | 20% | 11 |
| 6 | 1 | 5 | 0.231 | 8.210 |  | 25% | 13.8 |
| 8 | 1 | 5 | 0.191 | 9.867 |  | 30% | 15.9 |
| 10 | 1 | 6 | 0.189 | 11.968 |  | 35% | 17.4 |
| 12 | 2 | 6 | 0.182 | 13.887 |  | 40% | 19.6 |
| 14 | 2 | 7 | 0.173 | 16.362 |  |  |  |
| 16 | 2 | 7 | 0.157 | 19.581 |  |  |  |
| 18 | 2 | 8 | 0.152 | 23.785 |  |  |  |
| 20 | 2 | 9 | 0.143 | 26.395 |  |  |  |

Observation 1: For FDD 2T2R test case, when HARQ is enabled, the TP is improved for all SNRs, especially in lower SNR regions SNR 0-10 and higher SNR regions SNR 18-20. The BLER is decreased for almost all SNRs.

| FR1 FDD 2T4R without HARQ |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | RANK | CQI | BLER | TP |  | TP | SNR |
| 0 | 1 | 3 | 0.059 | 5.271 |  | 10% | 1.1 |
| 2 | 1 | 3 | 0.085 | 7.587 |  | 15% | 4 |
| 4 | 2 | 3 | 0.118 | 9.677 |  | 20% | 6 |
| 6 | 2 | 4 | 0.090 | 12.984 |  | 25% | 7.6 |
| 8 | 2 | 5 | 0.093 | 16.931 |  | 30% | 9.5 |
| 10 | 2 | 6 | 0.047 | 20.246 |  | 35% | 11 |
| 12 | 2 | 7 | 0.039 | 25.271 |  | 40% | 12.2 |
| 14 | 2 | 8 | 0.023 | 30.623 |  | 45% | 13.4 |
| 16 | 2 | 9 | 0.022 | 36.258 |  | 50% | 14.6 |
| 18 | 2 | 10 | 0.017 | 42.209 |  | 55% | 15.7 |
| 20 | 2 | 10 | 0.014 | 47.062 |  | 60% | 17 |

| FR1 FDD 2T4R with HARQ |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | RANK | CQI | BLER | TP |  | TP | SNR |
| 0 | 1 | 3 | 0.045 | 5.280 |  | 10% | 1 |
| 2 | 1 | 3 | 0.075 | 7.695 |  | 15% | 3.6 |
| 4 | 2 | 3 | 0.093 | 10.253 |  | 20% | 5.7 |
| 6 | 2 | 4 | 0.076 | 13.468 |  | 25% | 7.3 |
| 8 | 2 | 5 | 0.087 | 17.468 |  | 30% | 9.2 |
| 10 | 2 | 6 | 0.041 | 20.625 |  | 35% | 10.8 |
| 12 | 2 | 7 | 0.038 | 25.385 |  | 40% | 12.1 |
| 14 | 2 | 8 | 0.027 | 30.660 |  | 45% | 13.4 |
| 16 | 2 | 9 | 0.021 | 36.318 |  | 50% | 14.6 |
| 18 | 2 | 10 | 0.015 | 42.251 |  | 55% | 15.7 |
| 20 | 2 | 10 | 0.011 | 46.887 |  | 60% | 16.8 |

Observation 2: For FDD 2T4R test case, when HARQ is enabled, the TP is improved for SNR 0-18, especially in SNR regions SNR 4-10. The BLER is decreased for almost all SNRs.

| FR1 TDD 2T2R without HARQ |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | RANK | CQI | BLER | TP |  | TP | SNR |
| 0 | 1 | 2 | 0.054 | 7.855 |  | 10% | 5 |
| 2 | 1 | 3 | 0.104 | 12.554 |  | 15% | 8.8 |
| 4 | 1 | 3 | 0.140 | 17.702 |  | 20% | 12.2 |
| 6 | 1 | 5 | 0.230 | 23.512 |  | 25% | 14.8 |
| 8 | 1 | 5 | 0.195 | 28.522 |  | 30% | 16.9 |
| 10 | 1 | 6 | 0.184 | 35.076 |  | 35% | 18.6 |
| 12 | 1 | 6 | 0.206 | 40.806 |  |  |  |
| 14 | 2 | 6 | 0.215 | 48.232 |  |  |  |
| 16 | 2 | 7 | 0.220 | 57.199 |  |  |  |
| 18 | 2 | 8 | 0.139 | 68.807 |  |  |  |
| 20 | 2 | 8 | 0.121 | 81.114 |  |  |  |

| FR1 TDD 2T2R with HARQ |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | RANK | CQI | BLER | TP |  | TP | SNR |
| 0 | 1 | 2 | 0.062 | 8.216 |  | 10% | 4.9 |
| 2 | 1 | 3 | 0.094 | 13.503 |  | 15% | 8.4 |
| 4 | 1 | 3 | 0.116 | 17.798 |  | 20% | 11.5 |
| 6 | 1 | 5 | 0.200 | 24.559 |  | 25% | 14.7 |
| 8 | 1 | 5 | 0.161 | 30.044 |  | 30% | 16.8 |
| 10 | 1 | 6 | 0.165 | 36.013 |  | 35% | 18.6 |
| 12 | 1 | 6 | 0.138 | 43.419 |  |  |  |
| 14 | 2 | 6 | 0.144 | 49.138 |  |  |  |
| 16 | 2 | 7 | 0.144 | 57.505 |  |  |  |
| 18 | 2 | 8 | 0.124 | 68.381 |  |  |  |
| 20 | 2 | 8 | 0.081 | 81.657 |  |  |  |

Observation 3: For TDD 2T2R test case, when HARQ is enabled, the TP is improved for all SNRs expect SNR18, especially in SNR regions SNR 0-2 and SNR 4-16. The BLER is decreased for almost all SNRs.

| FR1 TDD 2T4R without HARQ |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | RANK | CQI | BLER | TP |  | TP | SNR |
| 0 | 1 | 3 | 0.020 | 14.676 |  | 10% | 1.8 |
| 2 | 1 | 3 | 0.108 | 21.635 |  | 15% | 4.3 |
| 4 | 2 | 3 | 0.084 | 29.756 |  | 20% | 6.6 |
| 6 | 2 | 4 | 0.077 | 37.393 |  | 25% | 8.2 |
| 8 | 2 | 5 | 0.070 | 50.442 |  | 30% | 9.9 |
| 10 | 2 | 6 | 0.066 | 62.991 |  | 35% | 11.1 |
| 12 | 2 | 7 | 0.051 | 79.570 |  | 40% | 12.7 |
| 14 | 2 | 7 | 0.017 | 89.959 |  | 45% | 14.4 |
| 16 | 2 | 8 | 0.002 | 106.019 |  | 50% | 15.7 |
| 18 | 2 | 9 | 0.007 | 123.261 |  | 55% | 16.9 |
| 20 | 2 | 10 | 0.007 | 139.551 |  | 60% | 18.1 |

| FR1 TDD 2T4R with HARQ |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SNR | RANK | CQI | BLER | TP |  | TP | SNR |
| 0 | 1 | 3 | 0.023 | 15.115 |  | 10% | 1.5 |
| 2 | 1 | 3 | 0.074 | 22.515 |  | 15% | 4.2 |
| 4 | 2 | 4 | 0.031 | 30.393 |  | 20% | 6.4 |
| 6 | 2 | 4 | 0.056 | 38.607 |  | 25% | 8.1 |
| 8 | 2 | 5 | 0.045 | 51.224 |  | 30% | 9.6 |
| 10 | 2 | 6 | 0.062 | 65.336 |  | 35% | 11 |
| 12 | 2 | 7 | 0.044 | 79.590 |  | 40% | 12.6 |
| 14 | 2 | 8 | 0.008 | 90.288 |  | 45% | 14.4 |
| 16 | 2 | 9 | 0.010 | 106.031 |  | 50% | 15.7 |
| 18 | 2 | 10 | 0.007 | 123.299 |  | 55% | 16.9 |
| 20 | 2 | 10 | 0.008 | 139.896 |  | 60% | 18.1 |

Observation 4: For TDD 2T4R test case, when HARQ is enabled, the TP is improved for all cases, especially in lower SNR regions SNR 0-10. The BLER is decreased for almost all SNRs.

- Conclusion

Based on the discussion above, the following observations are concluded.

Observation 1: For FDD 2T2R test case, when HARQ is enabled, the TP is improved for all SNRs, especially in lower SNR regions SNR 0-10 and higher SNR regions SNR 18-20. The BLER is decreased for almost all SNRs.

Observation 2: For FDD 2T4R test case, when HARQ is enabled, the TP is improved for SNR 0-18, especially in SNR regions SNR 4-10. The BLER is decreased for almost all SNRs.

Observation 3: For TDD 2T2R test case, when HARQ is enabled, the TP is improved for all SNRs expect SNR18, especially in SNR regions SNR 0-2 and SNR 4-16. The BLER is decreased for almost all SNRs.

Observation 4: For TDD 2T4R test case, when HARQ is enabled, the TP is improved for all cases, especially in lower SNR regions SNR 0-10. The BLER is decreased for almost all SNRs.

- Reference

[1] R4-2302930, WF on absolute physical layer throughput requirement, Intel Corporation.