# 3GPP TSG-RAN WG4 Meeting # 106bis-e 	                   	      R4-2304666

# Electronic Meeting, April 17 – April 26, 2023

# Title: 	Discussion on 8Rx UE demodulation and CSI

# Source: 	ZTE Corporation

# Agenda item:	5.5.4.1.1

# Document for:	Approval

# 1 Introduction

- In RAN 4 #106 meeting, 8Rx UE demodulation and CSI requirements were discussed and a WF was agreed[1]. In this paper, we will provide our views on some issues and some simulation results on 8 Rx PDSCH for FR1.

# 2 Discussion

## 2.1 PDSCH requirements

Issue 2-3: Antenna correlation - Rank 2: ULA Medium A - Rank 4: ULA Low, FFS ULA Medium A - Rank 8: ULA Low, FFS ULA Medium B

- For Rank 4 and Rank 8, only consider ULA low antenna correlation.

Proposal 1: Only consider ULA low for Rank 4 and Rank 8.

Issue 2-4: Propagation conditions - For evaluation purpose: - TDLC300-100 for Rank2 and TDLA30-10 for Rank 4 and 8. - FFS on consideration of beamforming model on top of TDL channel model

- From what we understand, we think there is no necessary to consider beamforming model for 8Rx case.

Proposal 2: Only consider TDLC300-100 for Rank2 and TDLA30-10 for Rank 4 and 8.

Issue 2-5: MCS configuration - Companies are welcome to study appropriate MCS to be defined for Rank 2, 4 and 8 respectively.

With regard to MCS configuration, companies have different options for different Ranks. In this paper, we want to provide to our views on MCS configuration. Firstly, for Rank 2, we prefer to only consider MCS 13. And for Rank 4, we prefer to consider MCS 13 and MCS 19. For Rank 8, we tend to MCS 13 and MCS 17. Also we present the simulation results in next section.

Proposal 3: Only consider MCS 13 for Rank 2, MCS 13 and MCS 19 for Rank 4, MCS 13 and MCS 17 for Rank 8.

Issue 2-6: Two MCS configuration for Rank>4 - Option 1: Configure two MCSs for Rank>4 and discuss the spatial channel model to be used to guarantee different precoding gain for each layer - Option 2: Only consider single MCS

From what we understanding, different MCS correspond to different CQI. For two codewords case, different MCS should be selected in accordance with CQI values. In our evaluation, CQI feedback is not involved. There is no motivation for different MCS to be used by different codewords. Consequently, we tend to use the same MCS for two codewords.

Proposal 4: Only consider single MCS for two codewords.

Issue 2-11: Whether consider scenarios for CSI overlapping with PDSCH for CSI-RS ports larger than 4Tx - Option 1: Yes - Option 2: No

In general, if we use CSI-RS ports as larger as more than 8, there will be 8/16 REs per PRB not available for PDSCH, compared with 2REs per PRB in the current spec. which will bring challenges to the correct PDSCH coding due to higher data rate on the CSI-RS slots.

Proposal 5: To consider scenarios for CSI overlapping with PDSCH for CSI-RS ports larger than 4Tx.

Issue 2-13: N1 and N2 configuration for 8Tx case - Option 1: Use (N1,N2)=(4,1) (Samsung) - Option 2: Don’t consider this configuration

For rank 8 transmition, optional configurations for type I code book are (N1,N2)=(4,1) and (N1,N2)=(2,2). However, for different N1 and N2 configuration is informed by signalling which is decided by antenna configuration. And for demodulation part, we need to specify which configurations is used for PDSCH requirements.

Proposal 6: Only consider (N1,N2)=(4,1) configuration for 8Tx case.

## 2.2 Simulation results

In this section, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). We depict the SNR value corresponding to 70% of the peak throughput. All figures have been provided in the Appendix.

Table 2-1. Summary of PDSCH 8Rx simulation results

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz)/ (RB) | Modulation format and code rate | TDD UL/DL Pattern | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | 40 / 30 (106) | 16QAM, 0.48 | FR1.30-1 | 2 | TDLC300-100 | 2x8, ULA Medium | 70 | 25.4 |
| 1-2 | 40 / 30 (106) | 16QAM, 0.48 | FR1.30-1 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 7.1 |
| 1-3 | 40 / 30 (106) | 64QAM, 0.5 | FR1.30-1 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 22.8 |
| 1-4 | 40 / 30 (106) | 16QAM, 0.48 | FR1.30-1 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 14.4 |
| 1-5 | 40 / 30 (106) | 64QAM 0.43 | FR1.30-1 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20.3 |

# 3 Conclusion

In this contribution, we give some discussions on NR 8Rx demodulation. The conclusions are:

Proposal 1: Only consider ULA low for Rank 4 and Rank 8.

Proposal 2: Only consider TDLC300-100 for Rank2 and TDLA30-10 for Rank 4 and 8.

Proposal 3: Only consider MCS 13 for Rank 2, MCS 13 and MCS 19 for Rank 4, MCS 13 and MCS 17 for Rank 8.

Proposal 4: Only consider single MCS for two codewords.

Proposal 5: To consider scenarios for CSI overlapping with PDSCH for CSI-RS ports larger than 4Tx.

Proposal 6: Only consider (N1,N2)=(4,1) configuration for 8Tx case.

# 4 Reference

- R4-2302942, WF on 8RX demodulation and CSI requirements. Huawei, HiSilicon.

- RP-221496, WID revision: RF requirements enhancement for NR frequency range, Huawei, HiSilicon.

# 5 Appendix

Figure 1. 8 Rx PDSCH simulation results for Rank 2.

| - Rank 4, MCS 13, TDLA-30-10 | - Rank 4, MCS 19, TDLA-30-10 |
| --- | --- |

Figure 2. 8 Rx PDSCH simulation results for Rank 4.

| - Rank 8, MCS 13, TDLA-30-10 | - Rank 8, MCS 17, TDLA-30-10 |
| --- | --- |

Figure 3. 8 Rx PDSCH simulation results for Rank 8.