# 3GPP TSG-RAN WG4 Meeting # 106bis-e 	                   	      R4-2304671

# Electronic Meeting, April 17 – April 26, 2023

# Title: 	Discussion on demodulation performance requirements for NCR-MT

# Source: 	ZTE Corporation

# Agenda item:	5.29.6

# Document for:	Approval

# 1 Introduction

In RAN#97e meeting, the work item [RP-222673] on NR network-controlled repeater was approved after the completion of study item phase led by RAN1. In this contribution, we want to share initial analysis on demod requirement impacts from RAN4 perspective based on the listed feature from RAN1/RAN2 perspective. Generally speaking, since the NCR-MT is equipped with baseband functionality, then its demod requirement should be defined to check its practical performance. In the following section, we will provide the detailed assumptions for NCR-MT demod requirements.

- 2 Discussion

## 2.1 NCR-MT-General

- 2.1.1 Reference channels

- Demodulation requirements are defined based on single-slot FRCs similar as IAB-MT demod.

- PDSCH is scheduled only on ‘D’ slots without CSI-RS resource and TRS allocated.

Proposal 1. To consider demodulation requirements are defined based on single-slot FRCs similar as IAB-MT.

Proposal 2. To consider PDSCH is scheduled only on ‘D’ slots without CSI-RS resource and TRS allocated.

- 2.1.2 TDD pattern

From the Table 2.5-1, 2.5-2 and 2.5-3, we can find that currently only 40MHz/30kHz is defined for FR1 IAB-MT type 1-H/1-O and 100MHz/120kHz and 50MHz/60kHz is defined for FR2 IAB-MT type 2-O. From what we understand, 15kHz SCS for FR1 low band should be defined for NCR-MT, and more channel bandwidth is supposed to be supported.

Proposal 3. To reuse legacy TDD UL-DL pattern of IAB-MT requirement(30 kHz SCS: 7D1S2U, S=6D:4G:4U; 60, 120 kHz SCS: 3D1S1U, S=10D:2G:2U) for NCR-MT. Also consider to define 15kHz SCS for FR1 for NCR-MT which is supposed to support more channel bandwidth.

2.1.3 Antenna configuration and Rank number

- NCR-MT type 1-C might be different from IAB-MT type 1-H, these configuration need more discussions in RAN4.

Proposal 4. Antenna configuration and Rank number need more discussion for NCR-MT.

2.1.4 Reference signals in test parameters and reference channels

For SSB, TRS and CSI-RS, there is no significant impacts for demodulation requirements. And CSI reporting is exempt from the CSI-RS configuration omission.Therefor we can consider not define SSB, TRS, CSI-RS configurations as a part of demodulation performance test parameters or FRC.

Proposal 5. To consider not define SSB, TRS, CSI-RS configurations as a part of demodulation performance test parameters or FRC.

2.1.5 Propagation conditions

Consider TDLA 30-10 for FR1 and TDLA 30-75 for FR2.

Proposal 6. To consider TDLA 30-10 for FR1 and TDLA 30-75 for FR2.

2.1.6 Duplex mode--general

From the 38.174[1], we can find that IAB-MT only is defined in TDD band n41, n77, n78 and n79. However NCR-MT is still defined FDD band. Therefore, we need to consider FDD band requirements for NCR-MT.

Proposal 7. Propose to define both TDD band and FDD band requirements for NCR-MT.

## 2.2 NCR-MT-PDSCH

2.2.1 MCS

From the Table 2.5-1, 2.5-2 and 2.5-3, we can find that currently only 16QAM, 64QAM and 256QAM are defined for IAB-MT in Rel-16. However due to different traffic demand for NCR-MT, lower order modulation scheme is supposed to be supported.

Proposal 8. To consider lower order modulation scheme for NCR-MT.

2.2.2 Mapping type

Like IAB-MT demodulation requirements, only keep PDSCH performance requirements for mapping Type-A.

Proposal 9. To consider PDSCH performance requirements for mapping Type-A.

2.2.3 PRB bundling size

Like IAB-MT demodulation requirements, only keep requirements with PRB bundling size 2.

Proposal 10. To consider PRB bundling size 2 for PDSCH requirements.

2.2.4 Receiver type

Consider MMSE-IRC receiver for NCR-MT.

Proposal 11. To consider MMSE-IRC receiver as baseline for NCR-MT.

2.2.5 Overlapped CSI-RS

From what we understand, we can skip PDSCH cases for CSI-RS overlapped with PDSCH.

Proposal 12. Propose to skip PDSCH cases for CSI-RS overlapped with PDSCH.

2.2.6 Co-existence with LTE CRS

Skip PDSCH cases for CSI-RS overlapped with PDSCH.

Proposal 13. Propose to skip PDSCH cases for CSI-RS overlapped with PDSCH.

## 2.3 NCR-MT-PDCCH

2.3.1 Aggregation Level

From the Table 2.5-1, 2.5-2 and 2.5-3, we can find that currently only AL 2,4 and 8 are defined for IAB-MT. Alos we can consider the same aggregation levels for NCR-MT.

Proposal 14. To consider the same aggregation levels 2,4,8 for NCR-MT.

2.3.3 Propagation condition

For PDCCH, fading conditions are the same as PDSCH. So we can apply same approach as PDSCH.

Proposal 15. Apply same approach as PDSCH for propagation condition.

## 2.4 NCR-MT-PBCH

For NCR-MT PBCH, do not define PBCH requirements for NCR-MT.

Proposal 16. Propose to not define PBCH requirements for NCR-MT.

## 2.5 NCR-MT-SDR

For NCR-MT SDR, do not define SDR requirements for NCR-MT.

Proposal 17. Propose to not define SDR requirements for NCR-MT.

## 2.6 NCR-MT-CSI reporting

2.6.1 CQI

2.6.1.1 Propagation condition

Consider the similar propagation condition as IAB-MT.

Proposal 18. To consider AWGN propagation condition for CQI requirements.

2.6.1.2 CSI-RS

For CQI reporting test case, CQI CSI-RS Resource type and report config similar as IAB-MT. According to the Table 2.7-1, 2.7-2 and 2.7-3, we can make a follow conclusion

- For FR1, use periodic reporting for AWGN.

- For FR2, use periodic reporting for AWGN.

And limit requirements for CQI reporting to the wideband case similar as IAB-MT.

Proposal 19. For CQI reporting test case, use periodic reporting for AWGN for FR1 and FR2. And limit requirements for CQI reporting to the wideband case similar as IAB-MT.

2.6.2 PMI

Consider introducing PMI requirements for NCR-MT. And for PMI CSI-RS Resource type and report config, adopt PMI reporting requirements similar as IAB-MT.

Proposal 20. To consider introducing PMI requirements for NCR-MT.

2.6.3 RI

Consider introducing RI requirements for NCR-MT. And for RI CSI-RS Resource type and report config, adopt RI reporting requirements similar as IAB-MT.

Proposal 21. To consider introducing RI requirements for NCR-MT.

## 2.7 Detail test scope

2.7.1 TDD

In this section, we summarized the R-16 IAB-MT demodulation requirements as shown in following table 2.7-1, and 2.7-2.

Proposal 22. Reuse NR IAB-MT performance requirements as per overview in Table 2.7-1 and 2.7-2 for FR1 and FR2 for NCR-MT respectively .

Table 2.7-1 Overview for IAB-MT conducted test for FR1

| Demodulation performance requirements | Parameters for test setup |  | Re-use |
| --- | --- | --- | --- |
| PDSCH | Antenna configuration | 2x4 ULA Low, 4x4 ULA Low | - |
|  | Channel model | TDLA-30-10 |  |
|  | MCS | 13, 19, 24(Table2) |  |
|  | Rank | 1，2 for 2x4 ULA Low, 3, 4 for 4x4 ULA Low |  |
|  | CBW&SCS | 40MHz for 30kHz SCS |  |
|  | Resource mapping | Type A |  |
|  | Test metric | 70%, 30%(2x4 ULA Low) of maximum TP |  |
| PDCCH | Antenna configuration | 1x4 ULA Low, 2x4 ULA Low | - |
|  | Channel model | TDLA30-10 |  |
|  | CBW&SCS | 40MHz for 30kHz SCS |  |
|  | Aggregation level | 2, 4, 8 |  |
|  | DCI Format | 1_0, 1_1 |  |
|  | Test metric | 1% of Pm-dsg |  |
| PBCH | - | - | - |
| SDR | - | - | - |
| CQI | Antenna configuration | 2x4 | - |
|  | Channel model | AWGN |  |
|  | CBW&SCS | 40MHz for 30kHz SCS |  |
|  | CQI reporting | wideband |  |
|  | CSI-RS resource Type | periodic |  |
|  | ReportConfigType | periodic |  |
| PMI | Antenna configuration | 4x4 XP High, 8x4 XP High | - |
|  | Channel model | TDLA30-5 |  |
|  | CBW&SCS | 40MHz for 30kHz SCS |  |
|  | PMI reporting | wideband |  |
|  | CSI-RS resource Type | aperiodic |  |
|  | ReportConfigType | aperiodic |  |
| RI | Antenna configuration | 2x4 ULA Low, 2x4 ULA High, 4x4 ULA Low | - |
|  | Channel model | TDLA30-5 |  |
|  | CBW&SCS | 40MHz for 30kHz SCS |  |
|  | CSI-RS resource Type | periodic |  |
|  | ReportConfigType | periodic |  |

Table 2.7-2 Overview for IAB-MT radiated test for FR2

| Demodulation performance requirements | Parameters for test setup |  | Re-use |
| --- | --- | --- | --- |
| PDSCH | Antenna configuration | 2x2 ULA Low | - |
|  | Channel model | TDLA-30-75 |  |
|  | MCS | 13, 17, 18 |  |
|  | Rank | 1, 2 |  |
|  | CBW&SCS | 50MHz for 60kHz SCS, 100MHz for 120kHz SCS |  |
|  | Resource mapping | Type A |  |
|  | Test metric | 70%, 30%of maximum TP |  |
| PDCCH | Antenna configuration | 1x2 ULA Low, 2x2 ULA Low | - |
|  | Channel model | TDLA30-75 |  |
|  | CBW&SCS | 100MHz for 120kHz SCS |  |
|  | Aggregation level | 2, 4, 8 |  |
|  | DCI Format | 1_0, 1_1 |  |
|  | Test metric | 1% of Pm-dsg |  |
| PBCH | - | - | - |
| SDR | - | - | - |
| CQI | Antenna configuration | 2x2 | - |
|  | Channel model | AWGN |  |
|  | CBW&SCS | 100MHz for 120kHz SCS |  |
|  | CQI reporting | wideband |  |
|  | CSI-RS resource Type | periodic |  |
|  | ReportConfigType | periodic |  |
| PMI | Antenna configuration | 2x2 ULA Low | - |
|  | Channel model | TDLA30-35 |  |
|  | CBW&SCS | 100MHz for 120kHz SCS |  |
|  | PMI reporting | wideband |  |
|  | CSI-RS resource Type | periodic |  |
|  | ReportConfigType | periodic |  |
| RI | Antenna configuration | 2x2 ULA Low, 2x2 XP High | - |
|  | Channel model | TDLA30-35 |  |
|  | CBW&SCS | 100MHz for 120kHz SCS |  |
|  | CSI-RS resource Type | periodic |  |
|  | ReportConfigType | periodic |  |

And for TDD 15KHz，we give some initial simulation assumptions in following table.

Table 2.7-3 Overview for NCR-MT test for FR1

| Demodulation performance requirements | Parameters for test setup |  |
| --- | --- | --- |
| PDSCH | Antenna configuration | 2x4 ULA Low, 4x4 ULA Low |
|  | Channel model | TDLA-30-10 |
|  | MCS | 13,19 |
|  | Rank | 1，2 for 2x4 ULA Low, 3, 4 for 4x4 ULA Low |
|  | CBW&SCS | 10MHz for 15kHz SCS (DDDSU) |
|  | Resource mapping | Type A |
|  | Test metric | 70%, 30%(2x4 ULA Low) of maximum TP |
| PDCCH | Antenna configuration | 1x4 ULA Low, 2x4 ULA Low |
|  | Channel model | TDLA30-10 |
|  | CBW&SCS | 10MHz for 15kHz SCS |
|  | Aggregation level | 2, 4, 8 |
|  | DCI Format | 1_0, 1_1 |
|  | Test metric | 1% of Pm-dsg |
| CQI | Antenna configuration | 2x4 |
|  | Channel model | AWGN |
|  | CBW&SCS | 10MHz for 15kHz SCS |
|  | CQI reporting | wideband |
|  | CSI-RS resource Type | periodic |
|  | ReportConfigType | periodic |
| PMI | Antenna configuration | 4x4 XP High, 8x4 XP High |
|  | Channel model | TDLA30-5 |
|  | CBW&SCS | 10MHz for 15kHz SCS |
|  | PMI reporting | wideband |
|  | CSI-RS resource Type | aperiodic |
|  | ReportConfigType | aperiodic |
| RI | Antenna configuration | 2x4 ULA Low, 2x4 ULA High, 4x4 ULA Low |
|  | Channel model | TDLA30-5 |
|  | CBW&SCS | 10MHz for 15kHz SCS |
|  | CSI-RS resource Type | periodic |
|  | ReportConfigType | periodic |

2.7.2 FDD

For FDD band, we provide NCR-MT test for FR1 in following Table 2.7-4.

Table 2.7-4 Overview for NCR-MT test for FR1 FDD band

| Demodulation performance requirements | Parameters for test setup |  |
| --- | --- | --- |
| PDSCH | Antenna configuration | 2x4 ULA Low, 4x4 ULA Low |
|  | Channel model | TDLA-30-10 |
|  | MCS | 13, 19, 24 |
|  | Rank | 1，2 for 2x4 ULA Low, 3, 4 for 4x4 ULA Low |
|  | CBW&SCS | 10MHz for 15kHz SCS |
|  | Resource mapping | Type A |
|  | Test metric | 70%, 30%(2x4 ULA Low) of maximum TP |
| PDCCH | Antenna configuration | 1x4 ULA Low, 2x4 ULA Low |
|  | Channel model | TDLA30-10 |
|  | CBW&SCS | 10MHz for 15kHz SCS |
|  | Aggregation level | 2, 4, 8 |
|  | DCI Format | 1_0, 1_1 |
|  | Test metric | 1% of Pm-dsg |
| CQI | Antenna configuration | 2x4 |
|  | Channel model | AWGN |
|  | CBW&SCS | 10MHz for 15kHz SCS |
|  | CQI reporting | wideband |
|  | CSI-RS resource Type | periodic |
|  | ReportConfigType | periodic |
| PMI | Antenna configuration | 4x4 XP High, 8x4 XP High |
|  | Channel model | TDLA30-5 |
|  | CBW&SCS | 10MHz for 15kHz SCS |
|  | PMI reporting | wideband |
|  | CSI-RS resource Type | aperiodic |
|  | ReportConfigType | aperiodic |
| RI | Antenna configuration | 2x4 ULA Low, 2x4 ULA High, 4x4 ULA Low |
|  | Channel model | TDLA30-5 |
|  | CBW&SCS | 10MHz for 15kHz SCS |
|  | CSI-RS resource Type | periodic |
|  | ReportConfigType | periodic |

- Proposal 23. To consider Table 2.7-3 assumptions for NCR-MT 15kHz TDD band requirements.

- Proposal 24. To consider Table 2.7-4 assumptions for NCR-MT FDD band requirements.

- 3 Conclusion

In this contribution, we give some discussions on demodulation performance requirements for NCR-MT , The conclusions are:

Proposal 1. To consider demodulation requirements are defined based on single-slot FRCs similar as IAB-MT.

Proposal 2. To consider PDSCH is scheduled only on ‘D’ slots without CSI-RS resource and TRS allocated.

Proposal 3. To reuse legacy TDD UL-DL pattern of IAB-MT requirement(30 kHz SCS: 7D1S2U, S=6D:4G:4U; 60, 120 kHz SCS: 3D1S1U, S=10D:2G:2U) for NCR-MT. Also consider to define 15kHz SCS for FR1 for NCR-MT which is supposed to support more channel bandwidth.

Proposal 4. Antenna configuration and Rank number need more discussion for NCR-MT.

Proposal 5. To consider not define SSB, TRS, CSI-RS configurations as a part of demodulation performance test parameters or FRC.

Proposal 6. To consider TDLA 30-10 for FR1 and TDLA 30-75 for FR2.

Proposal 7. Propose to define both TDD band and FDD band requirements for NCR-MT.

Proposal 8. To consider lower order modulation scheme for NCR-MT.

Proposal 9. To consider PDSCH performance requirements for mapping Type-A.

Proposal 10. To consider PRB bundling size 2 for PDSCH requirements.

Proposal 11. To consider MMSE-IRC receiver as baseline for NCR-MT.

Proposal 12. Propose to skip PDSCH cases for CSI-RS overlapped with PDSCH.

Proposal 13. Propose to skip PDSCH cases for CSI-RS overlapped with PDSCH.

Proposal 14. To consider the same aggregation levels 2,4,8 for NCR-MT.

Proposal 15. Apply same approach as PDSCH for propagation condition.

Proposal 16. Propose to not define PBCH requirements for NCR-MT.

Proposal 17. Propose to not define SDR requirements for NCR-MT.

Proposal 18. To consider AWGN propagation condition for CQI requirements.

Proposal 19. For CQI reporting test case, use periodic reporting for AWGN for FR1 and FR2. And limit requirements for CQI reporting to the wideband case similar as IAB-MT.

Proposal 20. To consider introducing PMI requirements for NCR-MT.

Proposal 21. To consider introducing RI requirements for NCR-MT.

Proposal 22. Reuse NR IAB-MT performance requirements as per overview in Table 2.7-1 and 2.7-2 for FR1 and FR2 for NCR-MT respectively .

- Proposal 23. To consider Table 2.7-3 assumptions for NCR-MT 15kHz TDD band requirements.

- Proposal 24. To consider Table 2.7-4 assumptions for NCR-MT FDD band requirements.

# 4	Reference

[1]  RP-222673, New WID on NR network-controlled repeater, ZTE, Sanechips.