3GPP TSG-RAN WG4 Meeting # 106bis-e						R4-2304675

Online, April 17 – April 26, 2023

Agenda Item:	5.14.2.3

Source: 		Qualcomm Incorporated

Title: 	On Rx requirements for ATG UE RF

Document for:	Approval

# 1 Introduction

In RAN4#106, the UE RF requirements for ATG were discussed and the WF was approved in [1]. This paper will provide our views on ATG UE Rx requirements.

# 2 Discussion

## 2.1 Maximum input level

In RAN4#104-bis-e, the following was agreed regarding maximum input level

Further analyse and assume BS output power, minimum distance between BS and UE, couple loss and coexistence output. Then come back to this Maximum input level.

For TN, the BS height of 35m was assumed and -25dBm was defined with the assumptions of min. CL is 70dB. As agreed in the co-existence study discussion, 3000m is the lower boundary for the aircraft altitude.

With the co-existence simulation calibration results, it could be observed that the min. CL for ATG network (both 2GHz and 4GHz) is about 100dB. Therefore, there is 30dB gap. With that, it is reasonable to apply 25dB relaxation considering 5dB margin for maximum input level requirement.

Proposal 1: RAN4 to consider 25dB relaxation for the maximum input level compared with TN requirements.

Note that the maximum input level is the max level for signal including the interference so the test with maximum interference level, e.g., Pinterference = -25dBm for ACS case 2, should be in line with maximum input level requirements.

Proposal 2: Pinterference for ACS case 2 should align with the maximum input level

# 3 Conclusions

This contribution provided our considerations on ATG UE Rx requirements. We have the following observation and proposals:

Proposal 1: RAN4 to consider 25dB relaxation for the maximum input level compared with TN requirements.

Proposal 2: Pinterference for ACS case 2 should align with the maximum input level

# 4 References

R4-2303643, “WF on ATG UE RF requirements”, Huawei, HiSilicon