3GPP TSG-RAN WG4 Meeting # 106bis-e						R4-2304676

Online, April 17 – April 26, 2023

Agenda Item:		5.14.2.3

Source: 		Qualcomm Incorporated

Title: 	TP on TR 38.876 for ATG UE Rx reuqirements

Document for:	Approval

# 1 Introduction

In this paper, we provide the following text proposal to capture the following ATG CPE Rx requirements.

# 2 Text proposal

## <<Start of Change for TR 38.876>>

### 7.1.3	ATG CPE Rx requirements

#### 7.1.3.4	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the ATG CPE. For CPE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). CPE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For CPEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

#### 7.1.3.2	Diversity characteristics

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the CPE for ATG.

The CPE for ATG is required to be equipped with a minimum of two Rx antenna ports in all the ATG operating bands in FR1.

#### 7.1.3.3	REFSENS requirements

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the CPE antenna ports, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The same assumptions of NR FR1 handheld UE are used for NR ATG CEP in ATG operating bands in FR1. Therefore, the reference sensitivity of NR FR1 handheld UE could be reused for ATG CPE.

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2 from TS 38.101-1 [xx] (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1 from TS 38.101-1 [xx]) with parameters specified in Table 7.1.3.2.2-1, Table 7.1.3.2.2-2 and Table 7.1.3.2.2-3

Table 7.1.3.2.2-1: Two antenna port reference sensitivity QPSK REFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| n1 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |

Table 7.1.3.2.2-2: Two antenna port reference sensitivity QPSK REFSENS for TDD bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm) | Duplex Mode |
| n78 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n79 | 15 | 10, 20, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |

Table 7.1.3.2.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n1 | 15 | 25 | 501 | 751 | 1001 | 1281 | 1281 |  | 1281 | 1281 | 1281 |  |  |  |  |  | FDD |
|  | 30 |  | 24 | 361 | 501 | 641 | 641 |  | 641 | 641 | 641 |  |  |  |  |  |  |
|  | 60 |  | 101 | 18 | 24 | 301 | 301 |  | 301 | 301 | 301 |  |  |  |  |  |  |
| n78 | 15 |  | 50 | 75 | 100 | 128 | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 | 36 | 50 | 64 | 75 |  | 100 |  | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  | 10 | 18 | 24 | 30 | 36 |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |
| n79 | 15 |  | 50 |  | 100 |  | 160 |  | 216 |  | 270 |  |  |  |  |  | TDD |
|  | 30 |  | 24 |  | 50 |  | 75 |  | 100 |  | 128 | 162 | 180 | 216 | 243 | 270 |  |
|  | 60 |  | 10 |  | 24 |  | 36 |  | 50 |  | 64 | 75 | 90 | 100 | 120 | 135 |  |

#### 7.1.3.4	Maximum input level

[To be updated]

#### 7.1.3.5	Adjacent channel selectivity

[To be updated]

## <<End of Change for TR 38.876>>