3GPP TSG-RAN WG4 Meeting # 106bis-e													R4-2304677

Online, April 17 – April 26, 2023

Agenda item:	5.17.2

Source: 	Qualcomm Incorporated

Title: 	Views on FR2 MIMO OTA requirements

Document for:	Approval

# Introduction

In RAN4#106, the framework captured for FR2 MIMO OTA performance requirements was approved in [1]. And the WF was approved in [2]. In this meeting, we will provide our views on the remaining issues for FR2 MIMO OTA requirements development.

# Discussion

## Lab Alignment Activity

In RAN4#106, there are following agreements captured in [2]:

Issue 1-1-3: FR2 lab alignment activity

<Agreement>:

- For FR2 lab alignment activity, at least 3 participating labs and 2~3 PADs for each band are required.

- For PAD selection, NSA is prioritized and SA is not precluded.

- FFS the lab alignment pass/fail limit.

To specify the lab alignment pass/fail limit, RAN4 needs to finalize the MU budget for FR2 3D-MPAC system firstly and then make the decision on pass/fail limit based on the MU budget and measurement results conducted in lab alignment activity.

Proposal 1: To specify the lab alignment pass/fail limit, RAN4 needs to finalize the MU budget for FR2 3D-MPAC system firstly and then make the decision on pass/fail limit based on the MU budget and measurement results conducted in lab alignment activity.

See the MU budget analysis in our company paper [3].

## Simulation Platform Validation Activity

The procedure of simulation platform validation has been discussed in RAN4#106 and the following agreements were captured in [1]

<Agreement>:

- The measurement results used for the simulation platform validation activity should be also from the aligned lab(s).

- Adopt MASC as the pass/fail criteria for simulation platform validation. The limit of MASC for validating simulation platform is FFS.

To guarantee the accuracy of simulation validation, the measurement results used for the simulation validation activity should come from aligned lab. It should be noted that the usage of prototype UE for simulation validation shall be allowed from validation perspective. In addition, since it is necessary to align the configurations between measurement device and simulation assumptions, the simulation platform validation activity could be done in one of lab to prevent disclosing too much information of measurement device. The correlation between simulation and measurement could be completed within company that wants to validate simulation platform and specific aligned lab that volunteers to participate in the validation activity.

Proposal 2: Prototype UE shall be allowed for simulation platform validation. The simulation platform validation activity could be done in one of volunteered labs for validation activity.

## Proportion of certain number of UE panel

The issue of whether to require the proportion of a certain number of antenna panel UE was discussed with the following agreements:

Issue 1-1-7: Whether to require the proportion of single panel UE

<Way forward>:

- FFS on the number of antenna panel and whether to consider the proportion of a certain number of antenna panel UE

  - Encourage companies to propose constructive and workable approaches on how to guarantee the proportion of certain panel UE at the next meeting.

As agreed in previous meetings, it will not disclose any UE information except supported bands in the measurement activity. Therefore, it is not possible to know the number of antenna panels implemented in measurement devices. With that, it is reasonable to derive the requirements based on the measurement results of commercial devices and does not need to consider any proportion of number of UE panel. In addition, RAN4 should respect the measurement results from the devices collected from market which reflects the status of mmW UE performance when defining FR2 MIMO OTA requirements.

Proposal 3:  RAN4 to derive the requirements based on the measurement results of commercial devices and does not need to consider any proportion of certain number of antenna panel UE.

# Conclusion

In this paper, we provided our views on FR2 MIMO OTA requirements. We have the following observations and proposals:

Proposal 1: To specify the lab alignment pass/fail limit, RAN4 needs to finalize the MU budget for FR2 3D-MPAC system firstly and then make the decision on pass/fail limit based on the MU budget and measurement results conducted in lab alignment activity.

Proposal 2: Prototype UE shall be allowed for simulation platform validation. The simulation platform validation activity could be done in one of volunteered labs for validation activity.

Proposal 3:  RAN4 to derive the requirements based on the measurement results of commercial devices and does not need to consider any proportion of certain number of antenna panel UE.

# References

- R4-2302975, “Framework for FR2 MIMO OTA performance requirements”, CAICT

- R4-2302927, “WF on NR MIMO OTA enhancement”, CAICT

- R4-23xxxxx, “MU budget for FR2 MIMO OTA test system”, Qualcomm