3GPP TSG-RAN WG4 Meeting #106bis-e	R4-2304685

Online, April 17 – April 26, 2023

Source:	China Telecom

Title:	Discussion on the phase I study assumptions for the advanced receiver for MU-MIMO

Agenda Item:	5.19.1.2

Document for:	Discussion

- Introduction

RAN4 #106 meeting is the first meeting for us to discuss the advanced receiver to cancel inter-user interference for MU-MIMO within the NR_demod_enh3-Perf WI. The WF is approved in [1] as the discussion outcome.

In this paper, our views on the phase I study assumptions are given.

- Discussion

The number of co-scheduled UEs

Status in the WF in [1]:

- For initial evaluation stage

- At least 1 co-scheduled UE

- FFS whether more than 1 co-scheduled UE need to be considered, interested companies are encouraged to bring analysis and evaluation results

In addition to 1 co-scheduled UE that has been covered, we support to also cover more than 1 co-scheduled UE at least for the phase I study. We believe it is important to verify the R-ML performance under inter-user-interference from different modulation orders, which could be commonly happen in the MU-MIMO scenario, before introducing this new feature to the Rel-18 spec. To enable different modulation orders from the IUI, it is proposed to cover 2 co-scheduled UEs at least in phase I study.

Proposal 1: Cover 2 co-scheduled UEs at least in phase I study.

Rank allocation for the target and co-scheduled UEs, with 2 co-scheduled UEs

Status in the WF in [1]:

- Proposals on ‘rank number for target UE + rank number for the 1st co-scheduled UEs + rank number for the 2nd co-scheduled UEs’:

  - Option 1: Rank 1+1+1

- Agreement:

  - Option 1 for interested companies to bring evaluation results in initial study stage

For the rank allocation with 2 co-scheduled UEs, we support rank 1 for each of the UE, i.e., option 1 as above.

Proposal 2: For the study case for more than 1 co-scheduled UEs, allocate rank 1 for each of the UE.

MCS for the target UE

Status in the WF in [1]:

- Cover MCS 13 for rank 1 and rank 2 for initial simulation

- Further discuss whether to cover MCS 4 for rank 1 and MCS 19 for rank 2 in the next meeting

- The assumption can be updated later based on available results.

MCS 13 is the configuration we used for the Rel-15 R-ML requirements for SU-MIMO scenario. In our understanding, it is the modulation order for the co-scheduled UE that is more important to verify the R-ML performance to handle the IUI interference. To try to limit the phase I simulation cases, we slightly prefer to only consider MCS 13 for the target UE and cover other modulation orders for the co-scheduled UEs.

Proposal 3: To try to limit the phase I simulation cases, we slightly prefer to only consider MCS 13 for the target UE and cover other modulation orders for the co-scheduled UEs.

Modulation order for the co-scheduled UE

Status in the WF in [1]:

- For R-ML, E-IRC and IRC (baseline in Rel-17, for performance comparison purpose) for initial simulation

  - For rank 1+1: QPSK (high priority for the next meeting)

  - For rank 2+2: 64QAM (high priority for the next meeting)

  - For rank 2+2: QPSK (high priority for the next meeting)

  - For rank 1+3: 16QAM (high priority for the May meeting)

  - For rank 1+1 (64QAM) +1 (QPSK) (lower priority)

  - For rank 1+1 (64QAM) +1 (16QAM) (lowest priority)

  - Other options on the modulation order for co-scheduled UE are not precluded.

  - These assumptions can be updated in the next meeting based on available simulation results.

In the Rel-17 evaluation, only 1 co-scheduled UE with random 16QAM modulated symbols is considered. R-ML receiver needs to detect the co-scheduled modulated symbols with the highest likelihood within the constellation. Only evaluate the detection within 16QAM constellation is not enough.

Furthermore, from the UE implementation complexity point of view, by increasing the modulation from 16QAM to 64QAM, to achieve similar gain, the R-ML receiver requires 3 times more complexity. We understand that the UE implementation complexity should be limited thus it is important to verify that the R-ML receiver can be workable under higher modulation order from the co-scheduled UE.

Considering the above it is proposed to cover QPSK, 16QAM and 64QAM for the co-scheduled layers.

Proposal 4: Support to cover QPSK, 16QAM and 64QAM for the co-scheduled layers, fine with the following cases agreed in the WF as a start point.

- For rank 1+1: QPSK

- For rank 2+2: 64QAM

- For rank 2+2: QPSK

- For rank 1+3: 16QAM

- For rank 1+1 (64QAM) +1 (QPSK)

- For rank 1+1 (64QAM) +1 (16QAM)

Antenna correlation

Status in the WF in [1]:

- For initial simulation in Phase I only:

  - Rank 1+1: ULA medium

  - Rank 1+1+1: ULA medium A, XPL medium

  - Rank 2+2, 1+3: ULA Low

  - The assumptions can be updated later based on available results

Based on our simulation result provided in [2], under ULA medium A correlation for rank 1+1+1 case, the SNR points for R-ML and E-IRC are as high as 30dB and 70% max TP cannot be achieved for IRC receiver. Therefore, we propose not to consider ULA medium A for rank 1+1+1 if reasonable SNR points cannot be achieved.

Observation 1: Under ULA medium A correlation for rank 1+1+1 case, the SNR points for R-ML and E-IRC are as high as 30dB and 70% max TP cannot be achieved for IRC receiver.

Proposal 5: Not to consider ULA medium A for rank 1+1+1 if reasonable SNR points cannot be achieved.

Precoder selection target and co-scheduled UEs

Status in the WF in [1]:

- Single panel Type 1

- Random PMI selection for the target UE

- Cover both orthogonal and random PMI selection (same as Rel-17 approach) for the co-scheduled UE in phase I

- The assumption can be updated later base on the available results.

Under random PMI selection for the target UE, neither orthogonal nor random PMI selection for the co-scheduled UE is nearer to the practical scenario, but only related to the interference level without any impact to the R-ML or E-IRC implementation. In the Rel-17 study, it has been proofed that IRC receiver can achieve reasonable gain under both orthogonal and random PMI selection.

Considering the above, we propose to only consider 1 PMI selection method for each simulation case to save the simulation workload.

Proposal 6: Only consider 1 PMI selection method for each simulation case to save the simulation workload.

PDSCH resource allocation for the target and co-scheduled UE

Status in the WF in [1]:

- For initial simulation in phase I, cover scenario 1, further discuss whether to cover scenario 2 in the next meeting

  - Scenario 1: Maximum transmission bandwidth configuration for all UEs.

  - Scenario 2: Maximum transmission bandwidth configuration for the target UE and partial transmission bandwidth configuration for the co-scheduled UEs.

  - Full OFDM symbol allocation for both scenarios.

In the real network, BS could allocate different PRBs and different modulation specifically for each UE, considering the different traffic load and different channel condition. Such scenario will impact the R-ML and E-IRC performance since both receivers will need to estimate the channel for the co-scheduled UE. We think the scenario with partial bandwidth allocation for the co-scheduled UE, should be considered in the phase I study. As has been proposed in [3], we propose to consider the following scenario for the study of R-ML and E-IRC with blind detection to the required information:

- Target UE with frequency domain full PRB allocation (52PRBs)

- Co-scheduled UE1 with PRB0~19 allocation with 16QAM transmission.

- Co-scheduled UE2 with PRB40~51 allocation with QPSK transmission.

- Full OFDM symbol allocation for both scenarios.

Proposal 7: Consider the following scenario for the study of R-ML and E-IRC with blind detection to the required information:

- Target UE with frequency domain full PRB allocation (52PRBs)

- Co-scheduled UE1 with PRB0~19 allocation with 16QAM transmission.

- Co-scheduled UE2 with PRB40~51 allocation with QPSK transmission.

- Full OFDM symbol allocation for both scenarios.

- Conclusion

Proposal 1: Cover 2 co-scheduled UEs at least in phase I study.

Proposal 2: For the study case for more than 1 co-scheduled UEs, allocate rank 1 for each of the UE.

Proposal 3: To try to limit the phase I simulation cases, we slightly prefer to only consider MCS 13 for the target UE and cover other modulation orders for the co-scheduled UEs.

Proposal 4: Support to cover QPSK, 16QAM and 64QAM for the co-scheduled layers, fine with the following cases agreed in the WF as a start point.

- For rank 1+1: QPSK

- For rank 2+2: 64QAM

- For rank 2+2: QPSK

- For rank 1+3: 16QAM

- For rank 1+1 (64QAM) +1 (QPSK)

- For rank 1+1 (64QAM) +1 (16QAM)

Observation 1: Under ULA medium A correlation for rank 1+1+1 case, the SNR points for R-ML and E-IRC are as high as 30dB and 70% max TP cannot be achieved for IRC receiver.

Proposal 5: Not to consider ULA medium A for rank 1+1+1 if reasonable SNR points cannot be achieved.

Proposal 6: Only consider 1 PMI selection method for each simulation case to save the simulation workload.

Proposal 7: Consider the following scenario only for the study of R-ML and E-IRC with blind detection to the required information:

- Target UE with frequency domain full PRB allocation (52PRBs)

- Co-scheduled UE1 with PRB0~19 allocation with 16QAM transmission.

- Co-scheduled UE2 with PRB40~51 allocation with QPSK transmission.

- Full OFDM symbol allocation for both scenarios.

- References

- [1] R4-2302929, WF on advanced receiver for MU-MIMO scenario, China Telecom, RAN4 #106, March, 2023

- [2] R4-2304686, Phase I study for the advanced receiver for MU-MIMO: simulation results, China Telecom, RAN 4# 106bis-e, Apr 2023

- [3] R4-2304684, Discussion on the receiver assumptions for the advanced receiver for MU-MIMO, China Telecom, RAN 4# 106bis-e, Apr 2023