# 3GPP TSG-RAN WG4 Meeting # 106bis-e                            R4-2304689

# Online, April 17 – April 26, 2023

# Source:        ZTE Corporation

# Title:           Discussion on FR2 UL 256qam

# Agenda Item:   5.7.2

# Document for:  Discussion

- Introduction

In RAN 96 meeting, a revised WID [1] for NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved, where for FR2 UL 256QAM, the objections are:

UL 256QAM

- Investigate and enable UL 256QAM for FR2-1 [RAN4]

  - Study the gain, operating SNR, phase noise model and implementation aspects

  - Specify the UE RF requirements

  - First priority: Targeted power classes are PC1, PC2 and PC5

  - Second priority: Targeted power class is PC3

In last RAN4 meeting, some issues such as LLS parameters, SLS parameters and EVM tests were concluded and were captured in the WF[2] , in which the UL 256QAM feasibility for 29GHz and 39GHz are:

  - For 29GHz, UL 256 QAM for PC2/PC5 UEs is feasible.

  - For 39GHz, UL 256 QAM for PC1/PC2/PC5 UEs is feasible.

On top of the feasibility, there were still some open issues left, including the Phase noise profile, minimum EIRP for EVM test, PTRS configuration for EVM test and MPR test and etc.

In this contribution, we give some discussions on Phase noise profile, minimum EIRP.

- Discussion

## 2.1 Phase noise

For UL256QAM, the related RF requirements like MPR and EVM shall be defined. Due to 256QAM is high order QAM modulation scheme which is much sensitive to the noise, especially for high frequency FR2 band, higher phase noise would lead higher MPR value and worse EVM performance. Therefore, companies proposed some new phase noise models on top of the existing two models (example 1 and example 2) defined in TR38.803.

In the WF [2], there were two new phase noise models proposed in last meeting, one is to adopt (example 1, example 2), and the other one is to adopt new parameters based on the multi-pole/zero model. By considering the existing two ones, there are four phase noise model alternatives as below:

Alternative 1. example 1 defined in 38803

Alternative 2. example 2 defined in 38803

Alternative 3. min(example 1, example 2)

Alternative 4. example 1 defined in 38803 with parameters updated

Figure 2-1shows the phase noise profiles for different alternatives.

Figure 2-1. Phase noise profile comparison between different alternatives

From figure 2-1 we can find that lower phase noise can be achieved by alternative 3 and alternative 4 compared to the example 1 and example 2. In table 2-1, we compare the EVM performance for the above alternatives for CP-OFDM@45GHz, which is same conditions as in companies’ paper in last meeting.

Table 2-1 EVM comparison between different alternatives

| CP-OFDM w/ phase noise, 64 RBs, 256QAM, 120k SCS, DMRS based CPE removal@45GHz |  |  |  |
| --- | --- | --- | --- |
| Phase noise | EVM(dB) with PTRS | EVM (dB) with no PTRS corrections | Net benefit of PTRS |
| UE alt 1 | -24.7 | -24.5 | 0.2 |
| UE alt 2 | -21.5 | -20.4 | 1.1 |
| UE alt 3 | -30.4 | -28 | 2.4 |
| UE alt 4 | -27.3 | -27.1 | 0.2 |

From table 2-1 we can find that alt 3 can achieve much smaller phase noise which means much smaller EVM can be expected @ 45GHz. Although the simulation is for 45GHz, it is reasonable to expect that the same results can be applied for other millimeter wave frequency.

For alt 3, although smallest phase noise can be achieved, the logic to minimize both example 1 and example 2 may not be clear. As explained in TS38.803, example 1 is based upon measurements made on a prototype CMOS device, with a larger PLL bandwidth, and example 2 is based on recent research on technology capabilities for UE and BS where CMOS is considered for the UE and GaAs is considered for the BS, so a lower PLL bandwidth than example 1 can be assumed for example2. It is clearly that Example 2 distinguishes UE and BS while example 1 is the common model for both UE and BS, from this perspective, the reasons for directly taking the minimum value of both, min(Ex 1, Ex 2), are not very clear. Nevertheless, we still believe the lower phase noise can not only improve the EVM performance, but also improve the MPR requirements.

Observation 1: Smallest phase noise can be achieved for Alt 3, i.e. min(Ex1, Ex2), and lower EVM caused by phase noise can be expected.

## 2.2 Minimum EIRP

In RAN4 #106 meeting, minimum EIRP requirement for EVM test was not reached and the alternatives are listed below[2].

Issue 3-1-1: The minimum EIRP requirements for EVM test - Option 1: The minimum output power for 256QAM during the EVM test can be relaxed by 14 dB based on the difference between the SNR of 256QAM (29.1dB) and the SNR of QPSK(15.1dB) (ZTE, Xiaomi, vivo, Huawei) Parameter; Unit; Level for PC1; Level for PC2; Level for PC5. UE EIRP; dBm; 4; -13; -6. UE EIRP for UL 256 QAM; dBm; 18; 1; 8. - Option 2: Use a “-1dB/dB” relation to calculate the minimum EIRP requirement for 256QAM and consider 1dB correction factor. (MTK, Ericsson) Parameter; Unit; PC1; PC2; PC5. UE EIRP; dBm; 4; -13; -6. UE EIRP for UL 256 QAM; dBm; 19.5; 2.5; 9.5. - Option 3: Further scaling the minimum EIRP with bandwidth based on Option 2 (Apple) Level for PC2. Parameter; Unit; 50 MHz; 100 MHz; 200 MHz; 400 MHz. UE EIRP for UL 256 QAM; dBm; 2.5; 2.5; 5.5; 8.5. Operating conditions; Normal Conditions. NOTE 1: PTRS is configured for 256 QAM.

## The above three options can be divided into two questions:

- Confirm the minimum EIRP of 256QAM

- Scaling by bandwidth for minimum EIRP of 256QAM

## For the first bullet, the argument point is whether it is necessary to consider additional correction factors to compensate for the loss caused by thermal and phase noise. In [3], link budget assessments were given assuming the channel bandwidth is 400MHz and the NF of gNB receiver 18dB which is a relative conservative value for a BS in our understanding. Therefore the scaling scheme may not be needed at least for BW not larger than 400MHz.

Proposal 1: To confirm option 1 as the minimum EIRP requirements for EVM test.

- Conclusion

Based on the simulation results and discussion, the following observation and proposal are given:

Observation 1: Smallest phase noise can be achieved for Alt 3, i.e. min(Ex1, Ex2), and lower EVM caused by phase noise can be expected.

Proposal 1: To confirm option 1 as the minimum EIRP requirements for EVM test.

- Reference

RP-221862 revised WID NR RF requirements enhancement for frequency range 2 (FR2), Phase 3, Nokia, Xiaomi

R4-2303709 WF on FR2-1 UL 256QAM, Xiaomi

R4-2301620 Discussion on UL 256QAM, Xiaomi