# 3GPP TSG-RAN WG4 Meeting # 106bis-e                               R4-2304690

# Online, April 17 – April 26, 2023

# Title: 	Discussion on demodulation for 4Tx of NR_ENDC_RF_FR1_enh2

# Source: 	ZTE Corporation

# Agenda item:	5.5.4.2

# Document for:	Discussion

# 1 Introduction

The objectives for 4Tx of further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1) are listed in the following [1]:

Enable 4Tx on a single carrier for CPE/FWA/vehicle/industrial devices [RAN4]

- Investigate framework and architecture Example bands:

  - TDD bands: n41, n77/n78

  - FDD bands: n1

    - Note 1: the total number of example bands should be limited to 3. n77/n78 are considered as one band during the study.

    - Note 2: other bands to be introduced in the release independent way later on from Rel-18

    - Note 3: specifying requirements for TDD bands has first priority

- Specify the UE RF requirements to support 4Tx

  - First priority: 4x4 UL MIMO

  - Second priority: investigate and if necessary specify TxD requirement to support the same power class in UL MIMO and single antenna port

  - PA configuration assumption:

    - First priority: 4x23dBm

    - Second priority: 2x23dBm + 2x26dBm, 4x26dBm

  - UE power class

    - First priority: PC 1.5

    - Second priority: PC2/PC3, and/or new power class if needed

    - Note 1: PC1.5 is only applicable for TDD bands

  - Note: detailed combinations for 2nd priority PA/UE power class assumptions are to be revisited in RAN#97

- NOTE1: Requirements are specified with phase approach. Objectives with 1st priority are considered first.

- NOTE2: It is assumed the devices are equipped with 4Rx antennas for band n1 requirement definition.

In RAN4# 106 meeting, some agreements were reached for 4Tx evaluation and the following evaluation setups need further discussion.

Sub-topic 2-5 MCS

Agreement: Use MCS table 1 for the evaluation. The following MCS can be used as starting point for evaluation.

- TDLA30-10: MCS16, MCS17 and MCS20

- TDLB100-400: MCS2

- TDLC300-100: MCS12 and MCS16

Sub-topic 2-8 Precoding

Agreement:

Take TPMI index 0 in Table 6.3.1.5-7 of 38.211 for performance requirement definition.

FFS on TPMI index 4 in Table 6.3.1.5-7 of 38.211 for vehicular use cases.

In this contribution, we share our understanding for the above issues and we give initial simulation results for 4Tx.

# 2	Discussion

## 2.1 MCS

It was agreed that MCS index table 1(Table 5.1.3.1-1 in TS 38.214) is used to select MCS for evaluation and the concerned MCS is list in Table 1.

Table 1: Concerned MCS index for PDSCH

| MCS Index IMCS | Modulation Order Qm | Target code Rate R x [1024] | Spectral efficiency |
| --- | --- | --- | --- |
| 2 | 2 | 193 | 0.3770 |
| 12 | 4 | 434 | 1.6953 |
| 16 | 4 | 658 | 2.5703 |
| 17 | 6 | 438 | 2.5664 |
| 20 | 6 | 567 | 3.3223 |

It was also agreed that only one MCS for each propagation condition will be selected for performance requirements definition. As MCS2 is selected for TDLB100-400, the remaining MCSs are MCS12 MCS16 for 16QAM and MCS17 and MCS20 for 64QAM.

Figure 2-1 shows the throughput comparison between MCS16, MCS17 and MCS20 for TDL-A30-10,

Figure 2-1 Throughput comparison between different MCS for TDL-A 30-10

From figure 2-1 we can see that MCS20 is feasible when TDL-A 30-10 is considered. Therefore it reasonable to select MCS20 for TDL-A30-10. If MCS20 is selected for TDLA30-10, a MCS corresponding to 16QAM should be considered. Figure 2-2 shows the throughput comparison between MCS12 and MCS16 for TDL-C 300-100.

Figure 2-2Throughput comparison between MCS12 and MCS16 for TDL-C 300-100

From figure 2-2 we can see that MCS16 is feasible when TDL-C 300-100 is considered. Therefore it reasonable to select MCS16 for TDL-C 300-100. Some initial simulation results are appended in Annex for reference.

Proposal 1: To use the following MCS and channel model for demodulation requirement:

- TDLA30-10: MCS20

- TDLB100-400: MCS2

- TDLC300-100: MCS16

## 2.2 TPMI

It was agreed to take TPMI index 0 in Table 6.3.1.5-7 of 38.211 for performance requirement definition.

Another TPMI of index 4 in Table 6.3.1.5-7 of 38.211 is FFS for vehicular use cases.

The difference between TPMI index 0 and TPMI index 4 is that the precoder of TPMI index 4 carries each layer on different antennas which may bring diversity comparing with the precoder of TPMI index 0.

With regard to demodulation requirement, a fixed precoder is used by companies and this precoder is not matched with CSI as no CSI feedback is concerned for requirements definition. In other words, there is not a TPMI index which is more suitable for channel conditions than other PMI indices.

On the other hand, TPMI index 4 is full coherent precoder, which is related to UE capability. Therefore, TPMI index 4 may not be feasible for UE without the capability of full coherent precoder.

Proposal 2: To only use the TPMI index 0 for demodulation requirement.

# 3 Conclusion

In this contribution, we give some conclusion for 4Tx of NR_ENDC_RF_FR1_enh2 :

Proposal 1: To use the following MCS and channel model for demodulation requirement:

- TDLA30-10: MCS20

- TDLB100-400: MCS2

- TDLC300-100: MCS16

Proposal 2: To only use the TPMI index 0 for demodulation requirement.

# 4	Reference

[1]  RP-223478 Revised WID Rel-18 NR_ENDC_ RF_FR1_enh2, Huawei, HiSilicon, NTT DOCOMO

[2]  R4-2302926 WF on FR1 4Tx demodulation requirement, Ericsson

# Annex

Figure A-1 Throughput for 5MHz BW 15kHz SCS TDL-A 30-10 mapping type A

Figure A-2 Throughput for 5MHz BW 15kHz SCS TDL-B 100-400 mapping type A

Figure A-3 Throughput for 5MHz BW 15kHz SCS TDL-C 300-100 mapping type A

Figure A-4 Throughput for 20MHz BW 15kHz SCS TDL-A 30-10 mapping type A

Figure A-5 Throughput for 20MHz BW 15kHz SCS TDL-B 100-400 mapping type A

Figure A-6 Throughput for 20MHz BW 15kHz SCS TDL-C 300-100 mapping type A

Figure A-7 Throughput for 50MHz BW 15kHz SCS TDL-A 30-10 mapping type A

Figure A-8 Throughput for 50MHz BW 15kHz SCS TDL-B 100-400 mapping type A

Figure A-9 Throughput for 50MHz BW 15kHz SCS TDL-C 300-100 mapping type A

Figure A-10 Throughput for 10MHz BW 30kHz SCS TDL-A 30-10 mapping type A

Figure A-11 Throughput for 10MHz BW 30kHz SCS TDL-B 100-400 mapping type A

Figure A-12 Throughput for 10MHz BW 30kHz SCS TDL-C 300-100 mapping type A

Figure A-13 Throughput for 100MHz BW 30kHz SCS TDL-A 30-10 mapping type A

Figure A-14 Throughput for 10MHz BW 30kHz SCS TDL-B 100-400 mapping type A

Figure A-15 Throughput for 100MHz BW 30kHz SCS TDL-C 300-100 mapping type A