3GPP TSG-RAN WG4 Meeting #106-bis-e	R4-2304695

Electronic Meeting, 17 April. – 26 April. 2023

Agenda Item:	4.1.1.2

Source:	Charter Communications, Inc.

Title:	Discussion of simulation results on UE RF NR-U UL CA MPR and A-MPR for PC5

Document for:	Discussion

# Introduction

In RAN4#106 meeting, a WF for introducing Carrier aggregation to Power class 5 (PC5) in NR-unlicensed spectrum for was approved [1].

“In Release 17 the CA_n96B/C UL CA and configurations are specified. Even so, it was clarified in RAN that although NR-U ULCA requirements were completed, NR-U ULCA could not be deployed since the A-MPR for n96B/C UL was not specified, especially for the U.S. NS_53 and NS_54.” [1]

In this paper we will provide some initial simulation results for NR-U UL CA MPR and A-MPR for NS_54. The paper will start off recapping the general requirements as well for the NS_54. Then there will be the initial results.

# Discussion

## PC5 CA MPR

Simulations were done for unlicensed spectrum with PC5. The simulations were calibrated with the following PA calibration point and impairments with the list provided below:

## Simulation calibrations

### PC5 UL CA (A-)MPR:

- Power Class: PC5

- Calibration Point: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB3 waveform at 27dB ACLR

- Post PA losses 4dB

- ACLR: 27 dB

- SEM: NR-U

- IQ image: -28dB

- Carrier leakage: -28dBc

Table 3: Setup for MPR

| Test Number | Modulation | Allocation | Precoder | Sub-Band |
| --- | --- | --- | --- | --- |
| 1 | Pi/2-BPSK | Full | DFT-S-OFDM | All |
| 2 | Pi/2-BPSK | Interlace | DFT-S-OFDM | All |
| 3 | Pi/2-BPSK | Wideband Full | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 4 | Pi/2-BPSK | Wideband interlace | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 5 | QPSK | Full | DFT-S-OFDM | All |
| 6 | QPSK | Full | CP-OFDM | All |
| 7 | QPSK | Wideband Full | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 8 | QPSK | Wideband Full | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 9 | QPSK | Interlace | DFT-S-OFDM | All |
| 10 | QPSK | Interlace | CP-OFDM | All |
| 11 | QPSK | Wideband interlace | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 12 | QPSK | Wideband interlace | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 13 | 16QAM | Full | DFT-S-OFDM | All |
| 14 | 16QAM | Full | CP-OFDM | All |
| 15 | 16QAM | Wideband Full | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 16 | 16QAM | Wideband Full | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 17 | 16QAM | Interlace | DFT-S-OFDM | All |
| 18 | 16QAM | Interlace | CP-OFDM | All |
| 19 | 16QAM | Wideband interlace | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 20 | 16QAM | Wideband interlace | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 21 | 64QAM | Full | DFT-S-OFDM | All |
| 22 | 64QAM | Full | CP-OFDM | All |
| 23 | 64QAM | Wideband Full | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 24 | 64QAM | Wideband Full | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 25 | 64QAM | Interlace | DFT-S-OFDM | All |
| 26 | 64QAM | Interlace | CP-OFDM | All |
| 27 | 64QAM | Wideband interlace | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 28 | 64QAM | Wideband interlace | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 29 | 256QAM | Full | DFT-S-OFDM | All |
| 30 | 256QAM | Full | CP-OFDM | All |
| 31 | 256QAM | Wideband Full | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 32 | 256QAM | Wideband Full | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 33 | 256QAM | Interlace | DFT-S-OFDM | All |
| 34 | 256QAM | Interlace | CP-OFDM | All |
| 35 | 256QAM | Wideband interlace | DFT-S-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |
| 36 | 256QAM | Wideband interlace | CP-OFDM | e.g. 0-1, 0-010, 0-100, 0-011, 1-100, 1-1100, 0-0111 |

For sub-band, all means there is no LBT failure/occupancy in any of the 20MHz channels. Full allocation follows the Table 5.3.3-2 in TS 38.101-1 for how many resource blocks are used in each 20MHz channel [2] and all available resource blocks are utilized in the simulations.

## CA MPR Simulation Results

Figure 1: Initial simulation results for UL CA MPR PC5 with 27dB ACLR for calibration purpose.

Table 1: Initial simulation results for UL CA MPR PC5 with ACLR 27dB for calibration purpose.

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) | Exception for Full5 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 |  |  |  |
|  | QPSK | ≤ [3.0] |  |  |
|  | 16 QAM | ≤ [3.5] |  |  |
|  | 64 QAM |  |  |  |
|  | 256 QAM |  |  |  |
| CP-OFDM | QPSK | ≤ [4.0] |  |  |
|  | 16 QAM | ≤ [4.5] |  |  |
|  | 64 QAM |  |  |  |
|  | 256 QAM |  |  |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2A.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2A.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Exception for Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration [B] in Table 6.2F.2A.2-2. |  |  |  |  |

## PC5 CA A-MPR

The networking signaling value NS_54 is used when operating in band n96. Agreement that to add Carrier aggregation (CA) with Power class 5 will be introduced.

## In-Band emissions

There are additional requirements for the transmit power density as seen in Table 6.2F.1-2 in TS 38.101-1 [2] for all unlicensed bands. A part of Table 6.2F.1-2 is shared below.

Approximately, for a full allocation in a 20MHz channel bandwidth, with maximum Power for PC5 (20dBm), the maximum mean power density (dBm/MHz) will be around 7, this means the in-band emission requirements for NS_53, will be leading the simulation results, while for NS_54 it will probably not be the restrictive/stringent requirement. The wider bandwidth is simulated, the more the power density is spread out in the channel bandwidth, the less restrictive the maximum mean power density is.

Table 6.2F.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n96 | NS_53 | 20, 40, 60, 80 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |

## Out-of-Band emissions

Additional requirements for the spurious emission limit for operations using the NS_54 the TS 38.101-1 [2] says; “When "NS_53" or "NS_54" or "NS_60" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5F.3.3.5-1.”

Table 6.5F.3.3.5-1: Additional requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

Based on the defined NR-ARFCN number for band n96 (Table 5.4.2.3-3 in TS 38.101-1 [2]) , the lowest number for each bandwidth using the formula

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |

This gives center frequencies for each edge channel depending of bandwidth of the lower edge of band n96 for bandwidth class B:

- 40MHz total aggregated Channel Bandwidth: 5965.02MHz

- 60MHz total aggregated Channel Bandwidth: 5974.98MHz

- 80MHz total aggregated Channel Bandwidth: 5985MHz

- 100MHz total aggregated Channel Bandwidth: 5995.02MHz

For band n96, the additional requirements are offset from the channel bandwidth edge by 20MHz. All other channels will follow the CA MPR backoff limits defined in Table 6.2F.2A.2-1 in TS 38.101-1 [2].

## A-MPR Simulation Results

Charter’s simulation results for NS_54 with Power Class 5, see Figure 2 and Table 2. This is simulated for shared spectrum access intra-band contiguous CA for bandwidth class B only and due to time constraint, we had only time to run some initial simulations with the hope to compare and calibrate with other companies during this April meeting.

Figure 2: Initial simulation results for UL CA A-MPR PC5 for Band n96B for NS_54.

Table 2: Initial simulation results for UL CA A-MPR PC5 for Band n96B with NS_54 for calibration purpose.

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) | Exception for Full5 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 |  |  |  |
|  | QPSK | ≤ [4.0] |  |  |
|  | 16 QAM | ≤ [4.5] |  |  |
|  | 64 QAM |  |  |  |
|  | 256 QAM |  |  |  |
| CP-OFDM | QPSK | ≤ [5.0] |  |  |
|  | 16 QAM | ≤ [5.0] |  |  |
|  | 64 QAM |  |  |  |
|  | 256 QAM |  |  |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2A.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2A.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Exception for Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration [B] in Table 6.2F.2A.2-2. |  |  |  |  |

# Conclusion

In this paper, Charter has provided some initial simulation results of MPR and A-MPR (for NS_54) for power class 5 shared spectrum access intra-band contiguous CA for bandwidth class B. We hope to get a calibration discussion with the end objective of adding A-MPR table for band n96 with NS_53 and NS_54 for intra-band contiguous CA for bandwidth class B and class C.

# Reference

[1] R4-2303484, WF on NR-U enhancement, Apple

[2] TS 38.101-1: User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V17.8.0 (2022-12)

[3] TR 37.890, Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations V0.16.0 (2022-06)