3GPP TSG-RAN WG4 Meeting # 106-bis-e                              R4-2304699

Electronic Meeting, April 17-26, 2023

Source: 	MediaTek Inc.

Title: 	Views on FR2-1 UL 256QAM

Agenda Item:	5.7.2

Document for:	Discussion

- Introduction

In RAN4 #106, a WF for UL 256QAM has been approved [1]. However, some open issues related to minimum EIRP requirements and PTRS configuration require further discussion in subsequent meetings. In this contribution, we will present our views on these unresolved issues.

- 2	Discussion

2.1 Minimum EIRP requirements

Regarding minimum EIRP requirements, the remaining open issue is listed in the below.

- Issue 3-1-1: The minimum EIRP requirements for EVM test - Option 1: The minimum output power for 256QAM during the EVM test can be relaxed by 14 dB based on the difference between the SNR of 256QAM (29.1dB) and the SNR of QPSK(15.1dB) (ZTE, Xiaomi, vivo, Huawei) Parameter; Unit; Level for PC1; Level for PC2; Level for PC5. UE EIRP; dBm; 4; -13; -6. UE EIRP for UL 256 QAM; dBm; 18; 1; 8. - Option 2: Use a “-1dB/dB” relation to calculate the minimum EIRP requirement for 256QAM and consider 1dB correction factor. (MTK, Ericsson) Parameter; Unit; PC1; PC2; PC5. UE EIRP; dBm; 4; -13; -6. UE EIRP for UL 256 QAM; dBm; 19.5; 2.5; 9.5. - Option 3: Further scaling the minimum EIRP with bandwidth based on Option 2 (Apple) Level for PC2. Parameter; Unit; 50 MHz; 100 MHz; 200 MHz; 400 MHz. UE EIRP for UL 256 QAM; dBm; 2.5; 2.5; 5.5; 8.5. Operating conditions; Normal Conditions. NOTE 1: PTRS is configured for 256 QAM.

The UE minimum EIRP for QPSK, 16QAM and 64QAM feature a -1dB/dB relation which is being used to derive the minimum UE EIRP for 256QAM for the EVM budget of 3.5%. Based on the previous work for the derivation of UE minimum EIRP for keeping -1 dB/dB relation, we can compromise on Option 1.

Proposal 1: If Option 2 is not agreeable, we can compromise to Option 1 if it is the majority view.

The minimum EIRP requirement for PC3 for different channel bandwidth has been proposed [1], and in Rel 15, the minimum EIRP for PC1 and PC3 captured in 38.101-2 (Table 6.4.2.1-2 and Table 6.4.2.1-3) is based on the Maximum coupling loss (MCL) analysis for 400MHz channel bandwidth. Therefore, in our view, further scaling the minimum EIRP with bandwidth based on Option 2 is unnecessary.

Observation 1: The minimum EIRP for PC1 and PC3 captured in 38.101-2 (Table 6.4.2.1-2 and Table 6.4.2.1-3) is based on the Maximum coupling loss (MCL) analysis for 400MHz channel bandwidth.

Proposal 2: Further scaling the minimum EIRP with bandwidth based on Option 2, i.e., Option 3, is unnecessary.

2.2 PTRS configuration

- Issue 3-2-1: PTRS configuration for EVM test - Option1: - FFS using a fixed PTRS configuration (K = 2, L = 1) for all devices as the default configuration for the EVM test. - Recommended PTRS configuration by UE via IE PTRS-DensityRecommendationSetUL for the EVM test is allowed. Whether UE shall be tested according to recommended PTRS configuration when IE is signalled or it shall be tested according to the default fixed PTRS configuration in all cases is FFS. - Recommended PTRS is optional.

The PTRS plays a crucial role especially at mmWave frequencies to minimize the effect of the oscillator phase noise on system performance. However, Companies have different views for the PTRS processing gain, e.g., some observation was raised that PTRS may degrade the signal quality for some phase noise profile [3], which is not consistent to other observation e.g., in [4], that no deterioration was visible and Is deterioration of the EVM following CPE correction using PTRS correction an actual possibility?. In this case, we propose to evaluate and reach a common understanding on impacts on EVM by using PTRS processing for different phase noise profiles at first, and then, we can continue to discuss PTRS configuration for EVM test.

Observation 2: Views for the PTRS processing gain are not aligned in RAN4.

Proposal 3: RAN4 to evaluate and reach a common understanding on impacts on EVM by using PTRS processing for different phase noise profiles at first, and then continue to discuss PTRS configuration for EVM test.

- 3	Conclusion

In this contribution, we have provided our views for the open issues on the WF [1].

Proposal 1: If Option 2 is not agreeable, we can compromise to Option 1 if it is the majority view.

Observation 1: The minimum EIRP for PC1 and PC3 captured in 38.101-2 (Table 6.4.2.1-2 and Table 6.4.2.1-3) is based on the Maximum coupling loss (MCL) analysis for 400MHz channel bandwidth.

Proposal 2: Further scaling the minimum EIRP with bandwidth based on Option 2, i.e., Option 3, is unnecessary.

Observation 2: Views for the PTRS processing gain are not aligned in RAN4.

Proposal 3: RAN4 to evaluate and reach a common understanding on impacts on EVM by using PTRS processing for different phase noise profiles at first, and then continue to discuss PTRS configuration for EVM test.

- 4	Reference

- R4-2303709, “WF on UL 256QAM”, Xiaomi.

- R4-1709419, “Further discussion on UE minimum transmit power for range 2”, CATT.

- R4-2300707, “On enabling FR2 UL256QAM”, Qualcomm Inc.

- R4-2301147, “UL 256QAM and CPE compensation based on PTRS ”, Anritsu Limited.